3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504460

# Wuhan, China, 07th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Rel-19 TC on SCell Activation and Deactivation Delay Requirement with less than 5 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce new Test Cases for SCell Activation and Deactivation Delay requirements with less than 5 MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the following test cases: - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle with less than 5MHz BW - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle with less than 5MHz BW - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX with less than 5MHz BW |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without validation, UEs may fail to meet activation/deactivation timing, causing latency issues, lower peak data rates, and inefficient CA performance in less than 5 MHz bandwidth scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change0>>

### A.6.5.3	SCell Activation and Deactivation Delay

#### A.6.5.3.X	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle with less than 5MHz BW

##### A.6.5.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation, and the PCell and SCell operate with a bandwidth of less than 5 MHz.

Supported test configurations are specified in Table A.6.5.3.X.1-1 below. General test parameters for known Scell activation case is specified in Table A.6.5.3.1.1-2. Supported Cell specific test parameters for NR PCell specified in Table A.6.5.3.1.1-3. Supported Cell specific test parameters for NR SCell specified in Table A.6.5.3.1.1-4 apply except those specified in Table A.6.5.3.X.1-3.

The test procedure specified in A.6.5.3.1 applies to this test.

Table A.6.5.3.X.1-1: known FR1 SCell activation in non-DRX for 160 ms SCell measurement cycle supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥5 MHz bandwidth, FDD duplex mode |

‘Table A.6.5.3.X.1-1A: known FR1 SCell activation in non-DRX for 160 ms SCell measurement cycle supported test configurations for NR SCell with less than 5 MHz

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, < 5 MHz bandwidth, FDD duplex mode (12 PRB) |
| Note: The UE is required to pass the test with configuration 1 for every supported 3MHz band in FR1. If the UE indicates the support for band n100, the UE is also required to pass the test with configuration 2 in addition to the test with configuration 1. |  |

Table A.6.5.3.X.-3: Cell specific test parameters for NR SCell for known FR1 SCell activation case, 160 ms SCell measurement cycle with less than 5 MHz

| c |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| BWchannel | ConfigSCell 1, | MHz | 3 MHz |  |  |
| BWoccupied | ConfigSCell 1 | RB | 12 |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.X FDD |  |  |
| SSB Configuration | ConfigSCell 1 |  | SSB.13 FR1 |  |  |
| Io Note3 | ConfigSCell 1 | dBm/ 2.7 MHz | -60.31 | -58.58 | -58.58 |
| NOTE : On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in clause 5.2.2.5 in TS 38.214 [26]. |  |  |  |  |  |

##### A.6.5.3.X.2	Test Requirements

Test requirements specified in Clause A.6.5.3.1.2 apply to this test except Tactivation_time will be replaced with TFirstSSB+ T∆ + 5ms.

#### A.6.5.3.X	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle with less than 5MHz BW

##### A.6.5.3.X.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.6.5.3.X.1. The supported test configurations are the same as defined in clause A.6.5.3.X.1. The test parameters are the same except those described in the following clause. The listed parameter values in tables A.6.5.3.X.1-1 will replace the values of corresponding parameters in tables A.6.5.3.X.1-1.

Table A.6.5.3.X.1-1: General test parameters for known FR1 SCell activation case, 640 ms SCell measurement cycle with less than 5MHz BW

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |

##### A.6.5.3.X.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + Trs + T∆ + 5 ms.

#### A.6.5.3.X	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX with less than 5MHz BW

##### A.6.5.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is unknown by the UE at the time of activation and the PCell and/or SCell operate with a bandwidth of less than 5 MHz.

Supported test configurations are specified in Table A.6.5.3.X.1-1 below. General test parameters as specified in Table A.6.5.3.3.1-2 with config 1 apply except those specified in Table A.6.5.3.X.1-2. Supported test configurations for NR SCell specified in Table A.6.5.3.3.1-3 apply except those specified in Table A.6.5.3.X.1-1A.

The test procedure specified in A.6.5.3.3 applies to this test.

Table A.6.5.3.X.1-1: General test parameters for unknown FR1 SCell activation case, 160 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 110 | During this time the PCell shall be known and the SCell configured, but not detected. |

##### A.6.5.3.X.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + TSMTC_MAX + 2*Trs + T∆ + 5 ms as defined in clause 8.3.

### <End of Change0>>