3GPP TSG-RAN WG4 Meeting #114bis	R4-2504501

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Other RRM requirements for MIMO ph5

Agenda item:	7.22.3.2

Document for:	Discussion

Introduction

In this document, we present Nokia’s views regarding the core requirements that RAN4 needs to discuss in relation to the WID RP-242394, in particular about asymmetric DL sTRP/UL mTRP deployment scenarios (Objective 5) and the aspects related to CJT calibration reporting (Objective 3).

Discussion on two TAs

It was agreed in RAN4#114 meeting to further discuss the following  [2]:

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset? Agreement: For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2. For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not). RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1. - Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s) - Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s). Note 1: the scenario 1 and 2 are based on RAN1 LS. Note 2: The above agreements apply when 2TA are configured.

When the UE is not configured with a PL offset and the UL-only TRP is transmitting reference signals such as SSB(s) (which is the Scenario#2), the UE can independently monitor and measure the DL reference signals associated with the respective TRPs. The Rel-18 two TAs enhancement for multi-DCI multi-TRP scenario allow UL transmissions from a UE toward two TRPs whose links may be characterized by a large difference in the propagation delays with respect to the CP length.

As with legacy single TA requirements, it is up to the UE to decide which RS, among the RSs transmitted by each TRP, to use as DL reference timing for each TAG.

Similar to the Rel-18 two TA enhancements for m-DCI m-TRP, when the UE is not configured with a PL offset, the UE shall monitor two DL reference timings, one per TAG or TRP, and apply each TA command to the respective DL reference timing to determine the UL transmission timing toward that TAG. In that case, the system performance would be improved by using the independent reference timing from both TRPs, since the UL timing would be more robust against small timing offset variations caused by UE movement.

Use of the independent reference timing from both TRPs will ensure that the UL timing is robust for UE movement.

With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2) the UE shall monitor two DL reference timings, one per TAG or TRP.

Additionally, when a UE needs to perform autonomous timing adjustments, monitoring a single downlink reference signal from the DL/UL capable TRP and using that for autonomous timing adjustments for the UL only TRP may result in increased timing errors towards the UL only TRP.

If the UE does not perform any autonomous timing adjustment towards the UL only TRP, then the network would be forced to send a greater number of timing advance commands to the UE resulting in larger signaling overheads.

While performing autonomous timing adjustments, the likelihood of timing errors is lower when the UE independently monitors and measures the DL RS from each TRP as compared to when the UE uses only one DL reference timing to perform autonomous timing adjustments to two TRPs.

If the UE doesn’t do autonomous timing adjustments towards the UL only TRP, then the network will need to send a relatively larger number of timing advance commands than it would when the UE performs autonomous timing adjustment.

In general, it would be beneficial to define conditions to determine if UE autonomous timing adjustment applies to just TRP1 or to both TRP1 and TRP2. For example, that condition can be based on if the UL-only TRP transmits RS or on some UE measurements, although other options are not precluded. At least we can consider the case that the UL-only TRP transmit DL RSs such as SSBs, or the case where low mobility is detected at the UE.

- With asymmetric DL sTRP/UL mTRP, specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

It was also left FFS in the last RAN 4 meeting whether to define requirements for FR2. We see no technical justification for defining requirements only for FR1 and not for FR2. Not defining any requirements for FR2 would limit the deployment of this feature to FR1 only which doesn’t fulfil the objective of this work item.

There is no technical justification for not defining timing requirements for FR2.

Not defining any timing requirements for FR2 will result in the feature being only deployed for FR1 which doesn’t fulfil the objective of the WID.

RAN4 to define timing requirements for FR1 as well as FR2.

Discussion on TCI switching requirements

Regarding asymmetric DL sTRP/UL mTRP TCI switching, the following was agreed and discussed in the last RAN4#114 meeting [1]:

Issue 2-3: Whether to define UL TCI state activation requirement based on SRS? Way forward: Further discuss the two proposals and down-select one proposal in the next meeting: - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement. - Proposal 3 (Xiaomi, Nokia): - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP. - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline. - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay. - Further refine the wording for P3 in the WF, if needed.

The uplink TCI state switching delay requirements specified up to Rel-18 have been defined for an uplink TCI state which is associated with a downlink reference signal. The TCI states in an UL-only TRP would be associated with the SRS which is an uplink reference signal.

The existing Rel-18 UL TCI state switching requirements have been defined considering a TRP which transmits downlink reference signal. In the case of asymmetric DL sTRP/UL mTRP, since the uplink only TRP does not transmit any downlink reference signal when PL offset is configured for the UE, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements. Hence, the applicability of the existing uplink TCI state switch delay requirements should be extended to include uplink TCI states which are associated to an uplink reference signal (SRS).

Existing Rel-17 and Rel-18 unified TCI switching requirements are not applicable for target TCI state without DL reference signal.

In the case of asymmetric DL sTRP/UL mTRP, since the uplink-only TRP does not transmit any downlink reference signal when PL offset is configured for the UE, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements.

Update the unified UL TCI state switching delay requirements to be applicable when the target UL TCI state is associated with an uplink reference signal.

An asymmetric DL sTRP/UL mTRP deployment scenario is a single-DCI scenario where the symmetric DL/UL TRP will transmit and receive the PDCCH and the PUCCH respectively. In Rel-18 MIMO evo, RAN4 has defined downlink as well as uplink TCI state switching delay for unified TCI state switch for single-DCI mTRP.

In case of downlink TCI state switching delay requirements, we see no difference in the downlink TCI state switching behavior and hence see no need to define new set of downlink TCI state switch delay requirements.

Similar to the downlink TCI state switching delay requirements, there is no difference in the uplink TCI state switching behavior either when the UE is indicated to switch to a target uplink TCI state associated to the uplink only TRP. Hence, the legacy RAN4 unified TCI state switching delay requirements for UL TCI state can be reused.

Since existing Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework is to be reused for asymmetric DL sTRP/UL mTRP, there is no change in DL and UL TCI state switch delay requirements.

For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI single-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

Although the delay for UL TCI state switch does not need to be changed from the legacy, the requirements still need some updates for the UL-only TRP TCI state switch case.

The legacy TCI state switch delay requirements have been defined based on a TCI state being known or unknown. For a TCI state to be considered as known there are set of conditions defined which are primarily centered around a downlink reference signal being transmitted by the TRP. The known/unknown conditions are used to determine if the UE has reported a valid recent measurement from the target TCI state before making the switch. In case the target TCI state is unknown the UE is expected to perform measurements on the target TCI state, in order to determine transmission parameters such as spatial filter of the target TCI state.

However, in the case of an asymmetric DL sTRP/UL mTRP, since the uplink-only TRP does not transmit any reference signal when PL offset is configured for the UE, this implies that the conditions defined up to Rel-18 for an uplink TCI state to be known cannot be used in the context of an asymmetric DL sTRP/UL mTRP.

Legacy uplink TCI state switching delay requirements are based on a TCI state switch being known or unknown.

Conditions defined for a TCI state to be considered as known or unknown are based on a recent measurement of a downlink reference signal.

For an uplink-only TRP, since there is no downlink reference signal being transmitted when PL offset is configured for the UE, the legacy known/unknown conditions cannot be reused.

RAN4 to define UL TCI state switch delay requirements for an uplink-only TRP with no distinction between known and unknown TCI states.

In the case of an uplink only TRP which does not transmit any downlink RS, the UL TCI state switch delay requirements shall apply when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

It should be noted that the uplink only TCI state switch delay requirements are also dependent on the PL-RS associated with the target TCI state being maintained or not. For example, consider the following case where the UE is configured with UL TCI states #1 and #2 (for the symmetric TRP) and UL TCI states #3 and #4 (for the UL only TRP)

In this example, TCI#1 and TCI#3 are the indicated UL TCI states. The PL-RS #2 associated with TCI#2 and TCI#4 is not maintained. If the UE is instructed to switch from TCI#1 and TCI#3 to TCI#2 and TCI#4, then the UE needs to measure the PL-RS associated with TCI#2 and TCI#4 since this PL-RS is not maintained.

Uplink only TCI state switch delay requirements are also dependent on the PL-RS associated with the target TCI state being maintained or not.

- While defining TCI state switch delay requirements, RAN4 to consider the maintenance status of the PL-RS of the DL TRP associated to the target TCI state of the UL only TRP.

An example of how this can be captured in the specification has been provided in our companion paper R4-XXXXX

Reporting for CJT calibration

General simulation setup discussion

The simulation setup for CJT calibration was agreed on [2], however some points need to be clarified for the simulation alignment. Firstly, the model for the signal generator for the TRP1 and TRP2 need to be clarified. Regarding that, 3 scenarios can be considered:

- Scenario 1: TRP1 and TRP2 transmitting only TRS

- Scenario 2: Data on all the OFDM symbols, except for the symbols carrying TRS.

- Scenario 3: Data on all the OFDM symbols; except for the resource elements carrying TRS.

There can be performance differences between the 3 scenarios. For example, on scenario 3 the ICI can affect frequency offset estimation. On Scenario 2 and 3, the timing offset estimation can be affected by ISI when the timing offset is close to CP length. On the other hand, Scenario 1 might not be realistic enough, since the symbols not carrying TRS are typically also used for PDSCH/PDCCH.

Illustration for Scenario 2 is shown below in Figure 1. Our companion paper with simulation results shows illustrations and some differences for the CJT calibration reporting among the 3 scenarios. Therefore, we think it is important that we consider a realistic scenario 3 for the simulations.

- RAN4 to consider CJT simulation setup with TRP1/TRP2 carrying random data on OFDM symbols not carrying TRS.

Other issue for simulation alignment is regarding TRS configuration. In the previous WF we agreed in a very general TRS configuration, but it is not clear how the TRS resources are used by the 2 TRPs. Therefore, we bring a proposal based on the TDCP. When defining the configuration of TRS we need to discuss how to keep orthogonality of the TRS from TRP1 and TRP2. One way we can achieve that is by using a configuration similar to Figure 1, where the TRS from each TRP uses a different k0 value. The value of k0 in this figure determines the shift in subcarriers for the TRS signal. In this case, by using k0=1 for TRP1 and k0=2 for TRP2 we can avoid the need for the UE to perform interference cancelation across TRPs for the CJT reporting.

Figure 1 Time frequency representation of resources used for TRS for TRP1 with k0=0 and TRP2 with k0=1 using Scenario 2.

Reuse TRS configuration from TDCP for the CJT requirements, with k0=0 for TRP1 and k0=1 for TRP2.

Another issue in the current simulation assumptions is that RAN4 has agreed on 128 quantization levels for frequency offset reporting. However, the latest RAN1 agreements allows for 16, 32, and 256 values. Out of which one last level is used for the “invalid” report, resulting in 15, 31, and 255 actual quantization levels. Therefore, it is important to revise the latest agreement and adopt a value that is supported by specifications. We suggest using 256 levels, since we can infer what would be the accuracy with smaller number of quantization levels based on 256, but not the other way around.

RAN4 to use MFO=256 for the simulation assumptions for FO reporting.

About phase offset, RAN4 has still not agreed on simulation assumptions for phase offset. In our previous paper we showed that phase offset reporting provides good throughput gains on system level. On link level, our companion simulation paper shows the accuracy results for the different scenarios in this paper.

The latest RAN1 agreements allows Mφ to be configured as 16 and 32 for PO reporting. Out of which one last level is used for the “invalid” report, resulting in 15 and 31 actual quantization levels.

Discuss phase offset simulations based on the following assumptions:

  - Quantization level: Mφ = 32 levels.

  - Phase offset between the two TRPs: π

For the TO reporting UE can be configured with MD value equal to 32, 64, 128, and 256. Out of which one last level is used for the “out of range” report, resulting in 31, 63, 127, and 255 actual quantization levels respectively.

Simulation Results

Our link level simulation campaign for Time offset, Frequency offset, and Phase offset reporting for CJT calibration showed that with that it is possible to define the accuracy requirements for all the three reported values. The detailed results can be found in our companion paper.

Scenario 1 shows overly optimistic results for all TO, FO, and PO calibration reporting. The usage of the averaging shows improvement in the accuracy for all the reported offsets. For the TO the accuracy is high enough for the pre-compensation of the combined delay offset for CJT operation in all the scenarios with or without averaging. For the FO and PO averaging increases the accuracy, but it is not clear whether such a high accuracy is required.

It is not necessary to use averaging at least for the Time offset reporting.

Scenario 1 shows overly optimistic results for all TO, FO, and PO calibration reporting.

For the PO reporting at the TRP2 additional phase rotation of 180 degrees was introduced for the waveform signal. For the PO measurement we used non-precoded CSI-RS and measured average relative phase rotation of the reference signals associated with TRP1 and TRP2 respectively. Since 180 degrees rotation was used it is irrelevant whether the quantization error is positive or negative and which TRP is chosen as a reference. In the simulation paper we showed results as 90th percentile of estimation quantization error. In multiple cases that we showed in our simulations results quantization and estimation errors cancelled out resulting in zero error, what can be explained by the fact that 180 degrees is one of the reported levels and estimation error is bellow half of the quantization step width.

Even though PO is mostly used for TDD operation and required to augment the SRS channel measurements, the proposed simplified LLS simulation setup and possible equivalent accuracy test is capable of showing if the UE is capable of accurately reporting the UE’s part of the measurement that is required to pre-compensate relative phase rotation at multiple TRPs. The second part of the PO that is acquired from the SRS measurements to complete the pre-compensation process does not require a UE accuracy test.

RAN4 to define accuracy requirements for TO, FO, and PO CJT calibration reporting.

Conclusion

The following Observations and Proposals were made:

Observation 1: Use of the independent reference timing from both TRPs will ensure that the UL timing is robust for UE movement.

Proposal 1: With asymmetric DL sTRP/UL mTRP, in case the UE is not configured with PL offset (Scenario#2) the UE shall monitor two DL reference timings, one per TAG or TRP.

Observation 2: While performing autonomous timing adjustments, the likelihood of timing errors is lower when the UE independently monitors and measures the DL RS from each TRP as compared to when the UE uses only one DL reference timing to perform autonomous timing adjustments to two TRPs.

Observation 3: If the UE doesn’t do autonomous timing adjustments towards the UL only TRP, then the network will need to send a relatively larger number of timing advance commands than it would when the UE performs autonomous timing adjustment.

Proposal 2: With asymmetric DL sTRP/UL mTRP, specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

Observation 4: There is no technical justification for not defining timing requirements for FR2.

Observation 5: Not defining any timing requirements for FR2 will result in the feature being only deployed for FR1 which doesn’t fulfil the objective of the WID.

Proposal 3: RAN4 to define timing requirements for FR1 as well as FR2.

Observation 6: Existing Rel-17 and Rel-18 unified TCI switching requirements are not applicable for target TCI state without DL reference signal.

Observation 7: In the case of asymmetric DL sTRP/UL mTRP, since the uplink-only TRP does not transmit any downlink reference signal when PL offset is configured for the UE, the existing Rel-18 requirements cannot be reused without changing the applicability of the uplink TCI state switch requirements.

Proposal 4: Update the unified UL TCI state switching delay requirements to be applicable when the target UL TCI state is associated with an uplink reference signal.

Observation 8: Since existing Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework is to be reused for asymmetric DL sTRP/UL mTRP, there is no change in DL and UL TCI state switch delay requirements.

Proposal 5: For the asymmetric DL/UL scenario, reuse Rel-17 and Rel-18 unified TCI single-DCI mTRP TCI state switching delay requirements for DL and UL TCI state switch.

Observation 9: Legacy uplink TCI state switching delay requirements are based on a TCI state switch being known or unknown.

Observation 10: Conditions defined for a TCI state to be considered as known or unknown are based on a recent measurement of a downlink reference signal.

Observation 11: For an uplink-only TRP, since there is no downlink reference signal being transmitted when PL offset is configured for the UE, the legacy known/unknown conditions cannot be reused.

Proposal 6: RAN4 to define UL TCI state switch delay requirements for an uplink-only TRP with no distinction between known and unknown TCI states.

Proposal 7: In the case of an uplink only TRP which does not transmit any downlink RS, the UL TCI state switch delay requirements shall apply when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

Observation 12: Uplink only TCI state switch delay requirements are also dependent on the PL-RS associated with the target TCI state being maintained or not.

Proposal 8: While defining TCI state switch delay requirements, RAN4 to consider the maintenance status of the PL-RS of the DL TRP associated to the target TCI state of the UL only TRP.

Proposal 9: RAN4 to consider CJT simulation setup with TRP1/TRP2 carrying random data on OFDM symbols not carrying TRS.

Proposal 10: Reuse TRS configuration from TDCP for the CJT requirements, with k0=0 for TRP1 and k0=1 for TRP2.

Proposal 11: RAN4 to use MFO=256 for the simulation assumptions for FO reporting.

Proposal 12: Discuss phase offset simulations based on the following assumptions:

a.	Quantization level: Mφ = 32 levels.

b.	Phase offset between the two TRPs: π

Observation 13: It is not necessary to use averaging at least for the Time offset reporting.

Observation 14: Scenario 1 shows overly optimistic results for all TO, FO, and PO calibration reporting.

Proposal 13: RAN4 to define accuracy requirements for TO, FO, and PO CJT calibration reporting.

- References

- R4-2502596, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek

- R4-2502667, LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP, CMCC