3GPP TSG-RAN WG4 Meeting #114bis	R4-2504505

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Analysis of skip ratio on XR capacity and mobility performance

Agenda item:	7.28.2

Document for:	Discussion

System-level simulation results

In the following, we evaluate the impact of skipping on both XR traffic and mobility performance.

Simulation results highlights

The main findings of the SLS evaluation are as follows:

XR capacity is affected by scheduling restrictions caused by measurement gaps. Skipping of measurement gaps is beneficial to minimize the impact of scheduling restrictions on XR capacity. XR capacity depends on  skipping ratio (i.e., number of skipped measurement over a time window) and skipping decision policy can be optimized to find the best trade-off between XR capacity and number of skipped/cancelled measurement opportunities.

There are scenarios where skipping measurement gap occasions has a low impact on mobility performance as long as measurement filtering and UE mobility configuration are properly configured.

Evaluation of skipping on XR Traffic performance in static scenario

Evaluation assumptions

In this subsection, we present the results of system-level simulations (SLS) of the skipping policies discussed in RAN1. Simulations have been performed in the Dense Urban FR1 scenario with parameters configured according to TR38.835. On top of the parameters defined in TR38.835, we configured measurement gaps considering the configurations (40,6), where the measurement gap repetition period (MGRP) is 40ms and the measurement gap length is 6ms. In our evaluation, we consider the following KPIs:

XR capacity: system load CP of policy P in terms of number of UEs/cell (N) when the ratio of satisfied UEs/cell RP is above or equal to 90% (CP=maxN:RP≥90%. A UE is marked as satisfied if 99% of XR frames are delivered within the packet delay budget (PDB) of 10ms (for VR/AR) or 15ms (for CG). The KPI is reported by computing the average across all UEs, all cells, and all SLS drops.

Skipping ratio: fraction of skipped measurement opportunities with respect to the total number of measurement opportunities defined by the measurement gaps configuration. The KPI is reported by computing the average across all UEs, all cells, and all SLS drops.

XR capacity loss: loss of XR capacity measured considering the system load when MG is disabled.

XR capacity gain: gain of XR capacity measured considering the system load when MG and skipping policies are enabled.

We evaluate the following skipping policies (alternatives):

Explicit DCI indication (Alt. 1-1): the logic of the skipping decision is implemented by the network and the skipping decision is dynamically delivered with dedicated DCI signaling to the UE. To this end, we implemented two policies for the skipping decision (i.e.,  to decide when to trigger DCI skip indication):

  - Policy presented in R4-2416468 during RAN4#112bis decides to skip the next measurement gap if the traffic is expected/predicted within a time window (TP) prior to a measurement gap, provided that the DCI can be received by the UE offset time units (TSO) prior to a measurement gap.

  - A random skipping policy that randomly skips measurement gaps according to an input skipping ratio (i.e., the skip ratio represent the probability of skipping the next measurement gap).

Additionally, we compare the skipping policies against the two following baseline configurations:

No MGs: measurement gaps are disabled. This corresponds to the ideal case where scheduling restrictions are not delaying scheduling decisions. This configuration provides an upper bound in terms of XR capacity.

MG enabled: measurement gaps and corresponding scheduling restrictions are enabled. This configuration provides a lower bound in terms of XR capacity.

Main parameters and results

In our SLS, we evaluate several configurations including the offset for retuning the radio interface (TSO) and parameters specific to the alternative solution. Table 2 in Section 3 summarizes the values of the parameters used in our analysis. Figure 4 shows the ratio of satisfied UEs , which is measured when the cell load is equal to 7 UEs/cell (i.e., the capacity when MGs are not configured), versus the skipping ratio when TSO={1, 3, 5} ms. Arrows in the figure depict the change in performance versus the change in skipping ratio. Dotted, dashed, and solid arrows correspond to offset values TSO=1ms, TSO=3ms, and TSO=5 ms, respectively.

In the presence of scheduling restrictions, results show that the system should skip approximately 70% of measurement occasions to fully recover the same XR capacity as when scheduling restrictions are disabled. When using Explicit DCI indication (Alt 1-1), XR capacity loss can be made negligible in some scenarios. In particular, it can be observed that the larger the skipping ratio is, the larger the capacity is in this static scenario. For example, with TSO=1ms the Alt 1-1 skipping policy can be configured to achieve the same number of satisfied XR UEs as without scheduling restrictions (black point w/o MG), which corresponds to skipping all gaps. It can be further observed that the parameters of the Alt 1-1 skipping policy can be optimized to maximize the number of satisfied XR UEs and minimize the skipping ratio. For example, Alt 1-1 with TSO=1 (green triangle pointing up) can be configured to have the same skipping ratio of Alt 1-1 when TSO=3ms (magenta triangle pointing down) or TSO=5ms (blue triangle pointing right), namely 44%. Additionally, as illustrated by the arrows in the picture, parameters of the skipping decision policy like TP can be optimized to reduce the skipping ratio with negligible performance impact on XR capacity. For example, when offset TSO=5ms, the skipping ratio moves from 44% down to 32% with a loss of 5% for the ratio of satisfied UEs (i.e., for 12% decrease of skipping ratio, the network can minimize the impact of capacity loss to 5%). In particular, the ratio of satisfied UEs remains well above 80%, which was considered a negligible capacity loss in Rel-17 SI on XR evaluation for DRX analysis. Therefore, the network can optimize the skipping decision policy to minimize the skipping ratio and no restriction on the skipping ratio is needed.

No restriction on the skipping ratio is needed because skipping decision policy can be optimized to minimize the skipping ratio with negligible impact on XR capacity.

Figure 5 – Static scenario: Ratio of satisfied UEs vs skipping ratio for Alt 1-1 when TSO={1, 3, 5} ms. Ratio of satisfied UEs has been measured when cell load is equal to 7 UEs/cell (i.e., the XR capacity of ‘w/o MG’ when PDB=10ms).

Table 1 below illustrates the capacity for different PDBs (10ms and 15ms as per TR38.835) and the skipping ratio of the alternatives we have considered in our SLSs. We also evaluated the same KPIs with legacy MGs (label “MGs enabled”) and without any MG configured (label “No MGs”).

In the presence of scheduling restrictions, results show that the system should skip approximately 70% of measurement occasions to fully recover the same XR capacity as when scheduling restrictions are disabled. When using Explicit DCI indication (Alt 1-1) the system experiences the largest capacity gain (in some configurations up to 75%). Skipping ratio for Alt. 1-1 can be reduced down to 26% (for Alt. 3 it is around 43%) while still maintaining high capacity gain (50% for Alt. 1-1).

More specifically, in our SLS evaluation summarized in Table 1 for measurement gap configuration (40, 6) ms, we show that Explicit DCI indication (Alt 1-1) achieves the best performance among all alternatives. For example, Explicit DCI indication with TB=3ms and TP=4ms achieves XR capacity of 7 UEs/cell and capacity gain of 43% with PDB=10ms. Even with large UE skipping retuning time (TSO=5 or 6 ms) the capacity gain can be kept around 50% and the skipping ratio can be minimized down to 26%. This is obtained by combining the prediction-based and the buffer-based skipping strategies as well as the flexibility provided by the dedicated DCI skipping indication.

Evaluation of skipping on mobility performance

Evaluation assumptions

In this subsection, we present the results of system-level simulations (SLS) of the skipping policies discussed in RAN1. Simulations have been performed in the Dense Urban FR1 scenario with parameters configured according to TR38.835. On top of the parameters defined in TR38.835, we assumed the following configurations:

- Three measurement gaps configurations: (40,6), (80,6), (160,6) where the gap length is 6ms and the interval is increased from 40ms to 160ms in order to reduce the retention of old measurements in L1 filtering.

- UE mobility enabled and configured according to TR38.838. In particular, the parameters in Table 2 have bene considered in our evaluation.

Table 2 – Configuration of UE mobility

| Parameter | Value(s) |
| --- | --- |
| UE speed | 50 km/h |
| Mobility model | Moving in a random direction in straight lines with constant speed (with 0 turn probability) |
| Handover | Baseline L3 handover |
| Handover interruption time | 40ms |
| Measurement event | Intra-frequency RSRP A3 based (measured on SSB) 1dB hysteresis, 2dB A3 offset, 64ms TTT (optional value 160ms) Preparation and execution time = 40, 40ms |
| RLF configuration | 4 N310, 4 N311, -6dB Qin, -8dB Qout, 1000ms T310 |
| L1 sliding window size | 5 |
| L3 filtering coefficient | 1 |

Figure 3 and Figure 4 shows the number of handovers (HOs) and the number of Ping-Pong as a function of the skip ratio for multiple values of MGRP and average cell load (measured as average UEs per cell). We observe that number of HOs coincides with the number of successful HOs. The number of HOs and ping pongs increase when the gap interval increases as well as when the number of UEs per cell increases. This is an expected result of increasing the measurement delay as a result of measurement skipping.

Figure 5 shows the simulation results of radio link failures as a function of the number of users and skipping ratio. When observing the ping pong statistics in combination with the RLF statistics, we can conclude that in the simulated scenario the impact of measurement skipping was reflected in an increase of handover ping pongs, however this was not increasing RLF statistics in a significant manner.

Therefore, there are scenarios where skipping measurement gap occasions has a low impact on mobility performance as long as the measurement filtering and UE mobility configuration is properly configured. For example, if the L1 sliding window is configured to retain at minimum number of L1 measurements, then the impact of skipping measurement gap occasions on mobility performance like number of HOs can be minimized. This is visible when we gap interval is set equal to 160ms, which reduces L3 filter responsiveness to channel fluctuations.

Skipping measurement gap occasions has a low impact on mobility performance for some scenarios as long as UE mobility configuration is properly defined/configured.

Figure 3 – Number of HOs as a function of the skip ratio for different MGRP and average cell load (measured as average UEs per cell).

Figure 4 – Number of Ping-Pong as a function of the skip ratio for different MGRP and average cell load (measured as average UEs per cell).

Figure 3 –Number of RLFs per UE as a function of the skip ratio for different MGRP and average cell load (measured as average UEs per cell).

Conclusion

The results above show

- It is beneficial to increase skipping ratio above 50% in some scenarios

- Skipping ratio of up to 75% could be achieve without significant impact on mobility in the scenario agreed in RAN1 for XR evaluation

  - Disclaimer: the same conclusion cannot be extended to any scenario, e.g. the same may not be extended to a deployment with large ISD or with a large number of frequency layers to measure. However, the limitation of skipping ratio is a task for the network deployment to optimize skipping ratio on a case by case basis.

Based on the above observations and results, we propose in our companion contributions R4-2504508 [1] that RAN4 should not define requirements that limit skipping ratio, since the network can optimize the skipping decision policy to achieve satisfying XR capacity gain (or equivalently negligible XR capacity loss) and at the same time it can minimize skipping ratio by optimally tuning the logic.

The following Observations and Proposals were made:

Observation 1: No restriction on the skipping ratio is needed because skipping decision policy can be optimized to minimize the skipping ratio with negligible impact on XR capacity.

Observation 2: Skipping measurement gap occasions has a low impact on mobility performance for some scenarios as long as UE mobility configuration is properly defined/configured.

- References

- R4-2504508, Discussion on XR ph3 RRM requirements, Nokia