TSG-RAN Working Group 4 (Radio) meeting #114-bis	R4-2504515

Wuhan, China, 7th to 11th April 2025

Source:	Ericsson

Title:	TS 38.141-2 Annex I: Further discussion on improvement of TRP measurement methods

Agenda item:	7.13.5.3

Document for:	Discussion

# Introduction

One of the work areas of the work item on NR base station (BS) RF requirement evolution for FR1/FR2 and testing simplification [1], targets the BS TRP estimation methods in Annex I in TS 38.141-2. The goal is to improve applicability and description. As this annex is normative, it is imperative to clearly describe how the manufacturer performs the TRP measurement and what are the criteria for deciding compliance to the TRP limits.

In a previous contribution [2] [3] we highlighted the most important issues, some being received as part of feedback from regulators during drafting of the harmonised standard in Europe and IEEE/ANSI C63.26 in US.

At many places in Annex I of TS 38.141-2 there are alternatives of measurement grids that are supposed to produce similar outcomes (at least according to the MU analysis in the TR 37.941). There are probably even more alternatives which are not included at this moment. This range of choices is not appreciated by regulators, as it offers a large number of choices without being clear what is normative and how to decide a pass or fail in different cases.

It would be beneficial to agree on one procedure that gives an accurate TRP estimate as a normative reference, while other methods can be informative and applicable if they are utilized according to the guidelines described in TR 27.941.

# Discussion

#### One method, many grids

The way Annex I is organized seem to give the impression that there are many test methods to pick from and the end result can be different, depending on the choice. Even if this is not necessarily the truth, there are risks associated with presenting the alternatives in this manner. First, we need to make a difference between “grid” and “method”. Picking the right grid is part of the method based on use of anechoic chamber and measurement points on a spherical grid. The measurement “method” also includes the post processing of the data, which is necessary to estimate the TRP.

NOTE 1: To better exemplify the difference we point to use of “reverberation chamber”, or PWS, as a different method, for example.

We propose that there should be one reference grid, preferably the one called the “Spherical equal angle grid” with angular steps not exceeding the “reference steps”. Any other grid can be also used as long as it can be proven that it gives either the same TRP result withing acceptable tolerances or an overestimate of the TRP, where applicable.

NOTE 2: For all unwanted emissions, an overestimate serves well, while for the BS radiated power the TRP value should be as accurate as possible.

The equi-angular spherical grid is well understood and widely studied in literature. The analysis is based on Spherical wave expansions wherein the dimensions of the device in terms of minimum sphere and cylinder determines the needed angular sampling. The reference steps correspond to λ/2 sampling on the minimum sphere and cylinder, respectively. Using reference steps on a spherical grid gives accurate TRP results.

One way to compare this reference grid with alternative grids is by defining a specific set of array models and investigate the outcome. As an example, the following parameters could be given

- Array lattice geometry

- Correlation level of antenna weights (for in-band and spurious domain)

- Beam steering directions

The output of such a model should be the EIRP value in an arbitrary set of points (x,y,z) in space. A set of such models could then be used to test whether a TRP method delivers results within a reasonable uncertainty.

Proposal 1: Agree on one single reference method for accurate TRP, we propose to be based on using the full anechoic chamber (CATR included) and a “ Spherical equal angle grid” with “reference angular steps”.

Proposal 2: Clean up Annex I of redundant grids and other information, add a note that alternative sampling grids and techniques/chambers can be used, as long as they are proved to lead to the same outcome as the reference method. Keep detailed descriptions in the TR 37.941. An alternative is to have a new informative Annex containing all these grids.

#### Pre-scan

When assessing the compliance towards spurious emissions limits, pre-scan is the natural starting point to identify frequency ranges that need special attention, but also to shorten the testing time by eliminating frequency ranges that do not present significant unwanted emissions. Currently, pre-scan it is described in the last sub-clause (I.13) of Annex I. The technical details of the pre-scan are missing, especially steps 1) and 2), i.e., instrument settings (frequency step, RBW, …) and angular steps. How to set up the pre-scan by using a turntable to rotate the EUT is not described. Handheld test antenna may be out-dated, since the actual TRP measurement for a BS with significant output power calls for using positioner-controlled measurements.

An interesting alternative is to define a pre-scan method based on reverberation chamber. Using beam-sweeping during a pre-scan can also bring additional benefits in term of reducing the test time and improving the result accuracy.

When it comes to polarization, there is a potential 3dB discrepancy between measuring the case linear polarizations and the total EIRP of a circularly polarized emission. The polarizability of the emission is a topic that is not described at all. If the device transmits two independent signals on two orthogonal polarizations the polarization is undefined, for example.

Proposal 3: Add complete details on the Pre-scan so that it can become a trustable starting point when assessing compliance to spurious emissions limits.

Proposal 4: Consider use of beam sweeping in the pre-scan, as a mean to speed-up the identification of frequency ranges of interest för TRP measurement.

Proposal 5: Consider a pre-scan method based on reverberation chamber.

#### TRP uncertainty

The uncertainty of the TRP using EIRP measurements in anechoic chambers (FAC, CATR or PWS), can be divided in three categories

- EIRP errors (calibration and measurement),

- Finite angular step,

- Summation over angular grid

Note 1: The EIRP errors are already covered in the MU tables for EIRP and the corresponding chamber type. These errors can be made smaller by improving the power calibration using e.g. power sensors.

Note 2: The reference steps are decided to have negligible sampling error in the case of “full sphere – reference steps” case. Sparse sampling is taken care of by using the ΔTRP correction factor.

Note 3: The post-processing error is the effect of random errors of the EIRP values when summed to get a TRP estimate. It is well known that the effect of summing values with random errors results in a decrease of the relative error, i.e., the uncertainty in TRP is most probably lower than the uncertainty in EIRP. Hence, the Summation error should be removed. This can be further studied if desired. This value was decided without an appropriate error propagation model.

Proposal 6: Investigate the effect of random errors of EIRP values on TRP by using the sum formula in Annex I clause 1.2.1.

# Summary of proposals

Proposal 1: Agree on one single reference method for accurate TRP, we propose to be based on using the full anechoic chamber (CATR included) and a “ Spherical equal angle grid” with “reference angular steps”.

Proposal 2: Clean up Annex I of redundant grids and other information, add a note that alternative sampling grids and techniques/chambers can be used, as long as they are proved to lead to the same outcome as the reference method. Keep detailed descriptions in the TR 37.941. An alternative is to have a new informative Annex containing all these grids.

Proposal 3: Add complete details on the Pre-scan so that it can become a trustable starting point when assessing compliance to spurious emissions limits.

Proposal 4: Consider use of beam sweeping in the pre-scan, as a mean to speed-up the identification of frequency ranges of interest för TRP measurement.

Proposal 5: Consider a pre-scan method based on reverberation chamber.

Proposal 6: Investigate the effect of random errors of EIRP values on TRP by using the sum formula in Annex I clause 1.2.1.

# References

[1]	RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC

[2]	R4-2419585, “Improvement of TRP measurement methods in TS 38.141-2, Annex I”, Ericsson

[3]	R4-2501647, “TS 38.141-2 Annex I: Discussion on improvement of TRP measurement methods”, Ericsson

DRAFT PROPOSAL:

Annex I (normative):
TRP measurement procedures

# I.1	General

This clause describes:

- the reference method for performing TRP measurements

- an optional pre-scan procedure that provides a way to identify the frequency ranges of interest for the TRP measurements, in the spurious domain.

Alignment between EUT and measurement antenna is important to achieve expected measurement uncertainty for TRP measurements:

1.	The measurement antenna needs to be aligned tangential to the measurement surface forming a sphere around the EUT, in order to correctly measure the TRP properly.

2.	Test methods require angular alignment between the selected measurement grid and EUT radiation pattern in order to measure peak values in the main beams. Angular misalignment can lead to differences in the actual and measured angular positions of the intended maximum EIRP.

NOTE: Alternative TRP test methods described in TR 37.941 (or Annex YYY (informative) ) can also be employed if the test set-up conditions allow and it can be proven that the outcome is similar to the reference method.

- I.2	Spherical equal angle grid

- I.2.1	General

TRPEstimate is defined as

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

when EIRP measurements is used or as

TRPEstimate=4πd2π2NMn=1N-1m=0M-1PD(θn,ϕm)sinθn

when power density measurements are used, and d is the test distance. N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM. The sampling intervals Δθ and Δϕ are described in I.2.2.

- I.2.2	Reference angular step criteria

The reference angular steps Δϕref and Δθref in degrees are defined as:

Δϕref= min⁡(180∘πλDcyl,15∘)

Δθref=⁡min(180∘πλD,15∘)

The upper limit for these reference angular steps of 15∘ ensures a low Summation Error (SE) when λ is large compared to the EUT dimensions.

Dcyl and D are calculated as:

`	Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in figure I.2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure I.2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

≈0

∆θref=min(180°πarcsin⁡(λDz),15°)

Δϕref=min(180°πarcsin⁡(λDy),15°)

Figure I2.2-2: Spherical coordinate for OTA conformance testing of EUT

SF=maxΔθgridΔθref,ΔϕgridΔϕref

Δθgrid Δϕgrid

Alternatively, when EUT radiating dimensions are not known, for each frequency within the downlink operating band including ΔfOBUE, the reference angular steps can be specified in terms of the beamwidth of the wanted signal as

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

BeWφ and BeWθ may be set to beamwidth declared for the OTA BS radiated transmit power requirement provided the same beam is applied to test in-band TRP requirements.

NOTE:	Beamwidth is approximately equal to half the first-null beam width.

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

ΔθrefΔϕrefθn,ϕn

TRPEstimate=1Nn=0N-1EIRP(θn,ϕn)

ΔθrefΔϕrefθn,ϕnθnϕn

θn=cos-11-2n+1N

ϕn=360°nΨ,  Ψ=1+52

EIRPav,cut-n=1Pj=1PEIRP(j)

TRPEstimate=1Nn=1NEIRPav,cut-n

xz

EIRPcut1ϕθ=θH EIRPcut2θϕ=0

u=sinθsinϕv=cosθ

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

u2+v2=1

Δuref=λDy

Δvref=λDz

u=sinθsinϕ, v=cos⁡(θ)

TRPestimate=ΔurefΔvref4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

SF=max∆ugrid∆uref,∆vgrid∆vref

∆ugrid∆vgrid

ΔϕΔθ

SF=maxΔϕΔϕref,ΔθΔθref

ΔTRP=SF-1SFmax-1⋅1.0 dB,

SFmax

ΔϕrefΔθref

TRPEstimate=EIRPpeakDEUTEIRPpeakDEUT

ϕ

TRPEstimate=1Kk=1KEIRPkEIRPk

- I.13	Pre-scan

Pre-scan is an optional first step in identifying the frequencies with unwanted emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP, but offers a criteria to declare compliance to the limit based on coarse measurements around the object and employing a large margin towards the limit. An emission frequency identified by a pre-scan shall be further investigated by any of the TRP measurement methods in this annex.

[more details needed about test model and test configuration, especially with respect to polarization (polarization state can be random for spurious emissions), measurement test antenna parameters (e.g. to be able to illuminate the whole EUT), measurement distance (e.g. far-field of the measurement antenna), condition on the dynamic range and noise floor]

Measurement antenna shall be able to illuminate the whole EUT and the measurement distance shall be larger than the Fraunhofer far-field distance of the measurement antenna. Noise floor in the test set-up shall be at least 30 dB under the specified limit.

The procedure for pre-scan is as follows:

1)	Scan the entire surface around EUT. [more details are needed on the measurement distance, antenna, angular step size, MU, etc.]

2)	Rotate test antenna to cover all possible polarizations of emissions and identify the maximum levels readings

3)	Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur.

4)	Emissions  presenting maximum levels reading that are 20 dB or more below the specified limit shall not require further measurements. This criteria is sufficient to prove compliance to the limit.

5)	Emissions presenting maximum levels reading that are less than 20 dB below the specified limit shall be subject to further investigations.