3GPP TSG-RAN WG4 Meeting #114bis					R4-2504519

7th Apr, 2025 – 11th April, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on 8Rx UE demodulation and CSI requirements with inter-cell and intra-cell inter-user interference

Agenda item:	 7.18.2

Document for:	Discussion

# 1.Introduction

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

In Rel-18 topic NR_ENDC_ RF_FR1_enh2 WI, 8Rx performance requirements for SU-MIMO (single user) were discussed. In Rel-19, 8Rx performance requirements with inter-cell, and intra-cell inter-user interference to be defined for below objectives [1].

### In [2], RAN4 agreed on some of the issues, and encouraged companies to run simulations with Rel-17 interference settings to further discuss considered scenarios. In this paper, we share some of our observations based on simulation results and share our proposals

# 2. PDSCH Demodulation Requirements inter-cell interference (ICI)

## 2.1 Simulation results

### Initial simulation results for inter-cell interference FDD and TDD scenarios are respectively shared in Table 2.1, and Table 2.2 with following test parameters.

Simulation settings:

- FDD, 15 kHz SCS, 10 MHz BW or TDD, 30 kHz SCS, 40 MHz BW

- TDA30-10, ULA Low

- Rel-17 interference settings with rank1 or rank2

- Interference power levels are 7.58 dB for 1 interferer, 7.77 dB and 2.28 dB for 2 interferers

- Antenna configuration: Rank1: 2 x 8; Rank2: 2 x 8; Rank4: 4 x 8

- SNR values @ 70% of maximum throughput

Table 2.1 FDD simulation results

| Setting | Channel | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) | Gap = Simplified – Baseline (dB) |
| --- | --- | --- | --- | --- | --- |
| Serving cell Rank 2 + 2 interfering cells | TDLA 30-10 | 17 | 9.1 | 11.3 | 2.2 |
| Rank 4 serving cell + 1 interfering cell | TDLA 30-10 | 13 | 9.1 | 13 | 4.3 |

Table 2.2 TDD simulation results

| Serving cell | Channel | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) | Gap = Simplified – Baseline (dB) |
| --- | --- | --- | --- | --- | --- |
| Serving cell Rank 2 + 2 interfering cells | TDLA 30-10 | 17 | 9.1 | 11.1 | 2 |
| Rank 4 serving cell + 1 interfering cell | TDLA 30-10 | 13 | 8.7 | 12.9 | 4.2 |

# 3. PDSCH Demodulation Requirements intra-cell inter-user interference

In [2], RAN4 agreed to have Rank2 requirements, and Rank4 case was still under discussion. The MMSE-IRC covariance matrix computation fundamentally stays the same for both inter-cell interference and intra-cell inter-user interference cases. The key aspect of interference + noise estimation and then whitening is common for both scenarios. Therefore, our view is that scenarios can be similar in defining requirements for intra-cell inter-user (MU-MIMO) too.

## 3.1 Rank choice

Rank4 for both serving and interfering cell requires TD-OCC DMRS (MAXLEN = 2) support which is an optional feature. This implies all the UEs which don’t support this capability are not going to be verified with the requirement. Moreover, such DMRS scheme has a higher overhead and is not a preferred option for network operators in our view. Furthermore, such DMRS scheme is not considered in RAN4 scenarios other than 8L, 8Rx SU-MIMO requirement. Rel-18-enhanced DMRS is a more suitable option to further consider Rank4 case to balance DMRS overhead. Considering above two aspects, we are of the view to not define Rank4 requirements.

Proposal 1: RAN4 shall not define MU-MIMO requirements for Rank4.

## 3.2 Simulation results for Rank2 MU-MIMO

Initial simulation results for Rank2 for both target UE and co-scheduled UE are shared in Table 3.1.

Table 3.1 Intra-cell and Inter-user interference simulation results

| Setting | Channel | Duplex | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) | Gap = Simplified – Baseline (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Rank2 Target UE + Rank2 co-scheduled UE | TDLA30-10 | FDD | 17 | 8.2 | 11.8 | 3.6 |
| Rank2 Target UE + Rank2 co-scheduled UE | TDLA 30-10 | TDD | 17 | 8.3 | 11.8 | 3.5 |

# 4. CQI requirements

In [2] Rank1 and Rank2 from serving cell along with Rel-17 interference settings were agreed for initial simulations. Table 4.1 has the simulation results.

Table 4.1 Gamma values for CQI scenarios

|  | Rank1 serving cell |  |  | Rank2 serving cell |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SINR (dB) | Baseline Gamma (A) | Simplified Gamma (B) | Gap (A - B) | Baseline Gamma (A) | Simplified Gamma (B) | Gap (A - B) |
| -3.0 | 2.8 | 2.9 | -0.1 | 4.0 | 3.9 | 0.1 |
| -2.0 | 2.7 | 2.4 | 0.2 | 3.8 | 3.6 | 0.2 |
| -1.0 | 2.2 | 2.1 | 0.1 | 3.2 | 2.7 | 0.5 |
| 0.0 | 2.2 | 2.2 | 0.0 | 3.2 | 2.9 | 0.3 |

Since SINR -1 dB is appropriate for cell-edge case, and rejecting rank1 interferer with rank2 from serving cell is not challenging at all with 8Rx, these settings can be considered for CQI requirements. From above results, for SINR -1 dB with Rank2, the gap between gamma of baseline and simplified is 0.5. The gap is wide enough to have independent requirements for both receiver types.

Proposal 2: RAN4 to use Rank2 serving cell with Rel-17 settings for 8Rx CQI requirements.

Proposal 3: RAN4 shall have independent gamma requirements for baseline and simplified receivers.

# 5. Applicability rules

## 5.1 Skip 2Rx/4Rx tests

In [2], the following issue on applicability rules was still open.

Issue 1-5-1: Test applicability to skip 2/4Rx requirements

We propose the following applicability rules to keep limited overall number of test cases to be passed by UE. Since the agreed 8Rx ICI scenarios have Rank2 and Rank4 for both receiver types, Rank1 requirements of Rel-17 with 2Rx/4Rx can be skipped if 8Rx requirements are met. Similarly, Rank1/2 Rel-17 requirements can be skipped if 8Rx MU-MIMO Rank2 requirements are met.

Proposal 4: RAN4 shall have the following applicability rule: UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx.

Proposal 5: RAN4 shall have the following applicability rule: UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx.

## 5.2 MMSE-IRC receiver capability definition

Issue 1-1-1: Definition of MMSE-IRC receiver capability for 8Rx UE

- Option 1: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 2: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 3: 
Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability

For the open issue below in [2], we support Option 3 since the support of 8 layers for baseline receiver is intended for SU-MIMO only.

Proposal 6: Rel-19 requirements are applicable for UE supporting SU-MIMO 8Rx receiver capability.

# 6. Release independence

Since interference requirements are being discussed in Rel-19 feature, release independence shall be considered from Rel-18.

Proposal 7: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# 7. Conclusions

Following is the summary of the proposals:

Proposal 1: RAN4 shall not define MU-MIMO requirements for Rank4.

Proposal 2: RAN4 shall have Rank2 serving cell with Rel-17 settings for 8Rx CQI requirements.

Proposal 3: RAN4 shall have independent gamma CQI requirements for baseline and simplified receivers.

Proposal 4: RAN4 shall have the following applicability rule: UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx.

Proposal 5: RAN4 shall have the following applicability rule: UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx.

Proposal 6: Rel-19 requirements are applicable for UE supporting SU-MIMO 8Rx receiver capability.

Proposal 7: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# References

[1] RP-241297, “Revised WID on NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #104, June 17– June 20, 2024, Shanghai, China.

[2] R4-2502275, “Way Forward for [114][324] NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #114, Feb 17– Feb 21, 2025, Athens, Greece.