3GPP TSG-RAN WG4 Meeting #114bis	R4-2504521

Wuhan, China, 7th March – 11th March 2025

Agenda item:	7.18.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on BS demodulation for NR MMSE-IRC.

Document for:	Discussion

- Introduction

In the last meeting RAN4 #114, it was discussed that RAN4 demodulation spec on NR MMSE-IRC and the simulation results [1]. This contribution proposes our views on open issues.

- Discussion

According to RAN4#114 WF [1], there are still open issues to be discussed.

Manufacturer declaration on MMSE-IRC receiver - Candidate options: - Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver Declaration identifier; Declaration; Description; Applicability. BS type 1-C; BS type 1-H. D.xxx; MMSE-IRC receiver; Declaration of BS supporting MMSE-IRC receiver.; x; x. - Proposal 2: Do not need to introduce a new Manufacturer Declaration on MMSE-IRC

It is common understanding that many vendors have already implemented NR MMSE-IRC in their BS. Thus, we don’t need to introduce a new manufacturer declaration for NR MMSE-IRC.

Observation 1: NR MMSE-IRC is widely deployed in existing BS.

Proposal 1: Introducing a new manufacturer declaration for NR MMSE-IRC is not needed.

Applicability - Candidate options on test requirement applicability for different BS classes and test scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS - Other options are not precluded. - Based on test requirement applicability, further discuss whether to consider additional applicability rule on how the test is performed: - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Proposal 2: If a BS declared to support HomNet or HetNet and pass the tests for HomNet, can also consider the test for HetNet as passed - Proposal 3: Do not define additional applicability rule for homogenous and heterogenous network scenarios

For LTE BS, the homogeneous network (HomNet) demodulation requirements are not applicable for LA class and home class. We think that additional applicability for NR LA class BS, NR BS does not define Home BS, is not needed, but we should clarify the applicability of HomNet requirements. Thus, we introduce the applicability rule that the demodulation requirements of HomNet are applicable for WA BS.

Proposal 2: Introducing applicability rule that the requirements of HomNet are applicable for NR WA BS.

- Conclusion

This contribution states general aspects of NR BS MMSE-IRC. The summary follows.

Observation 1: NR MMSE-IRC is widely deployed in existing BS.

Proposal 1: Introducing a new manufacturer declaration for NR MMSE-IRC is not needed.

Proposal 2: Introducing applicability rule that the requirements of HomNet are applicable for NR WA BS.

References

- R4-2402272, Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#114