3GPP TSG-RAN WG4 Meeting #114bis            	R4-2504539

Wuhan, China, 7th-11th, April, 2025

Agenda item:	7.19.2

Source:	Samsung

Title:	View on CSI reporting requirements framework for CSI prediction

Document for:	Discussion

- Introduction

In the last RAN4#114 meeting, CSI reporting requirement framework for CSI prediction was under discussed. The related agreed agreements are captured in the WF [1] as following

- CSI reporting requirement framework for CSI prediction - Model for CSI prediction - Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model - Training and test data consistency - Further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data (simulation) and test data (conformance test) and whether anything special be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations - Generalization - Postpone this discussion until the requirement and test definition is clear

Meanwhile, in order to facilitate the AI/ML model evaluation and alignment, the initial simulation assumption is agreed and captured in the [2]

In this contribution, the view on CSI reporting requirements framework for CSI prediction was provided. Meanwhile, the initial simulation results for AI/ML based CSI prediction calibration and comparison were provided.

# 2	LCM related requirement

Performance Monitoring

Based on revised WID, one of objective for RAN4 is to specify the necessary core requirements for LCM procedures including performance monitoring.

In RAN1#114, the above agreement was agreed for performance monitoring for AI/ML based CSI prediction at the UE side.

Agreement in RAN1#114 For CSI prediction using UE-sided model, for performance monitoring, at least following aspects have been proposed by companies on performance monitoring for functionality-based LCM - Type 1 - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2 - UE reports the predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting) - Type 3 - UE calculates the performance metric(s) - UE report performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting) - Functionality selection/activation/deactivation/switching what is defined for other UE side use cases can be reused, if applicable - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report - Note: down selection is not precluded - Note: UE may make decision within the same functionality on model selection, activation, switching operation transparent to the NW

Moreover, in RAN1#1, the above was agreed to clarify the boundaries between the three Types and the corresponding specification impact.

Agreement in RAN1#117 For the boundary between type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843 - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism

Furthermore, based on the agreement in RAN1#120 meeting, the calculation of monitoring metric was further clarified

Agreement in RAN1#120 For CSI prediction using UE-sided model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support - Based on intermediate KPI - Down select between SGCS and NMSE by RAN1#120bis - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric

For type 1 monitoring, UE may calculate the metric and report the monitoring output to the network. To facilitate UE reporting, the related CSI-RS resource shall be configured for measurement as predicted and monitoring, including the configuration f prediction and monitoring window.

For type 2 monitoring, gNB may compare the prediction CSI and ground truth CSI to determine the performance of the prediction. gNB may need to know the ground truth CSI based on UE report or measurement from Uplink.  To adjust the prediction window and other parameters, the network may also require ground truth CSI corresponding to multiple time instances within the prediction window of the predicted CSI. To report for target CSI may depend on UE’s capability may affect the CSI processing unit counting and its timeline. Therefore, the related CSI-RS resource shall be also configured, as well as the CSI processing timeline for predicted CSI reporting and ground truth CSI.

From RAN4 perspective, for type 1 and type 3 monitoring, the UE reported performance monitoring related information shall be tested in RAN4, the testability of performance monitoring metric or output shall be further studied.

For type 2 monitoring, gNB needs to calculate performance metric including at least intermediate KPI, such as SGCS, by comparing the current channel from the latest report with the predicted channel from the previous report.

For CSI prediction accuracy, RAN4 has agreed to apply the relative throughput as requirement metric. While, for performance monitoring metric, although RAN1 agreed to use intermediate KPI, such as SGCS, the similar level for testing should also be discussed to ensure consistency and accuracy of monitoring metrics.

Meanwhile, for type 2 monitoring, RAN4 may further study whether to define requirement for both predicted CSI and reported ground truth CSI at the same time.

Based on revised WID, the normative work in RAN1 for CSI prediction will start from Q1 of 2025. RAN4 impact for performance monitoring will start to discuss after the sufficient progress in RAN1.

Proposal 1:   Further study performance monitoring related requirement, and testability according to the sufficient progress in RAN1.

# 3	CSI reporting requirement

## 3.1	Test Conditions

SNR test point

In the previous meeting, for CSI prediction testing, it was agreed that the reference throughout is obtained based on the random PMI with Rel-15 Type 1 single panel codebook, which is the same approach for legacy PMI reporting test for different codebook requirements.

In legacy PMI test, the PMI reporting requirement are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to UE reported PMI compared to the case when the transmitter is using random precoding, in terms of the ratio as following

Where the  is 90 % of the maximum throughput obtained at  using the precoders configured according to the UE reports.

Same as legacy PMI test, for AI/ML CSI prediction testing, the same SNR test point with 90% of maximum throughout can be considered as baseline, it should be sufficient to check the functionality and ensure the proper performance with predicted CSI.

Proposal 2:  RAN4 can consider to apply the SNR test point related 90% of maximum throughput achieved with following PMI similar as legacy PMI test for AI-based CSI prediction as baseline.

Codebook for AI/ML based CSI prediction

Regarding the codebook configured for AI-based CSI prediction, there is no conclusion yet in RAN4#114 meeting, considering both Rel-16 Type II and Rel-18 eType II have still been evaluated in RAN1 for AI/ML based CSI prediction, and non-AI/ML based CSI prediction, so, it will be further decided pending on RAN1 progress. Based on RAN1 agreement in the last meeting as following, RAN1 agreed to apply Rel-18 doppler codebook to be used for AI/ML-based CSI prediction

Agreement in RAN1#120 For CSI prediction using UE-sided model, at least for inference, Rel-18 CSI framework is reused - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported, - For inference report - For N4>=1, supporting Rel-18 codebook ('typeII-Doppler-r18')

Proposal 3:  RAN4 applies Rel-18 eType II Doppler codebook for AI/ML based CSI prediction

For RAN4 requirement perspective, although it was agreed to consider the reference throughout is obtained based on the random PMI with Rel-15 Type 1 single panel codebook, we think it is also necessary to compare the ratio of throughput obtained based on following PMI with Rel-18 eType II  and random PMI with Rel-15 type I single panel codebook, considering it is expected that AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists.

Proposal 4:  RAN4 should also compare the ratio of throughput obtained based on following PMI with Rel-18 eType II  and random PMI with Rel-15 type I single panel codebook, when defining the requirement of AI/ML based CSI prediction with Rel-18 eType II doppler codebook.

Training and test data consistency

Based on revised WID, one of objective for CSI prediction is to study and specify the consistency of training/inference for model generalization performance.

RAN1 further agreed to study potential NW-side additional conditions that may impact on UE’s assumption for CSI prediction to ensure consistency between training and inference. In our understanding, it also impacts the configuration for AI based CSI prediction

Agreement in RAN1#118bis For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered Agreement in RAN1#118bis For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configuration are not precluded - Note: for other evaluation assumptions, reuse baseline of TR 38.843 - Note: report how to map antenna port to antenna elements - Note: report the backbone/structure of AI/ML model used

Also, for consistency between training and inference, as evaluated by companies, performance evaluation of UE-side CSI prediction with respect to data distribution mismatch that may arise from NW-side additional condition, considering TRP antenna settings with different TXRU mapping. Therefore, some NW-side additional conditions indication was considered and studied in RAN1.

In general, for consistency between training data and test data should be ensured. To ensure the consistency, we think RAN4 should generate the training data by using the same NW-side condition specified for test.

Proposal 5:  RAN4 should generate the training data by using the same NW-side specified for test

## 3.2	Test Assumption for CSI prediction

In the last meeting, in order to facilitate the AI/ML model evaluation and alignment, the initial simulation assumption was agreed. We would like to provide our view for further proceeding the AI/ML CSI prediction evaluation in step 2.

KPI

Agreement in RAN4#114bis KPI; Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Note: - Step1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - FFS Step2: check SGCS (or other metrics, e.g., TPs) after CSI feedback.

Regarding the KPI for AI/ML based CSI prediction, as agreed, the criteria of SGCS before CSI feedback was applies to check the performance of AI/ML model for the step 1, since RAN4 has agreed to apply the ratio of throughput achieved. Therefore, the throughput should be checked after CSI feedback when reported PMI was applied for DL PDSCH transmission.

Besides, companies proposed to check the SGCS after CSI feedback, in our understanding, it maybe not necessary from AI based CSI prediction functionality verification perspective. Since after CSI feedback, gNB only can obtain the PMI index, such as i1, i2, where the obtained PMI related index was calculated by UE based on PMI estimated algorithm and reported by UE via the UL PUCCH channel. Based on the obtained PMI index, gNB will reconstruct the PMI matrix and apply this reconstructed PMI matrix for DL PDSCH transmission.  During the test, the corresponding PMI reconstructed algorithm will be implemented at TE side.  In our understanding, the whole CSI feedback process is same as the existing PMI test, excepting for CSI prediction.

Observation 1: The whole CSI feedback process is same as the existing PMI test, excepting for CSI prediction

Regarding to check the SGCS after CSI feedback, since the raw channel matrix including predicted and measurement ground truth cannot be obtained by gNB, only possible way is to calculate the SGCS is based on PMI matrix from reported PMI index, calculated based on predicted CSI and measured CSI, separately. Since how to calculate the PMI index is up to UE implementation, and the purpose is to verify AI/ML functionality, instead to verify the PMI estimation algorithm, where has been verified in the PMI reporting requirement, we would like to check the final throughput as KPI for Step 2, if the AI/ML CSI prediction results are well aligned in step 1.

Proposal 6: RAN4 should check the throughput as KPI for Step 2. No need to check the SGCS after CSI feedback.

CSI feedback overhead

In last meeting, some parameter combination (PC) values were proposed to facilitate the simulation for step 2 with following value

Agreement in RAN4#114bis CSI feedback overhead; Option1: Parameter combination = 2 (PC2) Option2: PC4 Option3: PC6 Option4: PC7 Note: this part relates to step2, may be down selected later.

For PC value, this parameter is related with parameter combination represented by by (L, pυ, β) for Rel-18 TypeII Doppler codebook as following table, where (*) indicates legacy Rel-16 eType II codebook combination. For these parameter combination,  L is the number of othogonal beams for each polarization, selected from a set of oversampled 2D DFT beams in spatial domain, pυ is configured to determine the number of DFT basis vectors in frequency domain, and  β is configured to determine the number of maximum linear combination coefficients.

Table 1: Codebook parameter configurations for L,  β and pυ

| paramCombination-Doppler-r18 | L | pυ |  | β |
| --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 2 | 1/8 | 1/16 | ¼ |
| 2 (*) | 2 | ¼ | 1/8 | ½ |
| 3 (*) | 4 | ¼ | 1/8 | ¼ |
| 4 | 4 | ¼ | ¼ | ¼ |
| 5 | 4 | ¼ | ¼ | ½ |
| 6 (*) | 4 | ¼ | ¼ | ¾ |
| 7 (*) | 4 | ½ | ¼ | ½ |
| 8 (*) | 6 | ¼ | - | ½ |
| 9 (*) | 6 | ¼ | - | ¾ |

Based on RAN1 evaluation, for Rel-18 eType II Doppler codebook, PC 6 and PC 7 were considered as benchmarks value, and in existing RAN4 test, PC7 was used, where the PC7 is also same as the value when defining the requirements of Rel-16 Type II codebook.

Therefore, to align with RAN1 evaluation and existing RAN4 test, we would like to proposes set the PC7 (L=4, pυ=1/2, β=1/2) for performance evalation firstly

Proposal 7: RAN4 consider PC7 for AI/ML based CSI prediction with Rel-18 eType II doppler codebook.

CSI-RS configuration

To facilitate the practical channel estimation, the common CSI-RS configuration should be considered.  Since it was agreed to evaluation both 32Tx and 16Tx antenna configuration as following

Agreement in RAN4#114bis gNB TX antennas; Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements.

Then, based on existing PMI test for type I and Rel-18 eType II doppler codebook, the following CSI-RS configuration for each CSI-RS resource can be considered as starting point

For 32Tx

| NZP CSI-RS for CSI acquisition | Number of CSI-RS ports (X) | 32 |
| --- | --- | --- |
|  | CDM Type | CDM4 (FD2, TD2) |
|  | Density (ρ) | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) | Row 17,(5, 12) |

For 16Tx

|  | Number of CSI-RS ports (X) |  | 16 |
| --- | --- | --- | --- |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |

Proposal 8: RAN4 consider the following CSI-RS configuration for each CSI-RS resource as starting for evaluation AI/ML CSI prediction

For 32Tx

| NZP CSI-RS for CSI acquisition | Number of CSI-RS ports (X) | 32 |
| --- | --- | --- |
|  | CDM Type | CDM4 (FD2, TD2) |
|  | Density (ρ) | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) | Row 17,(5, 12) |

For 16Tx

|  | Number of CSI-RS ports (X) |  | 16 |
| --- | --- | --- | --- |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |

Test setup for CSI-RS resource

For Rel-18 eType II doppler codebook test, only 1 time distance for prediction was considered, and K as 4 for both FDD and TDD, the agreed test setup was described as following

Agreement in RAN4#110bis Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook

For RAN1 evaluation for AI CSI, both periodic and Aperiodic CSI-RS configuration are considered.

Agreement in RAN1#116 For the evaluation of the AI/ML based CSI prediction, consider the following CSI-RS configuration - Periodic: 5ms periodicity (baseline), 20 ms periodicity (encouraged) - Aperiodic: Optional, CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType -II ) - Note: Companies to report observation window (number/distance) and prediction window Agreement in RAN1#116bis For the AI/ML based CSI prediction, adopt the following assumption as a baseline for evaluation purpose - UE speed: 30km/h, 60km/h - Others can be additionally submitted, e.g., 10km/h, 120km/h - Observation Window (number/distance): 5/5ms, 10/5ms - Others can be additionally submitted, e.g., 4/5ms, 15/5ms - Prediction window (number/distance between prediction instance/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms, 4/5ms/5ms - Others can be additionally submitted, e.g., 2/5ms/5ms, 3/5ms/5ms, 1/5ms/10ms - For other assumption, reuse Rel-18 baseline Agreement in RAN1#116bis For the evaluation of the AI/ML based CSI prediction, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted - Note: The same selected parameter combination shall be applied for benchmarks - CSI report periodicity: 5ms, 20ms (encouraged) - Other can be additionally submitted, e.g., 10ms

Compared with RAN1 evaluation, it may be not suitable to use the Rel-18 TypeII-Doppler-r18 codebook test setup directly for AI based CSI prediction, where the aperiodic CSI-RS resource was configured. Therefore, RAN4 should further discuss the proper test setup for AI/ML based prediction based on Rel-18 Type II-Doppler codebook. At starting point, we would like to check the CSI-RS configuration in periodic manner firstly

The test setup for FDD with AIML based CSI prediction can be considered as following figure 1,

Figure 1. FDD case for PMI reporting with AI/ML CSI prediction

Proposal 9:  RAN4 should further discuss the proper test setup for AI/ML based prediction based on Rel-18 Type II-Doppler codebook. The following test setup for FDD with AI/ML CSI prediction can be considered as starting point

Channel Estimation and related SNR point(range) for AI/ML model verification

Based on simulation assumption, for the purpose of calibration and comparing the intermediate results, e.g., accuracy of AI/ML output CSI, the ideal DL channel estimation was used for evaluation as the first step. From RAN4 demodulation requirement perspective, it was straightforward to apply the practical channel estimation method for specifying the related requirement. In general, the channel estimation algorithm and SNR level will have impact on the accuracy of channel estimation of input CSI, which also will impact in the AI/ML model performance. Therefore, we would like to propose the realistic channel estimation (e.g., LS or LMMSE) as baseline.

Proposal 10:  RAN4 should consider the practical channel estimation (e.g., LS or LMMSE) as baseline for step 2

For SNR level, in general, it should be impossible to use one AI/ML model to cover all the SNR range. With low SNR range, the channel correlation property will be destroyed by noise signal. It is meaningless and difficult to use AI/ML model for CSI prediction training with low SNR level data. The data collection for model training should be discussed to sure the proper CSI prediction performance.  In legacy PMI test, the SNR with 90% of throughput achieved based on following PMI will be used for calculating the ratio of throughput with following PMI and that with random PMI. Therefore, we would like to propose to use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration.

Meanwhile, regarding the training, RAN4 should also discuss whether the AI model is training based on dataset with specific SNR condition or mixed dataset with the selected SNR range, when applying the practical channel estimation.

Proposal 11: RAN4 shall discuss how to choose the proper SNR range for AI/ML based CSI prediction model training and test when applying the practical channel estimation.  The following method can be considered as starting point:

- Use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration

RAN4 should also discuss whether the AI model is training based on dataset with specific SNR condition or mixed dataset with the selected SNR range, when applying the practical channel estimation

## 3.3	Initial simulation results

In this section, based on the agreed simulation assumption in the last meeting, the initial simulation results are provided to check the AI/ML model for CSI prediction for different UE velocity and different antenna configuration with following cases

Table 2. FDD case for PMI reporting with AI/ML CSI prediction

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2GHz for FDD |
| Bandwidth | 10MHz for FDD, |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD |
| Subcarrier spacing | 15kHz for 2GHz |
| gNB TX antennas | Option1: 32 (N1, N2) = (4,4) as defined in Rel-15 type1 Option2: 16 (N1, N2) = (4,2) as defined in Rel-18 CSI prediction requirements |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Rank per UE | Rank 1 and 2 |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |

As step 1, the intermediate KPI based the average SGSC over all sub-bands per layer was provided for both rank1 and rank 2 cases.

3.3. 1	Intermediate KPI

In the performance evaluation, the raw channel matrices estimated based on the CSI-RS signals are feed into the AI model as input data. The CSI-RS interval is defined as 5ms. The AI model predicted the channel information in the next one CSI-RS slot, based on the historical 5 CSI-RS channel information, i.e., the observation window is 5/5ms and the prediction window is 1/5ms/5ms. The following table show the initial performance comparison of the AI/ML-based CSI prediction schemes under various UE moving speed scenarios.

Table 3 SGCS performance comparison over different UE speeds for 16Tx4Rx with Rank 1, Observation window (5/5ms)

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  | Non-AI Benchmark#1 |
| --- | --- | --- | --- | --- |
|  | 20Hz | 50Hz | 100Hz |  |
| 20Hz | 0.996 | 0.318 | 0.176 | 0.807 |
| 50Hz | 0.361 | 0.995 | 0.186 | 0.363 |
| 100Hz | 0.182 | 0.169 | 0.995 | 0.185 |

Table 4 SGCS performance comparison over different UE speeds for 16Tx4Rx with Rank 2, Observation window (5/5ms)

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  |  |  |  | Non-AI Benchmark#1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20Hz |  | 50Hz |  | 100Hz |  |  |  |
|  | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| 20Hz | 0.996 | 0.990 | 0.318 | 0.191 | 0.176 | 0.138 | 0.807 | 0.666 |
| 50Hz | 0.361 | 0.204 | 0.995 | 0.989 | 0.186 | 0.140 | 0.363 | 0.204 |
| 100Hz | 0.182 | 0.141 | 0.169 | 0.141 | 0.995 | 0.989 | 0.185 | 0.142 |

Table 5 SGCS performance comparison over different UE speeds for 32Tx4Rx with Rank 1, Observation window (5/5ms)

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  | Non-AI Benchmark#1 |
| --- | --- | --- | --- | --- |
|  | 20Hz | 50Hz | 100Hz |  |
| 20Hz | 1.000 | 0.668 | 0.465 | 0.874 |
| 50Hz | 0.600 | 1.000 | 0.457 | 0.589 |
| 100Hz | 0.464 | 0.468 | 1.000 | 0.467 |

Table 6 SGCS performance comparison over different UE speeds for 32Tx4Rx with Rank 2, Observation window (5/5ms)

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  |  |  |  | Non-AI Benchmark#1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20Hz |  | 50Hz |  | 100Hz |  |  |  |
|  | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| 20Hz | 1.000 | 1.000 | 0.668 | 0.601 | 0.465 | 0.420 | 0.874 | 0.831 |
| 50Hz | 0.600 | 0.533 | 1.000 | 1.000 | 0.457 | 0.415 | 0.589 | 0.520 |
| 100Hz | 0.464 | 0.415 | 0.468 | 0.422 | 1.000 | 1.000 | 0.467 | 0.413 |

Table 7 SGCS performance comparison over different UE speeds for 16Tx4Rx with Rank 1, observation window (10/5ms)

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  | Non-AI Benchmark#1 |
| --- | --- | --- | --- | --- |
|  | 20Hz | 50Hz | 100Hz |  |
| 20Hz | 0.996 | 0.372 | 0.180 | 0.814 |
| 50Hz | 0.487 | 0.996 | 0.192 | 0.348 |
| 100Hz | 0.174 | 0.214 | 0.996 | 0.182 |

Table 8 SGCS performance comparison over different UE speeds for 16Tx4Rx with Rank 2, observation window (10/5ms)

| Testing Scenarios | AI/ML-based Model (Training, UE speed) |  |  |  |  |  | Non-AI Benchmark#1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20Hz |  | 50Hz |  | 100Hz |  |  |  |
|  | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| 20Hz | 0.996 | 0.990 | 0.372 | 0.217 | 0.180 | 0.128 | 0.814 | 0.676 |
| 50Hz | 0.487 | 0.298 | 0.996 | 0.990 | 0.192 | 0.145 | 0.348 | 0.206 |
| 100Hz | 0.174 | 0.140 | 0.214 | 0.149 | 0.996 | 0.989 | 0.182 | 0.143 |

| 20Hz | 0.996 | 0.990 | 0.318 | 0.191 | 0.176 | 0.138 | 0.807 | 0.666 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50Hz | 0.361 | 0.204 | 0.995 | 0.989 | 0.186 | 0.140 | 0.363 | 0.204 |
| 100Hz | 0.182 | 0.141 | 0.169 | 0.141 | 0.995 | 0.989 | 0.185 | 0.142 |

From the initial simulation result, it can be seen that the SGCS performance of the AI/ML-based scheme is significantly better than the nearest historical CSI scheme (benchmark#1) for all the cases.

And for the SGCS performance for Rank 2 case is better than that for Rank 1 cases, especially under the antenna configuration of 16Tx4Rx.

However, there’s a generalization problem for the AI/ML-based CSI prediction scheme, which needs further investigations.

We also provide the results with observation window with (10/5ms). With increasing the number of historical CSI-RS slot, the impact on the prediction performance for corresponding scenarios, but the generalization performance seems to be improved for some cases.

Observation 2:  AI/ML based CSI prediction show better performance compared with nearest historical CSI scheme (benchmark#1), while generalization problem needs further study when defining requirement

# 4	Conclusion

In this contribution, the view on CSI reporting requirements framework was provided, including LCM related performance monitoring, and CSI reporting requirements. Meanwhile, the initial simulation results for AI/ML CSI prediction were provided to facilitate the calibration and comparison.

LCM related Performance Monitoring

Proposal 1:   Further study performance monitoring related requirement, and testability according to the sufficient progress in RAN1.

CSI reporting requirement

Proposal 2:  RAN4 can consider to apply the SNR test point related 90% of maximum throughput achieved with following PMI similar as legacy PMI test for AI-based CSI prediction as baseline.

Proposal 3:  RAN4 applies Rel-18 eType II Doppler codebook for AI/ML based CSI prediction

Proposal 4:  RAN4 should also compare the ratio of throughput obtained based on following PMI with Rel-18 eType II and random PMI with Rel-15 type I single panel codebook, when defining the requirement of AI/ML based CSI prediction with Rel-18 eType II doppler codebook.

Proposal 5:  RAN4 should generate the training data by using the same NW-side specified for test

Observation 1: The whole CSI feedback process is same as the existing PMI test, excepting for CSI prediction

Proposal 6: RAN4 should check the throughput as KPI for Step 2. No need to check the SGCS after CSI feedback.

Proposal 7: RAN4 consider PC7 for AI/ML based CSI prediction with Rel-18 eType II doppler codebook

Proposal 8: RAN4 consider the following CSI-RS configuration for each CSI-RS resource as starting for evaluation AI/ML CSI prediction

For 32Tx

| NZP CSI-RS for CSI acquisition | Number of CSI-RS ports (X) | 32 |
| --- | --- | --- |
|  | CDM Type | CDM4 (FD2, TD2) |
|  | Density (ρ) | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) | Row 17,(5, 12) |

For 16Tx

|  | Number of CSI-RS ports (X) |  | 16 |
| --- | --- | --- | --- |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) |

Proposal 9:  RAN4 should further discuss the proper test setup for AI/ML based prediction based on Rel-18 Type II-Doppler codebook. The following test setup for FDD with AI/ML CSI prediction can be considered as starting point

Proposal 10:  RAN4 should consider the practical channel estimation (e.g., LS or LMMSE) as baseline for step 2

Proposal 11: RAN4 shall discuss how to choose the proper SNR range for AI/ML based CSI prediction model training and test when applying the practical channel estimation.  The following method can be considered as starting point:

- Use the SNR value with 90% of throughput achieved based on following PMI as starting point, based on this SNR, considering ±X dB range for data collection for AI/ML training, where the SNR value is derived based on existing Rel-18 eType II Doppler codebook under the same configuration

RAN4 should also discuss whether the AI model is training based on dataset with specific SNR condition or mixed dataset with the selected SNR range, when applying the practical channel estimation

Observation 2:  AI/ML based CSI prediction show better performance compared with nearest historical CSI scheme (benchmark#1), while generalization problem needs further study when defining requirement

# References

R4-2502856, WF on AI/ML for NR air interface

R4-2502970, Way forward on simulation assumptions for CSI prediction

# Appendix