3GPP TSG-RAN WG4 Meeting #114bis	R4-2504550

# Wuhan, China, 7 – 11 April 2025

Agenda item:		7.29.2.1

Source: 		Qualcomm Inc.

Title: 		On simultaneous operation between GNSS and NR NTN

Document for:		Approval

## 1	Introduction

RAN4 has received a reply LS from RAN2 [1] on previous LS from RAN4 [2]. This contribution discusses the reply back to RAN2.

## 2	Discussion

Since release 17, 3GPP has assumed that NR NTN is able to simultaneously operate with GNSS. This is needed for example for accurate Tx frequency pre-compensation. However, no studies were done on this. The current work item explicitly mentions that “GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.”

Simultaneous GNSS and NR-NTN operation is especially challenging for operating bands n255 and n254 due to the narrow frequency separation between the NR-NTN UL and GNSS. This results in both Tx emissions leaking on top of GNSS Rx, as well as the NR-NTN UL acting as jammer to GNSS. Both of these effects can result in UE not being able to get a location fix, potentially leading to field issues.

There are multiple options on improving the likelihood of successful GNSS reception. For example, UE could take advantage of a more complex dual-frequency GNSS receiver. This would increase UE complexity receiving signals at both lower and upper L-band would need more complicated RF front-end and antennas, as well as more advanced digital receiver. All UEs do not have these capabilities.

Alternatively, UE could use more advanced RF filters which would reject Tx leakage to GNSS. At the same time, GNSS band would also need to use similarly advanced filter to avoid NR NTN UL from jamming the receiver. However, filtering solutions are not always viable. For example, n254 operates immediately adjacent to GLONASS frequencies, so one would need a perfect “brickwall filter” to fully avoid interference scenarios. This kind of perfect filter does not exists. In some cases filters can work, but they do add to cost and size of the RF solution.

UE could also try to rely on natural gaps in UL transmissions. The satellite beam covers a large area and could serve a large number of users, and all of these users cannot transmit at the same time. However, there can also be situations when the network load is low, and a single user has a long continuous data transmission. Even half-duplex operation cannot guarantee long transmission gaps for UL heavy traffic.

As such, there is no fool-proof solution which in all scenarios for all UEs could guarantee simultaneous GNSS and NR-NTN operation, and standards solution is needed.

Generally, GNSS needs a sufficiently long and sufficiently interference free time period to get a reliable location fix. As different UE implementations have different RF filtering, antenna performance, emission performance and jammer tolerance, the definition on “sufficient” will vary between implementations. To facilitate the co-existence between GNSS and NR-NTN, a standardized TDM-solution which can guarantee gaps in NR UL transmissions would be needed. Wide configurability would be needed to address different UE implementations, while it is recognized some advanced UEs may not need this solution at all.

Based on our studies, as an example of one implementation, a TDM pattern which can guarantee maximum 50% duty cycle for few seconds could work Within these few seconds, the UL transmissions cannot be continuous for a long time. This kind of gap would be needed regularly, multiple times in a minute. Still, signaling should accommodate a wide range of parameters.

Proposal 1: Reply to RAN2 that to cover wide range of UE implementations periodicity of the IDC solution needs to range from few seconds to few tens of seconds. Duration of the solution can range from a second to ten seconds, and within this period Tx cannot exceed 50% duty cycle. Draft LS is provided in appendix.

Proposal 2: Emphasize to RAN2 that network needs to support the solution in real deployments.

RAN2 specifically mentioned autonomous denials in their reply. Current signaling for autonomous denials have spare values available.

AutonomousDenialParameters-r18 ::=  SEQUENCE {

autonomousDenialSlots-r18           ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1},

autonomousDenialValidity-r18        ENUMERATED {n200, n500, n1000, n2000}

}

In our view, a simple and quick fix for this could be to introduce new values of autonomous denials parameters such as more aggressive values with 50% duty cycles, for example, extending autonomousDenialSlots-r18 to n500 or n1000 and taking use of the two spare values are available in autonomousDenialSlots-r18

Proposal 3: Ask RAN2 to consider taking the spare values in autonomousDenialSlots-r18 into use and add values n500 and n1000.

## 3	Conclusion

In this contribution reply to RAN2 on simulataneous operation between GNSS and NR NTN was discussed. Following observations and proposals were made.

Proposal 1: Reply to RAN2 that to cover wide range of UE implementations periodicity of the IDC solution needs to range from few seconds to few tens of seconds. Duration of the solution can range from a second to ten seconds, and within this period Tx cannot exceed 50% duty cycle. Draft LS is provided in appendix.

Proposal 2: Emphasize to RAN2 that network needs to support the solution in real deployments.

Proposal 3: Ask RAN2 to consider taking the spare values in autonomousDenialSlots-r18 into use and add values n500 and n1000.

## 4	References

[1]	R4-2503119/R2-2501568, Response LS on simultaneous operation between GNSS and NR NTN, RAN2

[2]	R4-2419898, LS on simultaneous operation between GNSS and NR NTN, RAN4

## Appendix

Title:	draft Reply LS on simultaneous operation between GNSS and NR NTN

Response to:	R4-2503119/R2-2501568

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact person:	Toni Lähteensuo

tlaehtee@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 thanks RAN2 for the reply LS. To cover a range of different UE implementations, the periodicity of the IDC free time can range from few seconds to few tens of seconds, while some UEs may not require a new IDC solution. The duration of the IDC free time can range from a second to ten seconds, and within this time period UE Tx cannot be continuously on for a long duration and exceed 50% duty cycle.

In context of autonomous denials, RAN4 thinks one solution could be extending autonomousDenialSlots-r18 to n500 and n1000 and taking use of the two spare values which are available.

RAN4 would like to ask RAN2 to take the above information into account in developing a IDC solution which UE can rely to be supported in real deployments.

# Actions

ACTIONS:

To RAN WG2 and WG1: RAN4 respectfully asks RAN2 and RAN1 to ensure that NR NTN UEs can rely on an in-device co-existence solution for GNSS co-existence with sufficient gaps in NR UL transmissions, as described above.

# Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #115	May 19 - 23, 2025	Malta, MT

TSG-RAN4 Meeting #116	Aug 25 - 29, 2025	Bangalore, IN