3GPP TSG-RAN WG4 Meeting #114bis	R4-2504554

# Wuhan, China, 7 – 11 April 2025

Agenda item:		7.10.3.2

Source: 		Qualcomm Inc.

Title: 		UE RF requirements for NTN less than 5 MHz

Document for:		Approval

## 1	Introduction

3 MHz channel bandwidth was specified for TN in rel-18. In the work item approved for NTN, 3 MHz channel bandwidth is to be added with RAN1-design based on rel-18, with no expected RAN1 impact. In this contribution UE RF requirements for 3 MHz channel bandwidth are discussed.

## 2	Discussion

The agreements reached in RAN4#114 are captured in [1]. This document addresses A-MPR and reference sensitivity

A-MPR

In previous meeting a tentative agreement on A-MPR was made. We have done further checks on the A-MPR values and would propose the following modification to the tentative agreement for NS_03N and NS_04N A-MPR. The reason is that when multiple individual 2ms time segments are studied, the variance in the measured power results in 1 dB backoff being too small in some instants.

Table 1: A-MPR regions for NS_03N and NS_04N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 1611.5 <= fc < [1613] |  | Option 1: >= 2.16 | A2 |

Table 2: A-MPR for NS_03N and NS_04N

|  | Modulation | A2 |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 -> 1.5 |
|  | QPSK | 3.5 |
|  | 16QAM | 4.0 |
|  | 64QAM | 4.5 |
|  | 256QAM | 5 |
| CP-OFDM | QPSK | 4.5 |
|  | 16QAM | 4.5 |
|  | 64QAM | 4.5 |
|  | 256QAM | 4.5 |

Proposal 1: Adjust NS_03N and NS_04N pi/2 BPSK A-MPR to 1.5 dB, confirm other A-MPR values from the tentative agreement in previous meeting.

Refsens

With 3 MHz channel bandwidth there are 15 resource blocks available and therefore total received power with same PSD is reduced by 2.2 dB compared to 5 MHz channel bandwidth. The reduced total power also means that any impairments have more impact to the signal quality. For NR NTN, maximum input power and in-band blocking interferer signal level are reduced to -40 dBm. This means that NR NTN may not be as well optimized for maximum linearity as receiver for TN bands.

For 3 MHz channel bandwidth, most LTE bands apply 1.7 dB lower power for 3 MHz channel bandwidth than for 5 MHz channel bandwidth. This is because Rx IIP2 effects become more impactful. Rx IIP2 needs to be considered in setting the wanted signal power level.

Observation 1: Rx IIP2 needs to be considered in setting wanted signal level for 3 MHz refsens.

An example of the sensivity budged it provided below, applicable for the case where baseline noise figure is 9dB.

In previous meeting following agreement was made

- Agreement

  - For band n254, [2.2] dB improvement compared with 5 MHz CBW;

  - For band n256, [1.7]dB improvement compared with 5 MHz CBW;

  - FFS for band n255

Values in square brackets can be confirmed. It would be logical to apply 1.7 dB improvement compared to 5 MHz CBW for band n255 as n255 has a better refsens to begin with. This means that Rx IIP2 impact is more visible in n255 compared to the other bands. Simultaneously, n255 has also smaller duplex gap.

Proposal 2: Confirm the 2.2 dB improved for n254 and 1.7 dB improvement for n256 compared to 5 MHz CBW. Apply 1.7 dB improvement compared with 5 MHz CBW for band n255.

## 3	Conclusion

In this contribution UE RF requirements for less than 5 MHz for NTN were discussed. Following observations and proposals were made

Proposal 1: Adjust NS_03N and NS_04N pi/2 BPSK A-MPR to 1.5 dB, confirm other A-MPR values from the tentative agreement in previous meeting.

Observation 1: Rx IIP2 needs to be considered in setting wanted signal level for 3 MHz refsens.

Proposal 2: Confirm the 2.2 dB improved for n254 and 1.7 dB improvement for n256 compared to 5 MHz CBW. Apply 1.7 dB improvement compared with 5 MHz CBW for band n255.

## 4	References

[1]  R4-2502282, Way Forward for [114][311] NR_IoT_NTN_less_than_5MHz_UERF , Xiaomi