# 3GPP TSG-RAN WG4 Meeting #114bis		            R4-2504558

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	5.3.1

Source: 	Huawei, HiSilicon

Title:	(PHS_System_NRonly) On the requirements of modified MPR behavior

Document for:	Approval

# 1	Introduction

As per the WF [1] from last meeting and the current meeting agenda, we continue to discuss the definition of modifiedMPR-Behavior for NS_05(U).

# 2	Discussion

It has been pointed out in our previous contribution [2] that there exist ambiguity and even contradiction in the definitions of the modified MPR behaviour in TS 38.101-1.

Observation 1: There are mixed usages of “shall” and “may” in the definitions of the modified MPR behaviour. Different bands and even different bits for the same band follow different rationales, leading to confusion.

Table 1: Issues on the definitions of modified MPR behaviour

| Rationale | Instances |
| --- | --- |
| “may be set to 1” for earlier releases, “shall be set to 1” for current and future releases | Band n30, n71 n41 bit 0, bit 1 |
| “may be set to 1” for earlier releases, not needed for current and future releases | Band n96, n102 n41 bit 2 |
| “may be set to 1” for any releases | Band n34, n39, n40, , n78, n79 n41 bit 3 |

A band of particular intertest is n41, for which the definitions are duplicated below.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| … |  |  |  |
| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |

It can be observed that:

Observation 2: For band n41, both bit 0 and bit 1 “shall be set to 1”. It’s unclear how to interpret bit 2 if it’s also set to 1. It’s questionable whether bit 2 can fulfil its intended purpose under existing definitions.

In our view, the main use case for the modifiedMPR-Behavior is when an MPR/A-MPR requirement is changed in an open release (Rel-N). An early release (Rel-M) UE may optionally support the new requirement and indicate it by the modifiedMPR-Behavior parameter. It’s unnecessary for a Rel-N UE to indicate this parameter, since the new MPR/A-MPR requirement is the only one left in the Rel-N spec.

Furthermore, the MPR/A-MPR requirement can be changed multiple times and/or in different releases. Hence, it’s difficult to be forward-compatible with future releases if the signalling of a given bit is made mandatory (via using “shall”).

Based on the above analysis, we propose to define new modifiedMPR-Behavior for band n1 as below, following the update of the requirements for NS_05 and NS_05U [1].

Proposal 1: Add the definition of modifiedMPR-Behavior for band n1 in Rel-16 to Rel-19 specs to allow UEs of earlier releases to indicate the support of new requirements for NS_05 and NS_05U updated in Rel-19.

Table 3: Changes for Rel-16 to Rel-18 specs

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit may be set to 1 by a UE supporting this release of the specification. |

Table 4: Changes for Rel-19 spec

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit may be set to 1 by a Rel-18 or earlier UE. |

Additionally, some maintenance for the modifiedMPR-Behavior table is needed. Hence, we propose:

Proposal 2: Follow the rationale in defining modifiedMPR-Behavior for band n1, align the requirements defined in TS 38.101-1 for other bands.

# 3	Conclusion

Based on our analysis on the current modifiedMPR-Behavior table in TS 38.101-1 [3], we have made the following observations and proposals:

Observation 1: There are mixed usages of “shall” and “may” in the definitions of the modified MPR behaviour. Different bands and even different bits for the same band follow different rationales, leading to confusion.

Observation 2: For band n41, both bit 0 and bit 1 “shall be set to 1”. It’s unclear how to interpret bit 2 if it’s also set to 1. It’s questionable whether bit 2 can fulfil its intended purpose under existing definitions.

Proposal 1: Add the definition of modifiedMPR-Behavior for band n1 in Rel-16 to Rel-19 specs to allow UEs of earlier releases to indicate the support of new requirements for NS_05 and NS_05U updated in Rel-19.

Table 3: Changes for Rel-16 to Rel-18 specs

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit may be set to 1 by a UE supporting this release of the specification. |

Table 4: Changes for Rel-19 spec

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 and A-MPR as defined in clause 6.2.3.4 of 38.101-1 Rel-19 | This bit may be set to 1 by a Rel-18 or earlier UE. |

Proposal 2: Follow the rationale in defining modifiedMPR-Behavior for band n1, align the requirements defined in TS 38.101-1 for other bands.

# References

- R4-2502980 WF on UE requirements for PHS protection, KDDI, OPPO, RAN4#114

- R4-2502236 On the requirements of modified MPR behavior, Huawei, HiSilicon, RAN4#114

- 3GPP TS 38.101-1 v18.7.0