3GPP TSG-RAN WG4 Meeting RAN4#114bis      				R4-2504562 Wuhan, China

April 7 – 11, 2025

Agenda Item:	7.1.1.3.4

Source:	Lenovo

Title:	SRS Power Imbalance Handling

Document for:  Approval

- Introduction

In previous meetings, some contributions have provided simulation results showing that SRS insertion losses have little to no impact on system performance and throughput, while other contributions have provided simulation results showing there is a significant impact.  The fact that results differ is perhaps not surprising given that simulation assumptions may not be exactly aligned, and that channel estimation algorithms, scheduling algorithms, and traffic models may all be different.

However, from a theoretical perspective, it is well understood that the scheduler must have an accurate estimate of the channel in order to optimally choose the MCS for the downlink transmission.  Thus, if the required complexity is not too great, it makes sense to attempt to enable the gNB to estimate the uplink channel more accurately in the presence of the SRS imbalances.

In this contribution, we review some previous proposals [2,3] for addressing this issue.

- Methods for Handling SRS Insertion Loss

At a minimum, the UE should indicate whether or not it compensates for insertion losses below Pc,max.  With this information, the gNB at least knows whether the power difference between the SRS ports is fixed as in Figure 1, or whether this difference only occurs near maximum power as in Figure 2.  With this information, the gNB may be able to use some UE measurements to estimate the differences in the insertion losses between the SRS ports.

Proposal 1:	The UE should indicate whether or not it compensates for the SRS insertion losses at power levels below Pc,max

It should be noted that this does not put any burden on the UE other than to indicate its behavior.

Figure 1:  SRS Power Control Behavior without Insertion Loss Compensation

Figure 2:  SRS Power Control Behavior with Insertion Loss Compensation

The ability of the gNB to estimate the channel can be further enhanced if the UE signals the differences of the SRS insertion losses relative to a reference SRS port.  It can be assumed that the difference in the insertion loss between two SRS ports is equal to the difference in the configured maximum power for these two ports. Based on this observation, we have the following proposal.

Proposal 2:	The UE should signal the set of values δ1,j for all 2≤j≤N to the gNB where N is the number of SRS ports,

δi,j=PCMAX,f,ci-PCMAX,f,cj

and the SRS ports are numbered such that

δ1,j≥0  for all 2≤j≤N .

These values only need to be signalled when there is a change in the mapping or numbering of the antenna ports.

We then have the following two observations from [2].

Observation 1:	For a UE that does not compensate the SRS insertion losses, the power difference between any two SRS ports is fixed and independent of the SRS power control setting (as in Figure 1).  As a result, with knowledge of the insertion loss differences, the gNB can correct the SRS based channel estimates without the need for PHR reports.

To reiterate, if the UE does not compensate the SRS insertion losses, the gNB can correct the SRS-based channel estimates using knowledge of the differences of these insertion losses and without any need for power headroom reports.

The discussion and derivation of the following observation can be found in [2].

Observation 2: For a UE that compensates SRS insertion losses, the gNB can determine the transmit power for any SRS port from the set of insertion loss differences δ1,j, 2≤j≤N, and a power headroom report for port 1, so long as port 1 has the smallest insertion loss.

To reiterate, if the UE compensates for SRS insertion losses up to the maximum configured power, the gNB can determine the transmit power for any of the SRS ports using the set of insertion loss differences and a power headroom report for the SRS port with the smallest insertion loss.

Proposal 3:	A UE indicates that it compensates the actual SRS insertion loss for each SRS port should have he capability to signal a power headroom report for its SRS port having the smallest insertion loss.

#### Summary

To minimize the impact of large SRS insertion loss imbalances on SRS-based channel estimation and subsequent MCS selection on the downlink, we have the following proposals.

At a minimum, the UE should indicate its handling of the SRS insertion losses when transmitting SRS as in Proposal 1.

Proposal 1:	The UE should indicate whether or not it compensates for the SRS insertion losses at power levels below Pc,max

If the UE does not compensate for SRS insertion losses, then knowledge of the insertion loss differences, or equivalently, the differences in the configured maximum power values are sufficient for the gNB to correct the SRS based channel estimates.

Proposal 2:	The UE should signal the set of values δ1,j for all 2≤j≤N to the gNB where N is the number of SRS ports,

δi,j=PCMAX,f,ci-PCMAX,f,cj

and the SRS ports are numbered such that

δ1,j≥0  for all 2≤j≤N .

These values only need to be signalled when there is a change in the mapping or numbering of the antenna ports.

If the UE does compensate for the SRS insertion losses, and if the insertion losses are known to the gNB, then a power headroom report for the SRS port having the smallest insertion loss is sufficient for the gNB to correct the SRS based channel estimates at all power levels.

Proposal 3:	A UE indicates that it compensates the actual SRS insertion loss for each SRS port should have he capability to signal a power headroom report for its SRS port having the smallest insertion loss.

# References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- R4-2413306, “SRS Imbalance Determination with Reduced PHR Overhead,” Lenovo, RAN4#112

- R4-2409736, “SRS Power Imbalance Determination,” Lenovo, Motorola Mobility, RAN4#111.

- 3GPP TS 38.101-1 User Equipment radio transmission and reception; Part 1: Range1 Standalone