3GPP TSG-RAN WG4 Meeting RAN4#114bis      				R4-2504564 Wuhan, China

April 7-11, 2025

Agenda Item:	7.1.1.2.2

Source:	Lenovo

Title:	Bandwidth Extension for Intra-Band UL CA with a Single CC

Document for:  Approval

- Introduction

In RAN4#113 it was agreed that for uplink CA with a single activated cell, the uplink CA emissions requirements should apply so that the MPR for the single activated cell can be used instead of the CA MPR.  Based on this agreement, it seems that the CA bandwidth can be considered as an extended channel bandwidth for the single activated cell and the inner MPR region of the single activated cell can be expanded in the same manner as for an extended channel bandwidth applied to a single carrier without carrier aggregation.

- Extended Channel Bandwidth with Single Activated Cell

In RAN4#113, it was agreed that for uplink CA with a single activated cell, the uplink CA emissions should apply so that the MPR for the single activated cell can be used instead of the CA MPR.  With this agreement, the CA bandwidth can be considered an extended channel bandwidth for the single activated cell and the inner MPR region of the single activated cell can be expanded. The situation is illustrated in Figure 1 below.

Figure 1:  Extended Channel Bandwidth for Intra-Band Uplink CA with Single Activated Carrier

If the aggregated channel bandwidth is taken to be the extended channel bandwidth for the single activated carrier, then the inner region can be defined as below.  Let NRB,1 and NRB,2 denote the transmission bandwidth configuration for first and second aggregated carriers and let NRB,Extended = NRB,1 + NRB,2.  If the UE transmits on the lower frequency carrier, then the inner region can be defined (similar to [2]) such that the starting RBStart satisfies

RBStart,Low,Extended  ≤  RBStart  ≤  RBStart,High,Extended

where

RBStart,Low,Extended = max(1, floor(LCRB/2)) ,

RBStart,High,Extended = NRB,Extended – RBStart,Low,Extended – LCRB ,

LCRB ≤ NRB,1, and RBStart is defined in the interval 0  ≤ RBStart ≤ NRB,1 -1.  Alternatively, if the UE is configured for CA and transmits on the higher frequency carrier, then the inner region can be defined such that

RBStart,Low,Extended  ≤  RBStart + NRB,1  ≤  RBStart,High,Extended

where

RBStart,Low,Extended = max(1, floor(LCRB/2)) ,

RBStart,High,Extended = NRB,Extended – RBStart,Low,Extended – LCRB ,

and LCRB ≤ NRB,2, and RBStart is defined in the interval 0  ≤ RBStart ≤ NRB,2  -1

Proposal:	For uplink CA with a single activated carrier, define the extended channel bandwidth as the aggregated carrier bandwidth and apply this extended channel bandwidth to determine the inner MPR region.

- Summary

In RAN4#113, it was agreed that for uplink CA with a single activated cell, the uplink CA emissions should apply so that the MPR for the single activated cell can be used instead of the CA MPR.  With this agreement, the CA bandwidth can be considered an extended channel bandwidth for the single activated cell and the inner MPR region of the single activated cell can be expanded.

Proposal:	For uplink CA with a single activated carrier, define the extended channel bandwidth as the aggregated carrier bandwidth and apply this extended channel bandwidth to determine the inner MPR region.

# References

- R4-2420527, “WF on Power domain enhancement,” Huawei, HiSilicon, RAN4#113.

- R4-2418999, “Discussion on Power domain enhancements for single carrier,” LG Electronics UK, RAN4#113

- R4-2502252, “Bandwidth Extension for Intra-Band UL CA,” Lenovo, RAN4#114.