3GPP TSG-RAN WG4 Meeting #114-bis									                      R4-2504581

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.22.2

Source:	Ericsson

Title:	On the UE RF requirements for NR MIMO Phase 5

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for the RAN1-led WI on “NR MIMO Phase 5” WI [1], where the objectives of the WI with the identified potential RAN4 UE RF impacts are the following:

RAN1:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Acronyms:

- sTRP: single TRP (transmit-receive point)

- mTRP: multiple TRP (transmit-receive point)

- CJT: coherent joint transmission

- DU: distributed unit

- CRI: CSI-RS resource indicator

RAN2:

Specify higher layer support of the enhancements listed above.

RAN4:

Specify necessary core requirements for the enhancements listed above.

In this contribution, we continue the discussion on the UE RF requirements for “Support for 3-antenna-port codebook-based transmissions” (“3Tx” topic) and “Asymmetric DL sTRP/UL mTRP” working areas.

# Discussion

## 3Tx

In RAN4#114, a WF has been approved [2]. In the following, we discuss the remaining open issues for the “3Tx” topic.

### Power class

On the support for different power classes, the following has been captured in the WF:

## <Sub-topic 1-1> Power class

<Way forward>: PC for 3Tx requirement

- For PC3 and PC2, specify requirements for FWA and handheld UE

- FFS on PC1.5 with 3Tx, will be decided in the next meeting

  - Option 1: Consider it in the next release

  - Option 2: Specify requirements in Rel-19

In the previous meetings, it was concluded (agreed) that the uplink full power transmission (ul-FullPowerTransmission in PUSCH-Config), introduced in Rel-16, will not be supported for PC1.5 in Rel-19.

The default “Rel-15 behavior” (where the uplink full power transmission is not supported) for a PUSCH transmission scheduled by a DCI format other than DCI format 0_0, or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook' and if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, is [3]:

- “…the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource. The UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.”

Since 3 Tx operation is set up by configuring the UE with a 4 port SRS resource with port 1003 disabled, the text above may be confusing. Following Rel-15 principles, the UE should scale down by 3 (‘the maximum number of SRS ports supported by the UE in one SRS resource’ should be equal to 3) rather than 4 for 3 Tx, even though a 4 port SRS resource is configured to the UE. While RAN1 are presently discussing how to update the specification text above, RAN1 have agreed that the scaling down should be by 3 for PUSCH (and also for SRS) so that Rel-15 principles are followed:

Agreement (RAN1#116bis) For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0). - FFS: Whether specification needs to be updated to reflect the above Agreement (RAN1#116bis) For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource.

In particular, for a UE equipped with 3 Tx chains, the Rel-15 behavior for 3-layer PUSCH transmission implies that the scaling factor is equal to 1 (since both ‘the number of antenna ports with a non-zero PUSCH transmission power’ and ‘the maximum number of SRS ports supported by the UE in one SRS resource’ are equal to 3), and that the power is equally split across three antenna ports (three layers). The power of 29dBm (PC1.5) could be achieved by a UE equipped with 3 PAs of 26dBm each.

Furthermore, Rel-15 behavior is supported for 2Tx and 4Tx UEs, with no particular limitation on PC1.5 for an operating band supporting it.

Proposal 1: PC1.5 should be supported by 3Tx UEs with no uplink full power transmission modes defined, i.e. with the “Rel-15 behavior”. It should be feasible for both FWA and handheld UEs.

The most controversial point in the previous meeting was whether the Tx Diversity (TxD) is precluded or not by RAN1 for ‘3Tx’ topic in this WI. The reason is that in TS38.101-1 [4], clause 6.2G.1, it is stated: “When a UE indicates PC1.5 for a given band it achieves maximum power by means of Tx Diversity in the current version of the spec. Therefore, Tx Diversity is implied for PC1.5 even if the UE does not indicate Tx diversity capability.”

From the previous excerpt one can conclude that the support for PC1.5 depends on the support for TxD.

In our understanding, RAN1 does not generally address power class or transparent TxD (there is no transparent TxD in the RAN1 spec), so TxD was not precluded (or explicitly supported) for PC1.5 for ‘3Tx’ topic in this WI.

Observation 1: RAN1 does not generally address power class or transparent TxD (there is no transparent TxD in the RAN1 spec), so TxD was not precluded (or explicitly supported) for PC1.5 for ‘3Tx’ topic in this WI.

Therefore, RAN4 should be free to assume that the TxD is available for 3 Tx in Rel-19. There is no valid reason in our view to preclude it, as the lack of support for full power Mode 1 is not equivalent to lack of support for TxD.

As explained above, Rel-15 power scaling with a non-coherent codebook implies that the maximum required power is PPowerClass/3 when one port is selected, i.e. RAN1 assumption is that each Tx chain is capable of PPowerClass /3. Hence, for 3 Tx, the maximum output power requirements are actually lower per PA than for 2 Tx PC1.5 (which is assumed to be achieved with TxD). On the other hand, when the UE falls back to single port for DCI 0_0 when configured for UL MIMO, or when it is configured for single port operation (even though it supports non-coherent UL MIMO), PC1.5 could be achieved by using TxD.

Another point is that, as soon as the UE supports PC1.5 in general (for either 2Tx or 4Tx), it means that it must support TxD. If such UE indicates the support for 3 layers at the same time, that UE could achieve PC1.5 with 3Tx and TxD. There is no reason to artificially preclude the TxD support for 3Tx case.

Proposal 2: RAN4 should be free to assume that the TxD is available for 3 Tx in Rel-19 since RAN1 does not generally address power class or transparent TxD. If there is still a doubt in the group, an LS could be sent to either RAN or RAN1.

### MPR

On the MPR requirements topic, the following was captured in the approved WF [2]:

## <Sub-topic 1-2> MPR

<Way forward>: PC2 for 3Tx

- Option 1: Use the same MPR as MPR for PC2 with 2Tx, as it’s for UL-MIMO and from simulation results

- Option 2: Use the same MPR as MPR for PC2 with 1Tx, as it’s the worst case

According to some evaluations submitted in the previous meetings, e.g. [5], for both PC2 and PC1.5 it was concluded that the MPR required is slightly smaller than the current corresponding 2Tx UL MIMO MPR requirement. Thus, it was proposed to reuse corresponding 2Tx MPR requirements for 3Tx as well, which is reasonable in our view.

For 2Tx MPR requirements for PC1.5, there are currently two tables in TS38.101-1: the default table and the table targeted to large FWA form factor with 20 dB or above antenna isolation. One approach would be to reuse both tables for 3Tx, and the other approach would be to only adopt the table targeted to large FWA form factor with 20 dB or above antenna isolation (since the required MPR requirements for 3Tx are smaller than the MPR for 2Tx).

Proposal 3: For both PC2 and PC1.5, reuse the legacy 2Tx MPR requirements to specify 3Tx MPR requirements. For PC1.5, either adopt only the 2Tx MPR table ‘targeted to large FWA form factor with 20 dB or above antenna isolation’ or adopt both 2Tx MPR tables specified in TS38.101-1.

## Asymmetric DL sTRP/UL mTRP

In the approved WF [2], the following was captured on this topic:

<Sub-topic 2-1> UE RF impact

<Way forward>: RAN4 will decide between following options in the next meeting

- Option 1: No UE RF impact of asymmetric DL sTRP/UL mTRP

- Option 2: Adopt following dCR, where the companies are encouraged to propose wording improvements in the next meeting

## 6.6M	Beam correspondence for [UEs with different TCI states for UL and DL]

- For a UE that supports [[future_sTRP_2TA_capability] and the IE unifiedTCI-StateType is configured as ‘separate’, and the QCL type D reference signals associated with the UL and DL TCI states are different,] the requirements of clause 6.6 apply. Side-conditions in clause 6.6 apply to each of the TCI states.

On this topic, the following question has been sent to RAN1 in RAN4#112-bis meeting in the approved LS [6]:

Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID.

In RAN1#119, RAN1 has replied with the following [7]:

Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

Based on the reply from RAN1, the scenario where the UL mTRP transmits SSB should be included in this WI.

RAN4 has been discussing on which UE RF requirements will have an impact by introducing the ‘Asymmetric DL sTRP/UL mTRP’ for FR2. In [8], it was recognized that by introducing this feature there will be an impact on the beam correspondence requirements, only for the case where the UL mTRP node transmits SSB.

However, according to the WID[1], the Rel-17/18 unified TCI framework is assumed, fully reusing the legacy QCL/UL spatial relation rules. In RAN1#117, the following was concluded:

Conclusion (RAN1#117) For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

From the previous, the scenario where the UL mTRP node transmits SSB is only covered by the Rel-18 unified TCI framework for FR1 with “up to two joint TCI states”.

Regarding the Rel-18 unified TCI framework for FR2, as raised in our RAN1 contribution for this meeting [9], it is not clear what does “{one DL TCI state + up to two UL TCI states} can be applied” exactly mean. In our understanding, there are two alternatives to interpret that agreement:

- Alt 1: {one DL TCI state + up to two UL TCI states} can be configured and indicated, or

- Alt 2: 2 separate DL/UL TCI state pairs can be configured and indicated, but only {one DL TCI state + up to two UL TCI states} are used

Only in the case of Alt 2, the scenario where the UL mTRP node transmits SSB can also be covered by the Rel-18 unified TCI framework for FR2.

Proposal 4: RAN4 shall wait for the outcome of RAN1 discussion on the interpretation of Rel-18 unified TCI framework agreement for FR2 (the meaning of ‘{one DL TCI state + up to two UL TCI states} can be applied.’) before proceeding with specifying the beam correspondence requirements for the asymmetric DL sTRP/UL mTRP deployment scenario for FR2.

# Conclusion

Based on the discussion in the previous sections we have made the following observation:

Observation 1: RAN1 does not generally address power class or transparent TxD (there is no transparent TxD in the RAN1 spec), so TxD was not precluded (or explicitly supported) for PC1.5 for ‘3Tx’ topic in this WI.

We have also made the following proposals:

Proposal 1: PC1.5 should be supported by 3Tx UEs with no uplink full power transmission modes defined, i.e. with the “Rel-15 behavior”. It should be feasible for both FWA and handheld UEs.

Proposal 2: RAN4 should be free to assume that the TxD is available for 3 Tx in Rel-19 since RAN1 does not generally address power class or transparent TxD. If there is still a doubt in the group, an LS could be sent to either RAN or RAN1.

Proposal 3: For both PC2 and PC1.5, reuse the legacy 2Tx MPR requirements to specify 3Tx MPR requirements. For PC1.5, either adopt only the 2Tx MPR table ‘targeted to large FWA form factor with 20 dB or above antenna isolation’ or adopt both 2Tx MPR tables specified in TS38.101-1.

Proposal 4: RAN4 shall wait for the outcome of RAN1 discussion on the interpretation of Rel-18 unified TCI framework agreement for FR2 (the meaning of ‘{one DL TCI state + up to two UL TCI states} can be applied.’) before proceeding with specifying the beam correspondence requirements for the asymmetric DL sTRP/UL mTRP deployment scenario for FR2.

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105.

- R4-2503037, “WF on UE RF requirements for MIMO enhancement”, Samsung, RAN4#114.

- 3GPP TS38.213 V18.5.0, “NR; Physical layer procedures for control”.

- 3GPP TS38.101-1 V19.0.0, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- R4-2414983, “R19 MPR for 3Tx”, OPPO, RAN4#112-bis.

- R4-2417117, “LS on UE RF issues for Rel-19 MIMO enhancement”, Huawei, RAN4#112-bis.

- R1-2410870, “Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement”, Qualcomm Incorporated, OPPO, RAN1#119.

- R4-2417632, “dCR on RF requirements for a UE with different TCI states for UL and DL”, Qualcomm Incorporated, RAN4#113.

- R1-2502810, “UE features for NR MIMO Phase 5”, Ericsson, RAN1#120bis.