3GPP TSG-RAN WG4 Meeting #114-bis									                      R4-2504586

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.10.2.3.1

Source:	Ericsson, Sony, Nokia

Title:	On the UE RF Tx requirements for IoT-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for IoT-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the SAR related issues for IoT-NTN HPUE, captured in the approved WF in RAN4#114 [2].

# Discussion

In the approved WF [2], the following was captured on the SAR related issues topic:

Issue 2-1-1: Whether and how to assume some scenarios like satellite orbits and traffic model etc. for the study of SAR issue.

- Proposals

  - Option 1: No need.

    - [E.g. The scheme has to be general enough to cover various scenarios;]

  - Option 2: Needed.

    - [E.g. If typical scenarios do not have an issue may be the discussion is not needed.]

  - Option 3: Others

WF (Ad-hoc agreement):

- Option 1

Issue 2-1-3: Whether and how to introduce duty cycle related method.

- Proposals (Not necessarily mutually exclusive):

  - Scheme 1: (vivo)

Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful.

  - Scheme 2: (oppo)

UE indicate whether the following Tx time duration should be counted into the UL duty cycle statistics in addition to current maxUplinkdutycycle capability. How the information is reported is up to RAN2.

  - Scheme 3: (Samsung)

To use beam hopping information as the UE uplink duty ratio in time domain if duty cycle-based SAR solution is to be specified.

And to discuss following impacts to existing duty cycle solution, considering our target is for UE to maintian its power class

Scheme 3-1: UE to fall back power classes based on information above ( legacy duty cycle solution, not quite aligned with purpose to transmit as much and as long as possible)

Scheme 3-2: UE to fall back several dB but not entire power class, based on information above ( technically UE can reach more power than Scheme 3-1)

Scheme 3-3: UE to not fall back any power, but to monitor SAR by itself. ( In this case, UE may fail SAR compliance.)

- Scheme 3-3a: UE to enter scheme like a ’DTx’, or Tx UL Gap for a certain duration.

- Scheme 3-3b: UE to give up a certain ratio of uplink opportunities for a certain duration to recover its margin for SAR compliance. (Like above scheme 1 from vivo)

- Scheme 3-3c: UE to fall back to lower power class for a certain duration to recover its margin for SAR compliance.

  - Scheme 4: (Ericsson, Sony)

Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value. (Ericsson, Sony)

If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details: (Ericsson, Sony)

    - If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

      - DPC = ‘00’: the criteria to report DPC is not met

      - DPC = ‘01’: ΔPPowerClass = 0 dB

      - DPC = ‘10’: ΔPPowerClass = 3 dB

      - DPC = ‘11’: ΔPPowerClass = 6 dB

    - If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

      - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

      - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

      - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

      - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

- WF

  - Further discuss the proposed schemes related to duty cycle related SAR scheme.

  - Other schemes or no duty cycle related scheme are not precluded.

## Consideration of different proposals

In the previous meeting, there was a proposal to introduce a scheme where the UE is voluntarily dropping uplink transmissions by self-assessment to control the SAR. Even if the network would have a control on enabling that behavior, the UE behavior would still be too unpredictable and hence the impact on the performance. Dropping transmissions rather than reducing a power seems like too conservative approach.

Another proposal suggested that the UE indicate whether the following Tx time duration should be counted into the UL duty cycle statistics in addition to current maxUplinkdutycycle capability. The main motivation for such proposal is that in some regions, SAR is not required to be met when NTN UE is in the handheld status and is only required when NTN UE is near the head. Reporting only maxUplinkdutycycle capability along with the proposed information on whether to take into account the following Tx time duration is in general not sufficient for the network to know when exactly the UE is switching to the higher power class or falling back to the lower power class. In our view, the lack of that information is one of the biggest disadvantages of the original uplink duty cycle approach without the additional reporting. In addition, it would be better to pursue a solution which would be applicable in all regions.

Finally, one proposal suggested using the maximum beam hopping information as the UE uplink duty ratio in time domain if duty cycle-based SAR solution is to be specified. Such information could serve as a complementary information, but it is not sufficient as it only represents the upper bound of the UE uplink duty ratio, not the actual value. In addition, the network would not be aware of the UE switching to the higher power class or falling back to the lower one.

## Proposed solution

For Specific Absorption Rate restriction (SAR) mitigation, in NR TN specifications (TS38.101-1 [3]) an optional method based on the “UL duty cycle” is specified, where the UE can report maxUplinkDutyCycle-PC2-FR1 or maxUplinkDutyCycle-PC1dot5-MPE-FR1 capabilities (otherwise a default duty cycle of 50% or 25% applies, respectively). If the percentage of transmitted uplink symbols at the UE exceeds the corresponding threshold, the UE is supposed to do a power-class fallback.

Such method is not well designed for FDD bands, as it is not specified how exactly the UE implements such a solution (i.e. what is the averaging period for the duty cycle at the UE), since it is only stated that “the percentage of uplink symbols transmitted in a certain evaluation period is larger than X% (The exact evaluation period is no less than one radio frame);” This period could be anything from one radio frame to many radio frames, and the network is not aware of how the UE makes that choice. Moreover, allegedly, no UEs in the field provide these capabilities. UEs use P-MPR method instead for managing excessive duty cycles.

However, the P-MPR method is blind for the network as the network is not aware if a P-MPR is applied due to a proximity sensor or in case of “simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications” (the original intentions), or for reducing the average power to facilitate SAR compliance.

Observation 1: “Duty-cycle” based method is not well designed for FDD bands and is allegedly not used in the field. P-MPR method is used instead, which is on another hand “blind” for the network.

In Rel-18, as part of “Enhancement of increasing UE power high limit for CA and DC” part of “Further NR coverage enhancements” WI, reporting of ΔPPowerClass (DPC) was introduced in the PHR (Power Headroom Report) for improving network scheduling by providing the network with the actual power-capability state of a UE. The ΔPPowerClass (DPC) in the PCMAX,f,c for a serving cell c is set by a UE depending on the actual scheduled UL duty cycle in accordance with its reported UL duty cycle capabilities. The background to the reporting is that RAN4 was informed by RAN1 that [4]:

RAN1 made the following agreement concerning enhancements to realize increasing UE power high limit for CA and DC. Agreements For enhancements to realize increasing UE power high limit for CA and DC, RAN1 can study based on RAN4’s input - Whether RAN1 enhancements to information exchange between UE and gNB are needed to improve scheduling and network performance when using higher power CA/DC. FFS how to realize such information exchange, e.g., signalling enhancement, and what is the spec impact.

The original scope was only CA and DC, but since the similar issues were recognized for non-CA case as well, it was also included to the RAN4 discussion.

In RAN4, it was agreed that such reporting is triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (the introduced UE capability deltaPowerClassReporting-r18). This would indeed circumvent some of the difficulties associated with duty-cycle reporting, as the network is told when a power class is modified for SAR mitigation without limiting UE implementation to this end. Thus, a “duty cycle” based method together with similar reporting could be considered as a solution for SAR compliance for NTN HPUEs as well.

However, as stated above, the problem is that no UEs in the field provide the UL duty-cycle capabilities and UEs use P-MPR method instead (as that method is a better fit for FDD bands). So, for NTN HPUEs the optional DPC-reporting feature should also be available to UEs using P-MPR for managing excessive UL duty cycles, which would ensure that the feature can be used for improving scheduling in the field based on the actual power capability for supporting UEs.

Observation 2: “Duty-cycle” based and the P-MPR based methods in combination with reporting of the actual power-capability state of a UE could be considered for NTN HPUEs, similar to Rel-18 enhancements introduced for TN.

The “updated” Rel-18 “Single Entry PHR MAC CE” has the following structure [5]:

Currently, the DPC field is defined as follows, irrespective of P field, which for FR1 only indicates that the power backoff is applied due to the power management (as allowed by P-MPRc):

DPC = ‘00’: the criteria to report DPC is not met

DPC = ‘01’: ΔPPowerClass = 0 dB

DPC = ‘10’: ΔPPowerClass = 3 dB

DPC = ‘10’: ΔPPowerClass = 6 dB

The PHR reporting that could be applied to NTN UEs could reuse exactly the same structure as for the “Single Entry PHR MAC CE”, but with an improvement where the DPC field could indicate either the actual ΔPPowerClass applied in PCMAX,f,c or the actual P-MPR applied (with granularity of 3dB):

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB when 3 < P-MPRc ≤ 6 dB

Proposal 1: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value.

Proposal 2: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details:

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

# Conclusion

Based on the discussion in the previous sections we have made the following observations:

Observation 1: “Duty-cycle” based method is not well designed for FDD bands and is allegedly not used in the field. P-MPR method is used instead, which is on another hand “blind” for the network.

Observation 2: “Duty-cycle” based and the P-MPR based methods in combination with reporting of the actual power-capability state of a UE could be considered for NTN HPUEs, similar to Rel-18 enhancements introduced for TN.

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value.

Proposal 2: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details:

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2503006, “WF on HPUE UE requirement for IoT-NTN”, vivo, RAN4#114.

- 3GPP TS38.101-1 V19.0.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- R1-2210739, LS on enhancements to realize increasing UE power high limit for CA and DC, RAN1.

- 3GPP TS38.321 V18.5.0, “NR; Medium Access Control (MAC) protocol specification”.