3GPP TSG-RAN WG4 Meeting #114bis	R4-2504601

Wuhan, China, April 7 – 11, 2025

Agenda Item:	7.17.2.2

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF

Document for:	Discussion

# Introduction

The objective of R19 NR RRM Phase-5 work item is captured below in the WID [1]. In this paper, we continue the discussion on the first objective, particularly on optimizing CSSF outside gap in CA/DC scenarios for L3 measurement delay reduction.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In the last meeting, RAN4 further made further progress on the previous three tentative solutions to optimize CSSF outside gap in R19 for L3 measurement delay reduction. The first one proposes to measure only one serving CC per band if multiple serving CCs are in the same band. The second one is whether to use a variable ratio to share the searchers with different CCs. The third one is whether to introduce a third searcher to optimize CSSF. However, RAN4 only agreed on Solution 1 and 3 as captured below WF [2].

Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. Note: Following the WID, the above agreement applies for the measurement without gap. Issue 1-3-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

For solution 3 in Issue 2-4-3, RAN4 further discussed the issue of introducing the third searcher to optimize CSSF. This solution was controversial as it is mentioned in the WID that 2 searchers are the baseline assumption in this WI. However, the compromised agreement above was to apply three searcher enhancement for two scenarios only:

- FR1 scenario and

- FR1 + FR2 scenario

In addition, for the second scenario (FR1+FR2) it is only applied when PCell is in FR1, which the typical scenario in the field. Regarding the FFS part on whether NR-DC is included for the above scenarios, our suggestion is to focus CA operation mode when 3 searchers are used and not to consider NR-DC. In in NR-DC there are two cases

- FR1 + FR2 NR-DC (FR1 PCell and FR2 PSCell)

- FR1 + FR1 NR-DC (FR1 PCell and FR1 PSCell)

Since in the agreement of FR1 + FR2 scenario, we agreed to consider PCell in FR1, however if we consider NR-DC, then both PCell and PSCell need to be considered in FR1, while other SSCs can be FR2. But such scenario does not exist in the legacy table of the CSSF for NR-DC. Therefore, we prefer to remove NR-DC from FR1 + FR2 scenario, but we are fine to keep NR-DC for FR1 scenario.

Proposal 1: Remove NR-DC from (FR1 + FR2 scenario) while it can be kept for (FR1 scenario).

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: Remove NR-DC from (FR1 + FR2 scenario) while it can be kept for (FR1 scenario).

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.