3GPP TSG-RAN WG4 Meeting # 114bis	R4-2504603

Wuhan, CN, Apr 07 – Apr 11, 2025

Agenda Item: 7.22.3.2

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for other RRM requirements

Document for: Discussion

# Introduction

RAN#105 agreed on the revised WID “NR_MIMO_Ph5” [1]. According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WID [1] is divided into two items to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “Other RRM requirements”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul.

- Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

## 2.1 UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

In the last meeting, RAN4 reached the following agreements as captured in the WF [2]:

Issue 1-1: How to decide on the number of samples used for CJT calibration? Agreement: - RAN4 to decide on the number of samples based on its impact on the reported interval (granularity of the report) for the given report resolution, while using the following parameters - Report resolution (MD = 128, MFO = 128). - Offset range (AD and AFO) - AD = CP - AFO = 0.2 ppm - Time offset between the two TRPs: 0.5 CP - Frequency offset between the two TRPs: 0.1 ppm - Note that report resolution and offset range are network configuration via RRC signalling. - Simulation results can be provided based on the CDF accuracy at 5%, 50% and 95%.

In the last RAN4 meeting, it’s agreed that the number of samples for CJT calibration report will be chosen based on the impact on the reported interval (granularity of the report). However, for frequency offset report, RAN1 did not specify report resolution for MFO = 128. Therefore, we provided the simulation result based on largest report resolution for both delay and frequency offsets (MD = 256, MFO = 256).

Table 1 shows all the simulation parameters based on simulation assumption agreed in previous RAN4 meeting.

Table 1. Simulation parameters for timing and frequency offset estimation

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Duplex, SCS | - FDD, 15kHz SCS - TDD, 30kHz SCS |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDLC300-10 |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB |
| Numerology | 14 OFDM symbol per slot |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples (TRS resource sets) for measurement | 1, 3, 5 2nd priority: 2, 4, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) |
| CSI-RS periodicity (TRS) | 20ms / 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |

Based on the Table 1 and the agreement in last RAN4 meeting, we provide simulation result of timing offset and frequency offset on Table 2 and Table 3.

Table 2 shows the timing offset error estimation based on AD = CP and MD = 256. The number of quantization level is 256. The TO estimation error means the difference between measured TO and the ideal TO which is 0.5 CP (Time offset between the two TRPs). The ideal TO (0.5 CP) is exactly mapped to quantization level 128 (# level 128). Therefore, this level will be used as a reference to calculate the error in the reported interval. E.g., the highlighted part (# level 28) is the difference between measured TO (# level 100) and ideal TO (# level 128). From Table 2, we could observe the minimum TO estimation error is around CDF 50%.

Observation 1: The results at CDF 50% show minimum timing offset estimation error.

Table 2. Simulation result for timing offset error estimation.

| Parameters set | SCS | SNR | Timing offset (TO) b/w two TRPs | TO estimation error in AD = CP and MD = 256 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CDF5%, #levels (256) |  |  | CDF50%, #levels (256) |  |  | CDF95%, #levels (256) |  |  |
|  |  |  |  | #samples |  |  |  |  |  |  |  |  |
|  |  |  |  | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 |
| #1 | 15KHz | -3 | 0.5CP | 28 | 16 | 13 | 1 | 1 | 0 | 29 | 19 | 18 |
| #2 | 15KHz | 0 | 0.5CP | 12 | 6 | 4 | 1 | 1 | 1 | 17 | 12 | 10 |
| #3 | 15KHz | 3 | 0.5CP | 5 | 3 | 2 | 1 | 1 | 1 | 10 | 7 | 6 |
| #4 | 30KHz | -3 | 0.5CP | 23 | 12 | 9 | 1 | 1 | 1 | 30 | 19 | 17 |
| #5 | 30KHz | 0 | 0.5CP | 13 | 7 | 5 | 2 | 1 | 1 | 16 | 11 | 10 |
| #6 | 30KHz | 3 | 0.5CP | 10 | 6 | 4 | 2 | 1 | 1 | 14 | 9 | 9 |

Table 3 show the frequency offset error estimation based on AFO = 0.2 ppm and MFO = 256. The number of quantization level is 256. The FO estimation error means the difference between measured FO and the ideal FO which is 0.1 ppm (frequency offset between the two TRPs). The ideal FO (0.1 ppm) is exactly mapped to quantization level 128 (# level 128). E.g., the highlighted part (65) is the difference between measured FO (# level 63) and ideal FO (# level 128). From Table 3, we could observe the minimum FO estimation error is around CDF 50%.

Observation 2: The results at CDF 50% show minimum frequency offset estimation error.

Table 3. Simulation result for frequency offset error estimation

| Parameters set | SCS | SNR | Frequency offset (FO) b/w two TRPs | FO estimation error in AFO = 0.2 ppm and MFO = 256 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CDF5%, #levels (256) |  |  | CDF50%, #levels (256) |  |  | CDF95%, #levels (256) |  |  |
|  |  |  |  | #samples |  |  |  |  |  |  |  |  |
|  |  |  |  | #1 | #3 | #5 | #1 | #3 | #5 | #1 | #3 | #5 |
| #1 | 15KHz | -3 | 350 Hz | 65 | 39 | 29 | 1 | 1 | 1 | 61 | 39 | 28 |
| #2 | 15KHz | 0 | 350 Hz | 39 | 22 | 18 | 2 | 2 | 2 | 36 | 24 | 17 |
| #3 | 15KHz | 3 | 350 Hz | 25 | 15 | 12 | 1 | 1 | 2 | 26 | 16 | 12 |
| #4 | 30KHz | -3 | 350 Hz | 106 | 60 | 36 | 2 | 3 | 6 | 133 | 71 | 60 |
| #5 | 30KHz | 0 | 350 Hz | 75 | 44 | 29 | 6 | 4 | 4 | 83 | 48 | 37 |
| #6 | 30KHz | 3 | 350 Hz | 47 | 25 | 18 | 4 | 4 | 4 | 52 | 31 | 24 |

From the TO/FO estimation error in Table 2 and Table 3, it can be observed that the accuracy of quantization (reported interval) is improved significantly when the number of samples is increased from 1 to 3. However, there’re quite similar accuracy of quantization level for the number of samples 3 and 5 for both timing offset and frequency offset error estimation. In our view, there’s no strong reason to consider two more samples to define accuracy for 5 samples since there’s not much improvement in the accuracy from number of 3 samples to 5 samples.

On the other hand, it can be observed that for low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased. Therefore, to guarantee reasonable benefit from the reported intervals of the delay and frequency offsets, higher SNR side condition (3dB) need to be adopted for accuracy requirements.

Proposal 1: RAN4 to consider 3 samples to define RRM requirements for timing and frequency offsets between two TRPs.

Observation 3: At low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased.

Proposal 2: RAN4 to adopt higher SNR side condition for the accuracy requirements of the delay and frequency offsets.

In addition, we recommend considering the maximum span between CDF5% and CDF95% as a reference for the minimum accuracy requirement for both TO and FO measurement. For example, if the reference point (ideal TO) is 0.5 CP (#128 level), in the parameter set#1 of TO estimation error shown in Table 2, the CDF5% is 16(# levels) and 19(# levels) for 3 samples. The maximum span between CDF5% and CDF95% is 19(# levels). If UE send CJT calibration report for TO between two TRPs within the range of 19 (# levels) compared to the reference point (ideal TO: 0.5 CP, #128 level), it would meet the accuracy requirement for TO measurement.

Proposal 3: RAN4 to consider the accuracy requirement as the maximum span between CDF 5% and CDF95% for both TO and FO measurement.

## 2.2 Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

For single DL TRP and multiple UL TRPs, RAN4 discussion on the related issues are captured in the WF [2] as below:

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset? Agreement: For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2. For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not). RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1. - Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s) - Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s). Note 1: the scenario 1 and 2 are based on RAN1 LS. Note 2: The above agreements apply when 2TA are configured. Issue 2-2: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state. - The above agreement can be revisited if not aligned with RAN1 further agreement. Issue 2-3: Whether to define UL TCI state activation requirement based on SRS? Way forward: Further discuss the two proposals and down-select one proposal in the next meeting: - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement. - Proposal 3 (Xiaomi, Nokia): - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP. - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline. - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay. - Further refine the wording for P3 in the WF, if needed

In the last meeting, RAN4 discussed the two scenarios for 2TA enhancement:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s).

For Scenario#1 RAN4 agreed to define timing requirements for FR1. However, for FR2, it was FFS to check whether RF session will make a different conclusion on FR2. In our view, there is no new discussion that will impact FR2 differently from FR1. Therefore, the agreement should also include FR2.

Proposal 4: RAN4 define the timing requirements for FR2 for Scenario#1.

For Scenario#2, RAN4 has sent LS to RAN1 to check whether the previous agreement “One downlink reference timing is supported and applied to both TAGs.” applies to scenario 2. Therefore, RAN4 can wait for the response from RAN1 to discuss the timing requirements for Scenario#2.

Proposal 5: Wait for RAN1 response to discuss the timing requirement for Scenario#2.

It is noted that in the last meeting companies understand that UL TCI state can be associated to SRS. However, in the legacy MIMO requirements, there was no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS. Therefore, similar principle can be reused here in R19 MIMO.

Proposal 6: If UL TCI states can be associated to SRS, then reuse the same principle in Rel-17 which is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

## 2.3 RRM Spec impact for CR drafting

The observed impacted spec based on the current RRM discussion for NR_MIMO_Ph5 (Part2) is shown below. This will be used as a guidance for CR work split.

Observation 4: RRM Spec impact for CR drafting for NR_MIMO_Ph5 (Part2):

| Scenario | Impacted clauses |
| --- | --- |
| - 2TA enhancement for asymmetric DL sTRP/ UL mTRP | 7.1 UE transmit timing 7.1.1 Introduction 7.1.2 Requirements 7.1.2.1 Gradual timing adjustment |
| - Active uplink TCI state switching delay for unified TCI for single-DCI mTRP | 8.16 |
| - CJT calibration using delay/frequency offset measurements and reporting | New sections: 9.X CJT calibration using delay/freq offset measurement for reporting. 9.X.1 Introduction 9.X.2 Requirements applicability 9.X.3 Measurement Reporting Requirements 9.X.4 Delay offset measurement requirements 9.X.5 Frequency offset measurement requirements 9.X.6 Measurement restriction for CSI-RS 9.X.7 Scheduling availability of UE |

# Summary

In this paper, the discussion of R19 MIMO is provided. The following observations and proposals are concluded:

Observation 1: The results at CDF 50% show minimum timing offset estimation error.

Observation 2: The results at CDF 50% show minimum frequency offset estimation error.

Observation 3: At low SNR value (-3dB) the quantization (reported interval) is very inaccurate even when the number of samples is increased.

Observation 4: RRM Spec impact for CR drafting for NR_MIMO_Ph5 (Part2):

| Scenario | Impacted clauses |
| --- | --- |
| - 2TA enhancement for asymmetric DL sTRP/ UL mTRP | 7.1 UE transmit timing 7.1.1 Introduction 7.1.2 Requirements 7.1.2.1 Gradual timing adjustment |
| - Active uplink TCI state switching delay for unified TCI for single-DCI mTRP | 8.16 |
| - CJT calibration using delay/frequency offset measurements and reporting | New sections: 9.X CJT calibration using delay/freq offset measurement for reporting. 9.X.1 Introduction 9.X.2 Requirements applicability 9.X.3 Measurement Reporting Requirements 9.X.4 Delay offset measurement requirements 9.X.5 Frequency offset measurement requirements 9.X.6 Measurement restriction for CSI-RS 9.X.7 Scheduling availability of UE |

Proposal 1: RAN4 to consider 3 samples to define RRM requirements for timing and frequency offsets between two TRPs.

Proposal 2: RAN4 to adopt higher SNR side condition for the accuracy requirements of the delay and frequency offsets.

Proposal 3: RAN4 to consider the accuracy requirement as the maximum span between CDF 5% and CDF95% for both TO and FO measurement.

Proposal 4: RAN4 define the timing requirements for FR2 for Scenario#1.

Proposal 5: Wait for RAN1 response to discuss the timing requirement for Scenario#2.

Proposal 6: If UL TCI states can be associated to SRS, then reuse the same principle in Rel-17 which is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

# Reference

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung

[2] R4-2502596, WF on RRM requirements for NR_MIMO_Ph5 (Part2)