3GPP TSG-RAN WG4 Meeting #114bis	R4-2504606

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.28.2

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for XR enhancements

Document for:	Discussion

# Introduction

RAN4 achieved multiple agreements over the last few meetings on DCI based MG cancellation mechanism introduced by RAN1. In this contribution paper, we further discuss the remaining issues identified in the WF [1] from the last meeting.

# Impact on RRM requirements due to MG deactivation

Firstly, although RAN1 has agreed upon the dynamic indication to enable Tx/Rx in particular gaps, RAN2 may further consider semi-static indication schemes as well, e.g., RRC based MG deactivation pattern. We understand that due to unpredictable nature of the XR data, dynamic indication is better suited from signaling perspective but at the same time, from UE implementation perspective, it may be quite challenging to deactivate a single MG occasion as this needs extensive processing on the modem SW and RF procedures. RRC/MAC-CE based semi-static indications enables the UE to handle MG deactivations in an efficient manner.

Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC or MAC-CE based signaling as well.

A component of an FG can have Option 1 as a first value and Option 2 as a second value:

- Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI.

- Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

Note: UE indicates only one value of this component.

Coming back to the DCI based dynamic indication of MG deactivation, RAN4 has agreed to X = 5 ms as a baseline value and X = 3ms based on optional UE capability. Furthermore, RAN1 discussed the UE behavior for subsequent DCI for the same measurement gap occasion and made the following agreement:

For UEs indicating Option 1 above, UE may simply ignore the MG cancellation indications in all the subsequent DCIs for the same MG occasion. However, for UEs indicating Option 2, our understanding is that the UE will process and honor the subsequent DCIs and if indicated ‘0’, the UE will not cancel the MG occasion. This behavior impacts UE implementation as the procedure to cancel the MG occasion may already have started. Furthermore, RAN1 doesn’t talk about the timeline for such DCI indicating ‘0’. If this indication is received very close the MG occasion, it may not be possible to revert the procedure. We think it’s important to establish the timeline to honor the subsequent DCI indication in Option 2 and this timeline should be no shorter than X (5ms or optional 3ms).

Proposal 2: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI atleast X ms before the MG occasion, where X=5ms as baseline and X=3ms as optional UE capability.

- [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

Since RAN4 agreed that the MG cancellation applies only to L3 measurements and to follow NR-U-like approach for designing the formulas for measurement delay extension, we now need to discuss the exact formula to extend the L3 measurement requirements. We tend to think that it’s straightforward to use NR-U approach and add the number of skipped MGs to the number of SSB samples for each requirement.

Proposal 3: For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

As an example, Table 9.2.6.2-1 can be modified as below:

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, (5+Nskip) x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x (5+Nskip) x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil((5+Nskip) x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |

Some companies also brought various conditions to enable the MG skipping. These included – maximum number of MGs skipped during the measurement period, maximum acceptable time separation between two non-cancelled MGs etc. We are okay to specify the limit on maximum number of skipped MGs within a measurement period and maximum acceptable time separation between two non-cancelled MGs as a part of the requirements or indicate it to the network as UAI.

Regarding the applicability of the MG cancelation in CA, we think it’s straightforward that the indication applies to all the carriers in CA mode if the gap is per UE-gap, and the indication applies to all carriers on the FR where the indication is received if the MG is per-FR.

Proposal 4: If the UE indicates the MG to be per-FR, the MG skipping indication is only valid for the gap instance in the same FR with scheduled XR transmission, otherwise, the indication applies to all the carriers.

# Conclusion

Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well.

Proposal 2: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI atleast X ms before the MG occasion, where

- X=5ms as baseline and X=3ms as optional UE capability.

- [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

Proposal 3: For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

Proposal 4: If the UE indicates the MG to be per-FR, the MG skipping indication is only valid for the gap instance in the same FR with scheduled XR transmission, otherwise, the indication applies to all the carriers.

# References

[1] R4-2502700, “WF on RRM requirements for XR_Ph3”, Nokia, RAN4#114