3GPP TSG-RAN4 Meeting #114Bis	R4-2504609

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	3TX chain TX switching requirements

Source:	Qualcomm Incorporated

Agenda item:	5.3

Release:	Rel-19

Document for:	Approval

# 1.	Introduction

Some companies have been proposing 3TX chain TX switching to be worked on under TEI for multiple meetings [1], [2], [3]. Despite a large list of supporting companies, a WI for this feature was not approved in RAN#106 and it is out understanding that RAN plenary defines the WG work content. In the papers, an impact to RAN1 and RAN2 is proposed and also CRs are provided. In this paper we provide our input mainly to RAN4 proposals.

# 2. 	Discussion

## 2.1 RAN4 timing diagrams and other requirements

The CR for 38101-1, the terminology is again using slot/sub-slot. There is no such thing as sub-slot in 3GPP NR. Also the drawing erroneously implies that the switching occurs on the slot /sub-slot boundary which is not reflected in the CR or 38.214. We propose to use the terminology in e.g. Figure 6.3A.3.3.6-5 or then to change the 38.214 CR to define switching period to align slot boundary

Proposal 1: Do not use slot/sub-slot terminology in the 38.101-1 CRs

The figures are also exactly the same as what is repeated in the current 38.101-1 for five times. It is unclear for us why this same information about the transient period locations need to be repeated so many times without adding any new information. The text in CR could just refer to the existing diagrams since the new ones do not add any new information. The terminology could be changed like this CR is doing, using configurations to define how many carriers and ports are scheduled. Different features then could define the corresponding requirement by changing the context in the configurations and 3GPP RAN4 would take a step towards direction guided in [4].

Since this feature assume that both bands transmit continuously, the single threaded diagrams may not be sufficient for this purpose and two threaded diagram such as in Figure 6.3A.3.3.6-5 should be used.

Proposal 2: Draw timing diagrams in such way that the two band nature is clearly visible and switching period locations and transient periods are clearly shown

The CR use the word “assigned” which also has no meaning and causes problems. We propose to use wording configured or activated.

Proposal 3: Do not use word “assigned” in the RAN4 CRs since it has no meaning.

“Assigned” is used elsewhere but it has proven to be problematic since different companies understand it differently. Our view is that it means “scheduled” since this when UE assigned to do something in UL, configured or assigned in DL direction means that UE starts monitoring.

The reference should point to Table 6.2D.1-1 and not to Table 6.2A.1.3-1since this is when two bands transmit at the same time with one band being two port capable.

Proposal 4: Refer to Table 6.2D.1-1 and not to Table 6.2A.1.3-1 for UE power class.

## 2.2	Applicability of switched UL

The proposal is to define this feature for UEs that have three TX chains and within the context of two bands and switching is done between two ports on one band and one port on an other band i.e. switching the two port transmissions between the two bands as it is outline in the Figure 1 of [3]. In RAN1 CR to 38.214, however, the switchedUL option is included and if UE declares the support for switchedUL option in uplinkTxSwitchingOption, UE is not able simultaneously transmit on two bands. Therefore this feature can be already supported with Rel-17 TX switching feature by declaring uplinkTxSwitchingPeriod2T2T-r17 and adds nothing new.

Observation 1: Allowing switchedUL for 3Tx TX switching is same as Rel-17 2T2T switching feature

Therefore switchedUL option should not be included in this feature. It is hard to discuss this since RAN plenary has not approved an objective for this work but atleast the motivation should be reflected in the normative work.

Proposal 5: Remove switchedUL option from this feature and CRs,e specially the 38.214 CRs

## 2.3	Inclusion of 3Tx

Since we are talking about a UE with 3 TX chains, and as the tone of discussions and the ambiguity of definition of TX chains is not very clear, we should included also cases for 3T+0 and 0+3T in the feature. RAN4 is working with good progress [5] to enable 3Tx under NR_MIMO_Ph5 WI. All the other parts are ready but RN4 just needs to agree that the swithing periods apply to 3Tx case.

Proposal 6: uplinkTxSwitchingPeriod2T2Talso applies to 3T to 3T switching case and 3T to 2T and 3T to 1T and 3T to 0T cases.

UE is then able to define the correct switching period by declaring if it supports 3Tx in the band.  This agreement should be reflected in the RAN2 and RAN1 CRs.

## 2.4	Switching period applicability’s

We discuss in previous section hat the 2T2T can be re-used but it is worth while to think if RSN4 needs to define new capabilities. Switching between two UL MIMO modes may need some extra time in terms of grant preparation.

Observation 2: Switching periods may need more discussion in RAN4 to understand the UE implementation properly

# Conclusion

We observed and proposed.

Proposal 1: Do not use slot/sub-slot terminology in the 38.101-1 CRs

Proposal 2: Draw timing diagrams in such way that the two band nature is clearly visible and switching period locations and transient periods are clearly shown

Observation 1: Allowing switchedUL for 3Tx TX switching is same as Rel-17 2T2T switching feature

Proposal 3: Do not use word “assigned” in the RAN4 CRs since it has no meaning.

Proposal 4: Refer to Table 6.2D.1-1 and not to Table 6.2A.1.3-1 for UE power class.

Proposal 5: Remove switchedUL option from this feature and CRs,e specially the 38.214 CRs

Proposal 6: uplinkTxSwitchingPeriod2T2Talso applies to 3T to 3T switching case and 3T to 2T and 3T to 1T and 3T to 0T cases.

Observation 2: Switching periods may need more discussion in RAN4 to understand the UE implementation properly

As a conclusion, we urge the proponents and supporting companies to carefully review the proposed CRs since even with a small effort we found multiple problem with the proposed CRs.

# Reference

[1]	R4-2417905, “Tx switching for 3Tx Ues”, MediaTek Inc., China Telecom, China Unicom, OPPO, Softbank, T-Mobile USA, Verizon, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, US, 18th – 22nd November, 2024

[2]	RP-243003, “Tx switching for 3Tx UEs”, MediaTek Inc., BT Plc., China Telecom, China Unicom, CHTTL, CKH IOD UK LIMITED, ISSDU, OPPO, Softbank, Telecom Italia, Telstra, T-Mobile USA, Verizon, Vodafone, 3GPP TSG RAN#106, December 9 – 12, 2024, Madrid, Spain

[3]	R4-2501624, “Introducing Tx switching for 3Tx UEs in Rel-19 TEI”, MediaTek Inc., Orange, T-Mobile USA, Vodafone, Verizon, 3GPP TSG-RAN WG4 Meeting #114, Athens, GR, 17th – 21st February, 2025

[4]	RP-250046, “Lean and Streamlined 6G Standards”, Nokia, Apple, AT&T, BT, CMCC, Docomo, DT, Ericsson, Huawei, Intel, KT, MediaTek, Qualcomm, Reliance Jio, Samsung, SKT, T-Mobile USA, Telstra, Verizon, Vodafone, ZTE, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 10-14, 2025

[5]	R4-2503037, “WF on NR MIMO Phase 5”, Samsung, 3GPP TSG-RAN WG4 Meeting #114, Athens, GR, 17th – 21st February, 2025