3GPP TSG-RAN-WG4 Meeting #114bis 	R4-2504614

# Wuhan, China, 7th   – 11th April, 2025

# Title:	MSD for 3Tx UL CA combinations with 2Tx FDD + 1Tx TDD

# Source:	T-Mobile USA

# Agenda Item:	6.8.5

# Document for:	Approval

# Introduction

In the Rel-19 HPUE basket WID, 3 Tx inter-band UL CA combinations with UL MIMO and/or TxD in one of the bands are in scope. Several combinations with UL MIMO and/or TxD in the FDD band have been requested, but none have been specified so far. We were hoping that the existing MSD could be re-used, but this paper shows a potential problem that RAN4 needs to decide how to proceed.

# Discussion

The following combinations with 2Tx FDD have been requested by T-Mobile USA. Only the unique inter-band DL combinations are listed here and not the ones with downlink inter-band plus intra-band DL CA for simplicity, since the MSD is the same.

| Band combination configuration | UpLink configuration | Power class of 1st UL band | Tx number of 1st UL band | Power class of 2nd UL band | Tx number of 2nd UL band | Total Power class | IMD | IMD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A | CA_n25A-n41A | PC2 | 2Tx | PC2 | 1Tx | PC2, PC1.5 | No |  |
| CA_n25A-n77A | CA_n25A-n77A | PC2 | 2Tx | PC2 | 1Tx | PC2, PC1.5 | Yes | IMD2, IMD4 |
| CA_n41A-n66A | CA_n41A-n66A | PC2 | 1Tx | PC2 | 2Tx | PC2, PC1.5 | No |  |
| CA_n41A-n71A | CA_n41A-n71A | PC2 | 1Tx | PC2 | 2Tx | PC2, PC1.5 | Yes | IMD4 |
| CA_n66A-n77A | CA_n66A-n77A | PC2 | 2Tx | PC2 | 1Tx | PC2, PC1.5 | Yes | IMD2, IMD5 |
| CA_n71A-n77A | CA_n71A-n77A | PC2 | 2Tx | PC2 | 1Tx | PC2, PC1.5 | Yes | IMD4 |
| CA_n25A-n41A-n77A | CA_n25A-n41A | PC2 | 2Tx | PC2 | 1Tx | PC2, PC1.5 | Yes | IMD3, IMD5 |
| CA_n41A-n66A-n77A | CA_n41A-n66A | PC2 | 1Tx | PC2 | 2Tx | PC2, PC1.5 | Yes | IMD3 |
| CA_n41A-n66A-n77A | CA_n66A-n77A | PC2 | 2Tx | PC2 | 1Tx | PC2, PC1.5 | Yes | IMD5 |
| CA_n41A-n71A-n77A | CA_n41A-n71A | PC2 | 1Tx | PC2 | 2Tx | PC2, PC1.5 | Yes | IMD2, IMD3, IMD4 |
| CA_n41A-n71A-n77A | CA_n71A-n77A | PC2 | 2Tx | PC2 | 1Tx | PC2, PC1.5 | Yes | IMD2, IMD3 |
| CA_n25A-n41A-n77A | CA_n25A-n77A | PC2 | 2Tx | PC2 | 1Tx | PC2, PC1.5 | Yes | IMD5 |

From 38.880:

For 3Tx PC1.5 IMD MSD:

23+27.8 is used as the power configuration in the 3Tx PC1.5 IMD MSD calculation where band A is 23dBm and band B is 24.8dBm+24.8dBm with 2Tx

For the band combinations whose 2Tx MSD is not specified in the spec yet, 3Tx MSD needs to be evaluated. And it is encouraged to align the requirements between 2Tx and 3Tx band combinations.

Observation 1: For some FDD+TDD uplink combinations the MSD is already specified for PC1.5, but the MSD was calculated by keeping the FDD power at 23 dBm and increasing the TDD power to 27.8 dBm.

Since the above power increase was limited to one band by 4.8 dB, the resulting change in IMD power may not have been the same as if the power was increased in both bands by 3 dB each. For instance, for CA_n71-n77, it for 3 Tx PC1.5, the REFSENS was calculated as follows:

If instead, PC1.5 is achieved using 26 dBm on n71 and 26 dBm on n77, the IMD MSD would have been calculated as follows:

For 3Tx the power is 26 dBm + 23 dBm or 26 dBm total for n71 +  26 dBm for n77, 29 dBm total.

Therefore, the n77 and the n71 power are both increase by 3 dB.

Since the total IMD order is 5, the IMD increase is 3*5=15 dB, compared to 4.8 dB increase if only the n77 power was increased.

So, we can expect 15 dB IMD power increase because the IMD order is 5 and the transmitter power increase is 3 dB.

MSD due to interference power I is given by:

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have:

If I is increased by X dB, then MSD(X) is given by

Based on the 15 dB dB increase intermod power, the resulting MSD is 26.0 dB.

| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 26.0 | FDD | IMD57 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |

This analysis shows that the IMD for 3Tx 26 dBm+26 dBm can be much worse than the 23 dBm + 27.8 dBm IMD MSD, in this case, 10 dB worse.

Observation 2: The IMD analysis above shows that the IMD calculation for 26 dBm + 26 dBm can be significantly worse than for 23 dBm + 27.8 dBm.

Proposal: RAN4 should discuss the above issue and decide how to handle MSD for situations where the MSD calculations result in significantly different MSD for 26 dBm + 26 dBm compared to 23 dBm+27.8 dBm.

# Conclusion

Observation 1: For some 3Tx FDD+TDD uplink combinations the MSD is already specified for PC1.5, but the MSD was calculated by keeping the FDD power at 23 dBm and increasing the TDD power to 27.8 dBm.

Observation 2: The IMD analysis above shows that the IMD calculation for 26 dBm + 26 dBm can be significantly worse than for 23+27.8 dBm.

Proposal: RAN4 should discuss the above issue and decide how to handle MSD for situations where the MSD calculations result in significantly different MSD for 26 dBm + 26 dBm compared to 23 dBm + 27.8 dBm.

# Reference

[1] 3GPP TR38.746: “High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19)”: V0.2.0

[2] 3GPP TS38.101-1: “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 19)”: V19.1.0