3GPP TSG-RAN WG4 Meeting #114-bis                                        R4-2504615

# Wuhan, China, 7th – 11th of April 2025

Title: 	RRM requirements for NTN NB-IoT TDD

Agenda Item:	7.31.5 – RRM core requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC

Document for:	Discussion

# For information

As described in the approved WID RP-242415 for New WID on introduction of IoT-NTN TDD mode (Iridium, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024), the study and work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band.

This work item includes the following objectives:

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2]

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

Current Work Plan from R4-2502291 (from RAN4#114):

| RAN#107 (Mar. 2025) | RAN1#120bis | 0.5 | - Discuss remaining aspects on the TDD support in NB IoT NTN - Discuss (if needed) related RRC parameters and send LS to RAN2 - Discuss Draft CRs |
| --- | --- | --- | --- |
|  | RAN2#129bis | 0.5 | - Further discuss and down-select solutions to support TDD for NB IoT NTN - Conclude on potential impacts on higher layers - Start the definition and configuration of signalling of the periodic pattern, and associated UE procedures (if any) |
|  | RAN4#114bis | 0.5+0.25 | - Discuss the UE AMPR, UE frequency error requirement, - Discuss repetition number to be considered (associated with D/U=8 for 8 ms Rx/Tx time and N=9 for 90 ms periodicity) for both UE and SAN - Discuss (at least) FRC for REFSENS, in-channel selectivity, dynamic range, and related parameters such as number of tones, Tx/Rx time, related SNR target for specific FRC configuration. - Discuss (if needed) the equations to be used to derive (at least) above mentioned requirements based on different FRC configurations discussed. - Discuss work split for UE and SAN RF part (TS 36.102, TS 36.108 and prepare TS 36.181 work split according to potential impacts) |
|  | RAN1#121 | 0.5 | - Conclude on the solutions for the TDD support in NB IoT NTN - Further discuss (if needed) related RRC parameters and send LS to RAN2 - Finalize RAN1 CRs |
|  | RAN2#130 | 0.5 | - Complete the definition and RRC configuration of signaling of the periodic pattern, and associated UE procedures (if any) - Conclude support of TDD mode in NB IoT NTN - Discuss stage-2 design of TDD support in NB IoT NTN - Discuss RAN2 CRs |
|  | RAN4#115 | 0.5+0.25 | - Continue discussion on the UE AMPR, UE frequency error requirement, - Continue discussion on repetition number to be considered (associated with D/U=8 for 8 ms Rx/Tx time and N=9 for 90 ms periodicity) for both UE and SAN, - Continue discussion (at least) FRC for REFSENS, in-channel selectivity, dynamic range, and related parameters such as number of tones, Tx/Rx time, related SNR target for specific FRC configuration, - Derive related values for the at least for previous mentioned requirements: REFSENS, in-channel selectivity, dynamic range, etc., - Start discussion on UE and SAN CRs for RF part and SAN conformance testing, - Start CR split to prepare Draft CRs for the Core requirements |

Previous SAN agreements from RAN4#114 were described in WF R4-2502292 (“Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF”, THALES).

The RAN4#114-bis meeting agenda is:

7.31	Introduction of IoT-NTN TDD mode	[IoT_NTN_TDD]

7.31.1	General aspects	[IoT_NTN_TDD]

7.31.2	Band and system parameters (DL and UL channelization, channel bandwidth)	[IoT_NTN_TDD-Core]

7.31.3	SAN RF requirements	[IoT_NTN_TDD-Core]

7.31.4	UE RF requirements	[IoT_NTN_TDD-Core]

7.31.5	RRM core requirements	[IoT_NTN_TDD-Core]

7.31.6	Moderator summary and conclusions

# RRM TDD NB-IoT specific requirements in TS 36.133

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

At the previous RAN4#114 meeting the following WF has been approved:

R4-2502607  WF on RRM requirements for IoT_NTN_TDD

Type: other                   For: Approval
                                                           Source: Qualcomm

Decision:                   Approved.

With the following agreements during RAN4#114 RRM

Issue 0: General aspects

Agreement:

For TDD NB-IoT in NTN, the following aspects will be further considered in future RAN4 meetings:

- The target SNR

- Applicable coverage (whether the EC needs to be considered)

- Applicable Rmax

- Applicable DRX cycle

  - Note: The DRX cycle might not be non-uniformly periodic due to the H-SFN not matching with the new TDD pattern. This issue will be addressed by other working groups. In the meantime, RAN4 assumes the DRX cycle is uniformly periodic.

Issue 1: Cell reselection (4.6A.2)

Agreement:

For the cell reselection for TDD NB-IoT in NTN, discuss the following details in the next meeting:

- Tdetect, Tevaluation and Tmeasure

Issue 3: RRC re-establishment (6.5A)

Agreement:

In the core requirement, For the delay requirement (TUL_grant + TUE_re-establish_delay_NB-IoT) of RRC re-establishment for TDD NB-IoT in NTN,

- No update to TUL_grant

- For the following parameters in TUE_re-establish_delay_NB-IoT

  - No update:

    - TPRACH_NB-IoT

    - TSI_NB1-NC [and TSI_NB1-EC]

  - Consider update, and the details are FFS:

    - Tsearch_NB1-NC,i [and Tsearch_NB1-EC,i] for unknown cell

Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A)

Agreement:

For the latency requirement (TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH) of RRC Connection redirection to Non-Anchor Carrier for TDD NB-IoT in NTN,

- No update to TRRC_procedure_delay, TUL_grant, and TNPUSCH

- Update the following parameters

  - Tperiod_DL_bitmap: to be updated based on RAN1 design

  - TDL-UL switch: to be updated based on RAN1 design

Issue 5: RLM (7.23A)

Agreement:

Further discuss the following aspects for RLM for TDD NB-IoT in NTN:

- FFS update the hypothetical NPDCCH format (including Rmax) specified in Table 7.23A.2-1 upon RAN1 design in future meetings.

- Evaluation period when DRX is not in use:

  - Option 1: Reuse the existing requirement defined in 7.23A.2.1, unless RAN1 changes cause impact to this requirement

  - Option 2: The evaluation period defined in 7.23A.2.1 is scaled

    - Option 2a: scaled by N=9

    - Option 2b: scaled by FFS

    - Option 2c: no scaling, and define requirement based on 8ms DL subframes

- Evaluation period when DRX is in use:

  - FFS whether the evaluation period defined in 7.23A.2.1 needs to be scaled.

Issue 6: Channel quality report (8.14A.4)

Agreement:

Channel report requirement for TDD NB-IoT in NTN is defined as follows:

- FFS Reuse the existing measurement period

- In future meetings, may need to revisit the following aspects based on RAN1 design:

  - Hypothetical NPDCCH format specified in Table 8.14A.4-1

  - Report mapping tables defined in [9.1.22.15]

Issue 7: PUR (4.6A.3)

Agreement:

The existing PUR requirement defined for NB-NTN applies to the TDD NB-IoT NTN, if PUR requirements is in the WI scope.

Issue 8: Uplink transmit timing (7.20A)

Agreement:

The existing UE transmit timing requirement defined for NB-NTN applies to the TDD NB-IoT NTN.

- If any updates are identified upon RAN1 agreements related to 'uplink transmission gap' and 'segmented uplink transmission,' the corresponding updates will be made accordingly.

And with the following remaining open issues from RAN4#114:

Issue 2: Cell measurement in connected mode (8.14A)

Further discuss the following:

For the cell measurement period in connected mode for TDD NB-IoT in NTN,

- For intra-frequency measurement, the delay requirements shall be scaled by N=9

- For inter-frequency measurement, the minimum length of measurement occasions is defined as the max {existing value, periodicity of corresponding RS}.

- Revise the existing cell measurements condition “at least one 1 DL subframe per radio frame has to be available at the UE during the measurement period.”

- Serving cell measurement requirements apply provided the UE is provided with the configuration about the “active DL” periods.

Proposal 1: Further discuss SNR target and minimum expected value for enhanced coverage (e.g. SNR< - X dB).

Proposal 2: Further discuss cell detect time e.g. 80ms for normal and 1400ms for enhanced coverage.

Proposal 3: (7.23A, RLM; 8.14A Cell measurement in connected mode) Consider a multiple of N=9 (e.g. 9, 18, 36) for the scaling factor.

Proposal 4: (Clause 6.9A, RRC Connection redirection to Non-Anchor Carrier) RAN4 may consider to change TDL-UL switch from 8 to 50ms (based on RAN1 design).

# Conclusion

Proposal 1: Further discuss SNR target and minimum expected value for enhanced coverage (e.g. SNR< - X dB).

Proposal 2: Further discuss cell detect time e.g. 80ms for normal and 1400ms for enhanced coverage.

Proposal 3: (7.23A, RLM; 8.14A Cell measurement in connected mode) Consider a multiple of N=9 (e.g. 9, 18, 36) for the scaling factor.

Proposal 4: (Clause 6.9A, RRC Connection redirection to Non-Anchor Carrier) RAN4 may consider to change TDL-UL switch from 8 to 50ms (based on RAN1 design).