3GPP TSG-RAN4 Meeting #114-bis	R4-2504657

# Wuhan, China, 7th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev |  | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for Rx beam sweeping optimization for SSB based Intra-frequency measurement with NCSG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-03-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to R19 RRM-Ph5 WI, fast beam sweeping (FBS) for L3 measurement reduction in FR2 will introduce new requirement for Intra-frequency measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the scalling factor in clause 9.2.7: - Mpss/sss_sync - Mmeas_period |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R19 RRM-Ph5 enhancment for FBS will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

### 9.2.7	Intra-frequency measurements with NCSG

#### 9.2.7.1	Intra-frequency cell identification

For the UE supporting NCSG, if NCSG is provided, the UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + TSSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2.7.1-1, 9.2.7.1-2, 9.2.7.1-4 (deactivated Scell) or 9.2.7.1-5 (deactivated SCell).

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2.7.1-3 or 9.2.7.1-6 (deactivated SCell).

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2.7.2-1, 9.2.7.2-2, 9.2.7.2-3, 9.2.7.2-4 (deactivated SCell) or 9.2.7.2-5 (deactivated SCell).

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_ncsg,i in clause 9.1.5.3 for measurement conducted within NCSG.

KNCSG is the scaling factor for a SSB frequency layer to be measured within an associated NCSG pattern. KNCSG = 1 when the UE is not configured with concurrent GAPs. Otherwise, KNCSG = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAP and per-FR GAP within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated NCSG within the window W, including those overlapped with other GAP occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated NCSG within the window W after accounting for GAP collisions by applying the GAP collision rule in clauses 9.1.8.3, 9.1.12.3, and 9.1.13.3.

-	xRP = MGRP when configured GAP is MG, and xRP = VIRP when configured GAP is NCSG.

When concurrent GAPs are configured, requirements in this clause do not apply if Navailable =0.

Mpss/sss_sync_with_ncsg: For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync with_ncsg=40/X. For a UE supporting FR2 power class 2, Mpss/sss_sync with_ncsg =24/X. For a UE supporting FR2 power class 3, Mpss/sss_sync with_ncsg =24/X. For a UE supporting power class 4, Mpss/sss_sync with_ncsg =24/X

Mmeas_period_ with_ncsg: For a UE supporting power class 1 or 5, Mmeas_period_ with_ncsg =40/X. For a UE supporting power class 2, Mmeas_period_ with_ncsg =24/X. For a UE supporting power class 3, Mmeas_period_ with_ncsg =24/X. For a UE supporting power class 4, Mmeas_period with_ncsg =24/X.

- X=1, when UE is configured with highSpeedMeasFlagFR2-r17.

- X= [2], when UE supports [R19 multi-Rx] and is configured with [Serving Cell Signal Quality] criterion and UE has fulfilled the [Serving Cell Signal Quality] criterion.

- X=1 otherwise

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with NCSG and smtc2 is partially overlapping with NCSG, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

Table 9.2.7.1-1: Time period for PSS/SSS detection with NCSG (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, 5 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x 5 x KNCSG) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | 5 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Table 9.2.7.1-2: Time period for PSS/SSS detection with NCSG (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, Mpss/sss_sync_with_ncsg x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600ms, ceil(1.5x Mpss/sss_sync_with_ncsg x KNCSG) x max(VIRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Mpss/sss_sync_with_ncsg x KNCSG x max(VIRP, DRX cycle) x CSSFintra |

Table 9.2.7.1-3: Time period for time index detection with NCSG (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, 3 x KNCSG x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil(M2Note 1x 3 x KNCSG) x max(VIRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 3 x KNCSG x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Table 9.2.7.1-4: Time period for PSS/SSS detection with NCSG (deactivated SCell) (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320 ms | 5 x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | 5 x KNCSG x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

Table 9.2.7.1-5: Time period for PSS/SSS detection with NCSG (deactivated SCell) (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Mpss/sss_with_ncsg x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320 ms | Mpss/sss_with_ncsg x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Mpss/sss_with_ncsg x KNCSG x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

Table 9.2.7.1-6: Time period for time index detection with NCSG (deactivated SCell) (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | 3 x KNCSG x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320 ms | 3 x KNCSG x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | 3 x KNCSG x max(measCycleSCell, VIRP,DRX cycle) x CSSFintra |

----------------------END OF CHANGE 1----------------------------