3GPP TSG-RAN WG4 Meeting #114-bis                                        R4-2504658

# Wuhan, China, 7th – 11th of April 2025

Title: 	Considerations for reducing Ku-band coexistence configurations

Agenda Item:	7.11.2 - Coexistence study based on ITU regulations

Release:	Rel-19

Source: 	THALES

Document for:	Discussion

# For information

As described in the approved Way Forward for [112][314] NR_NTN_Ku_Band_General (R4-2413519, Eutelsat Group), the following agreements have been made at the previous RAN4#112 meeting:

#### Issue 1-2: Band definitions

Define as a first Ku band for Region 1 (CEPT countries): 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S).

Additional band definitions are expected once the applicable regulations have been identified and studied.

Then, some other decisions have been taken at RAN4#112-bis - see Way Forward for [112bis][313] NR_NTN_Ku_Band_General, (R4-2416593, Eutelsat Group).

#### Sub-topic 1-2: Region 3 band definition

Consider the following two bands for the NR NTN Ku band for the studied regulations (i.e. Taiwan, Japan, Australia) in the region 3:

- Band nX: downlink 10.70 – 12.75 GHz (Space-to-Earth) and uplink 14 – 14.5 GHz (Earth-to-Space)

- Band nY: downlink 10.70 – 12.75 GHz (Space-to-Earth) and uplink 13.75 – 14 GHz (Earth-to-Space) with priority for nX.

### Sub-topic 1-4: Inclusion of US in WI scope

The US is in scope for the WI using the current allocation of the Ku band.

### Sub-topic 1-5: Region 2 band definition

Agree the band 10.7-12.7 GHz (S-to-E) and 14.0-14.5 GHz (E-to-S) for North America.

Finally, the following WF has been agreed at the meeting RAN4#113 under Way Forward for [113][313] NR_NTN_Ku_Band_General (R4-2419909, Eutelsat Group).

Agree the following band definition:

Table X: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

# Proposals resulted from offline discussions

Observation 1: 100cm aperture VSAT configuration with 88.2dBm has been proposed for a new class of GSO, from a potential Set-2 configuration with antenna gain and transmission power compared with original GSO Set-1 configuration agreed in previous meeting.

Observation 2: For this reason, it does not make sense to consider a multiplication of coexistence simulations. The idea is that 100cm aperture VSAT will connect with GSO Set-2 only.

Proposal 1: Phased array 78 cm x 78 cm VSAT terminal and 100 cm VSAT terminal should consider connectivity with GSO configuration Set-2 only (with lower antenna aperture e.g. 2.5m and lower G/T e.g. 12dB/K compared with original GSO configuration Set-1 agreed in previous meeting).

Proposal 2: Phased array 45 cm x 45 cm VSAT terminal should consider connectivity with original GSO configuration Set-1 and also with LEO (LEO@600km & LEO@1200km).

Proposal 3: LEO@600km should be prioritized compared with LEO@1200km.

Proposal 4: Companies to discuss if Bessel function (for the equivalent circular aperture of 50, 60 or 100 cm) can be still used for the simplification of the VSAT antenna assumptions.

Proposal 5: Prioritize circular aperture of 60 & 100 cm VSAT antenna and map the transmission powers and respective NFs in a table representing the potential combinations of NTN scenarios.

Proposal 6: Companies to further discuss if the calibration scenarios could be avoided (at least for scenarios using equivalent circular aperture).

Observation 3: In the case of phase array VSAT scenarios, some TN-NTN calibration phase might be required in order to double-check interference of adjacent lobes if implemented similarly by all companies.

# Annex Ku-band Agreements at previous meetings

The following agreements have been made at the previous RAN4#112 meeting (see Way Forward [112][314] NR_NTN_Ku_Band_General, R4-2413519, Eutelsat Group) :

#### Issue 1-1: Work plan

#### The work plan in R4-2411506 is approved

#### Issue 1-2: Band definitions

Define as a first Ku band for Region 1 (CEPT countries): 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S).

Additional band definitions are expected once the applicable regulations have been identified and studied.

#### Issue 2-2: Coexistence scenarios

Adopt the interference scenarios in [Intelsat] R4-2411507 for the coexistence studies

#### Issue 2-4: Coexistence frequencies

#### Use 11GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band.

#### Issue 2-8: Coexistence TN channel model

For TN channel model, reuse existing version of TR 38.901.

#### Issue 2-9: Coexistence NTN channel model

For NTN channel model, reuse parameters from TR 38.811 and collect the following missing information:

- Shadow fading and clutter loss tables for the reference frequencies

- Tropospheric scintillation.

#### Issue 2-10: TN Parameters

As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from:

- The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz

- The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

Then, the following decisions have been taken at RAN4#112-bis (see Way Forward for [112bis][313] NR_NTN_Ku_Band_General, R4-2416593, Eutelsat Group), but some decisions were still missing at that time.

Sub-topic 2-1: SAN equivalent antenna aperture

Use 9.1m DL, 7.14m UL (GEO ), 0.91m DL, 0.714m UL (LEO DL 600 & 1200)

Issue 2-8: Coexistence TN channel model

For TN channel model, reuse existing version of TR 38.901.

Sub-topic 2-2: SAN antenna pattern

Use section 6.4.1 from 38.811

Sub-topic 2-3: SAN Tx antenna maximum gain

Use 58.5 dBi (GEO), 38.5 dBi (LEO 600 & 1200)

Sub-topic 2-4: SAN Rx antenna maximum gain

Use 58.5 dBi (GEO), 38.5 dBi (LEO 600 & 1200)

#### Sub-topic 2-5: SAN antenna beamwidth

Use 0.177 (GEO), 1.768 (LEO 600 & 1200)

#### Sub-topic 2-7: SAN max EIRP

43 dBw/MHz (GEO), 13 dBw/MHz (LEO 1200), 7 dBW/MHz (LEO 600) using 100 MHz bandwidth

#### Sub-topic 2-9: VSAT equivalent antenna aperture

Use 60 cm

#### Sub-topic 2-10: VSAT polarization

Assume circular first and if necessary derive the linear result

#### Sub-topic 2-11: VSAT max Tx antenna gain

Use 36.6 dBi (option 1 2W), 36.2 dBi (option 2 10W)

#### Sub-topic 2-12: VSAT max Rx antenna gain

34.3 dBi

#### Sub-topic 2-15: VSAT noise figure

Type 1 2.5 dB for VSAT LEO and GEO

Type 2 6 dB for VSAT to LEO

Other options are not precluded

#### Sub-topic 2-16: VSAT max Tx power

40 dBm (10 W)

#### Sub-topic 2-21: Channel model

38.811 is the channel model  for satellite used for Ka. It can be scaled for Ku.

For TN use 38.863 and change the frequency to 11 & 14 GHz

#### Sub-topic 2-22: TN parameters

NTN to NTN won’t be considered.

Continue with the TN - NTN coexistence study noting that the applicability of the VSAT ACS simulation result may have to be relaxed for the core requirement derivation.

Use the IMT parameters from TR38.922 version 0.3.0 for the IMT study on 15 GHz as a starting point.

Finally the following decisions have been taken at RAN4#113 (see Way Forward [113][313] NR_NTN_Ku_Band_General (R4-2419909, Eutelsat Group).

### Sub-topic 1-1: Defining bands by regulatory authority

Agree the following band definition

Table X: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

### Sub-topic 2-1: VSAT antenna pattern

Agree for VSAT antenna model, to progress the co-ex study with the VSAT equipped with parabolic antenna first. And to further discuss the phased array (flat-panel) antenna modeling in parallel.

Agree to use 38.921 unless there is a need to use sub array then 38.803

The following parameters are agreed:

VSAT phased array antenna characteristics

| Ku band VSAT phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ku band VSAT phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 2) | 0 or 3.5 ? |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 4) | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | [4.44 dBm] TBA |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward X+ axis |

Note 1:	Void

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element).

Note 4:	All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane.

Note 5: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration  (Y axis × Z axis).

Agree 3.5 dB element gain

### Sub-topic 2-2: SAN noise figure

Agree 3 dB

### Sub-topic 2-3: SAN G/T

Agree 30.9 dB/K and 10.9 dB/K based on 3 dB NF

### Sub-topic 2-4: VSAT Max Tx antenna gain

Agree first study is worst case (GSO) of 100 cm aperture (41.3dBi), 88.2 dBm EIRP, other options not precluded e.g. different size, EIRP and antenna type.

The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

### Sub-topic 2-5: VSAT noise figure

Agreed at previous meeting (see also THALES R4-2419724) to have 2.5 dB NF and 6 dB NF (as we did for Ka-band).

### Sub-topic 2-8: SAN antenna pattern

Agreed at previous meeting to use section 6.4.1 from TR 38.811 which corresponds to the same proposal. See also the SAN corresponding antenna beamwidth computed based on this assumption.

### Sub-topic 2-9: TN BS/UE antenna patterns for 11 GHz

Agree to use TN BS and UE antenna and beam forming pattern modelling in TS 38.921 for 11GHz carrier frequency for Ku band coexistence study

### Sub-topic 2-10: TN BS antenna pattern for 14 GHz

Agree to use TN BS antenna and beam forming pattern modelling in TS 38.922 for 14GHz carrier frequency for Ku band coexistence study. For TN UE, the isotropic antenna model will be used as mentioned in TS 38.922.

### Sub-topic 2-17: VSAT max Tx power

Based on sub-topic 2-4, the agreement is 50 W for GSO and 10 W for NGSO.

The following agreements have been made at the previous RAN4#114 meeting (see Way Forward [114][313] NR_NTN_Ku_Band_General, R4-2502290, Eutelsat Group), with the following agreements :

Sub-topic 1-1: Approval of the new work plan

Agree Proposal 1 from Intelsat in R4-2500434: Approve the proposed updated Work Plan proposed in Section 2.5.

Sub-topic 1-2: Definition of extended FR1-NTN and FR2-NTN frequency ranges

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500 MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

Sub-topic 1-3: Ku bands definition

Table 5.2.3-1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127504 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127504 MHz | FDD |
| NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. |  |  |  |

Table 5.2.3-2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127501 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. |  |  |  |

Sub-topic 2-1: Baseline simulation assumptions

In order to coordinate the simulation assumptions, a Tdoc containing the aggregate agreements from all the WF will be maintained. For this meeting that will be in R4-2502315 which will be noted.

Sub-topic 2-3: BS TN antenna pattern

Agree Proposal 1 from Ericsson in R4-2501398: RAN4 to use TN BS antenna parameters of 7125 to 8400 MHz from TR 38.922 Table 5.4.1.2-1 for 11 GHz assumptions.

Sub-topic 2-6: Tx power control model

Agree Proposal 8 from Xiaomi in R4-2500637: It is proposed to reuse transmission power control model in the clause 6.2.6 of TR38.863.

Sub-topic 2-7: Rx power control model

Proposal 9 from Xiaomi in R4-2500637: It is proposed to reuse Received power model in the clause 6.2.7 of TR38.863.

Sub-topic 2-8: Performance metrics

Agree Proposal 10 from Xiaomi in R4-2500637: It is proposed to reuse Performance metric in the clause 6.2.8 of TR38.863.

Sub-topic 2-9: Throughput SNR mapping

Proposal 11 from Xiaomi in R4-2500637: It is proposed to reuse Throughput ~ SNR mapping model in the clause 6.2.9 of TR38.863.

Sub-topic 2-10: Conducted power per antenna element

Agree to specify conducted power per antenna port but the figure is dependent on the decisions on EIRP made in the NTN Ku-band UE RF discussions.

Sub-topic 2-12: Co-existence study scope

Agree Proposal 2 from Ericsson in R4-2501398 assuming that A-ESIM and M-ESIM shall be shut down when approaching land: RAN4 to confirm the scope of the co-existence evaluation based only on L-ESIM.

Sub-topic 3-6: FR1-NTN ARFCN definitions

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

Sub-topic 3-7: FR2-NTN ARFCN definitions

Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1250000 |
| n508 | 120 | 1316672 – <8> – 1333328 | 1113336 – <8> – 1250000 |

Sub-topic 3-8: FR1-NTN GSCN definitions

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 30 kHz | Case C | 12850 – <1> – 14266 |

Sub-topic 3-9: FR2-NTN GSCN definitions

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 kHz | Case D | 12864 – <12> – 14256 |
| n508 | 120 kHz | Case D | 12864 – <12> – 14256 |

Sub-topic 3-10: Default Tx Rx frequency separation

Agreement: Variable TX RX spacing is assumed, how it is specified is FFS.

Ka band FR2 did not specify Tx Rx separation due to the large separation but may need to specify for Ku with close separation.