# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504680

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:			7.8.4

Source:	Ericsson

Title:	Topic summary for [114bis][122] NR_FR1_7MHz_BW

Document for:	Information

# Introduction

The scope of this summary is the introduction of 7MHz Channel Bandwidth for n26 and n5 WI, summarizing contributions submitted under AI 7.8.

Based on the agenda, this summary is divided in 3 topics: the first one handles some general aspect (e.g. Work Plan), the second one handles the system parameters and UE RF aspects while the third one is related to BS RF impacts.

The proposal for this meeting would be to reach agreement at least on the following issues:

- Issue 1-1 (initial BWP size)

- Issue 1-2 (Minimum channel BW for a band)

- Issue 2-1-1 (NS_12 – A-MPR – Frequency carrier Fc)

- Issue 2-1-3 (NS_15 – A-MPR – Frequency carrier Fc)

- Issue 2-1-5 (NS_13)

- Issue 2-1-6 (NS_14)

- Issue 3-1-1 (OTA requirements)

- Issue 3-1-2: New FRCs for 7 MHz channel BW

- Sub-topic 2-2: UE REFSENS

- BS RF: Requirements for which all companies’ proposals are the same.

# Topic #1: General

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503473 | CATT | Proposal 1: RAN4 to clarify in specifications that a cell with SIB1 carrier as 7MHz and an initial DL BWP size between 5MHz and 7MHz is not accessible to a UE not supporting 7MHz channel bandwidth. Proposal 2: RAN4 to clarify in specifications that the addition of 7MHz channel bandwidth to a band shall not alter the minimum channel bandwidth of the band. |
| R4-2503262 | China Telecom | Observation 1: The support of 7MHz is optional for Rel-19 and the previous releases, it’s reasonable that the network shall use the channel bandwidth which is mandatory supported in deployment. Proposal 1: From a specification perspective, there should be no restriction on the initial BWP size. This is more of a deployment-related issue. Observation 2: The channel bandwidth indicated in SIB1 can take any value, and consequently, so can the initial BWP size. Proposal 2: The initial BWP size can assume any value between 0 and the maximum allowable value. |
| R4-2503643 | ZTE Corporation, Sanechips | Proposal1: No need to add any additional restriction for initial BWP size to support of 7 MHz channel BW release independent from Rel-15 in RAN4. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description: Restriction on the initial BWP and 7 MHz channel BW.

Issue 1-1: Initial BWP size and 7 MHz channel BW

- Proposals:

  - Option1: A cell with SIB1 carrier as 7MHz and an initial DL BWP size between 5MHz and 7MHz is not accessible to a UE not supporting 7MHz channel bandwidth. (CATT)

  - Option 2: There should be no restriction on the initial BWP size. This is more of a deployment-related issue (7MHz channel BW is optional for Rel-19 and previous Release). (China Telecom, ZTE)

- Recommended WF

  - No restriction should be needed, to be further discussed.

### Sub-topic 1-2

Sub-topic description: Minimum channel BW for a band.

Issue 1-2: Minimum channel BW

- Proposals: clarify in specifications that the addition of 7MHz channel bandwidth to a band shall not alter the minimum channel bandwidth of the band.

  - Agree

  - Disagree

- Recommended WF

  - To be further discussed

# Topic #2: System parameters and UE RF

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503192 | Apple | Proposal 1: Consider Table 1 when specifying A-MPR for NS_12 Proposal 2: Consider Table 3 and Table 4 when defining A-MPR for NS_15 |
| R4-2503232 | Nokia | Proposal 1: RAN4 shall consider defining A-MPR for n26 PC3 NS_12 as shown in tables 6.2.3.21-1 and 6.2.3.21-2 below. |
| R4-2503474 | CATT | Proposal 1: Except RxBW, all other parameters to derive REFSENS are reused for 7MHz channel bandwidth for band n5. Proposal 2: Specify REFSENS for 7MHz channel bandwidth for band n5 as -96.53dBm. Proposal 3: Except RxBW, all other parameters to derive REFSENS are reused for 7MHz channel bandwidth for band n26. Proposal 4: Specify REFSENS for 7MHz channel bandwidth for band n26 as -96.09dBm. |
| R4-2503642 | ZTE Corporation, Sanechips | Proposal 1: NS_13 should be considered for 7MHz CBW in band n26. Proposal 2: Reuse the A-MPR for NS_13 in 38.101-1 Table 6.2.3.22-2. Update A-MPR regions for 7MHz CBW in 38.101-1 Table 6.2.3.22-1 as follows. Observation 1: A-MPR regions of CBW from 20MHz to 10MHz are becoming smaller for NS_14, and the A-MPR region of 10MHz CBW for NS_14 is very small. Proposal 3: Propose not to consider NS_14 for 7MHz CBW in band n26. |
| R4-2503950 | Ericsson | Proposal1: Consider NS_13 and NS_14 with 7 MHz channel bandwidth and evaluate the corresponding A-MPR needed for the 2 offsets (3 and 8 MHz). Proposal2: Adopt the following REFSENS values (red) for 7 MHz channel bandwidth: |
| R4-2504261 | MediaTek (Wuhan) Inc. | Proposal 1: RAN4 to consider REFSENS as -96.5 dBm for 7MHz CBW with SCS 15kHz for band n5. Proposal 2: RAN4 to consider REFSENS as -96 dBm for 7MHz CBW with SCS 15kHz for band n26. |
| R4-2504484 | Qualcomm Incorporated | Observation 1: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_12 is ≤ 4 dB and ≤ 6 dB respectively. Proposal 1: A-MPR for NS_12 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 4 dB and ≤ 6 dB respectively. Proposal 2: Further study is required over carrier frequencies and remaining sets of waveforms to complete the A-MPR analysis for NS_12. Observation 2: Worst case A-MPR for DFT-s-OFDM and CP-OFDM QPSK waveforms for NS_15 is ≤ 6.5 dB and ≤ 8 dB respectively. Proposal 3: A-MPR for NS_15 for DFT-s-OFDM and CP-OFDM QPSK waveforms shall be ≤ 6.5 dB and ≤ 8 dB respectively. Proposal 4: Further study is required over carrier frequencies and remaining sets of waveforms to complete the A-MPR analysis for NS_15. |

## Open issues summary

### Sub-topic 2-1

Sub-topic description: NS and A-MPR

The proposal is to focus on the A-MPR area definition in this meeting and finalizing the corresponding A-MPR values in the next meeting.

Note regarding the frequency carrier Fc: the center frequencies for A-MPR evaluations should be optimized for the most likely operator allocations:

- e.g. 7 MHz below in 817-824 MHz in areas where the public safety pool extends up to 816 MHz (NS_13)

- 7 MHz above the lower band edge above the public safety pool up to 813.5 MHz, e.g. 3 + 7 MHz in order of frequency if applicable (NS_12)

- 7 MHz above 824 MHz (blocks A, B, A’ and B’) according to operator input.

Issue 2-1-1: NS_12: A-MPR – frequency carrier Fc

- Proposals: Evaluate NS_12 A-MPR for 7 MHz channel BW considering a frequency carrier Fc:

  - Option1 : Fc = 817.5 MHz (Apple, Qualcomm)

  - Option 2 : Fc = 820.5 Mhz (Nokia)

- Recommended WF

  - To be discussed

Issue 2-1-2: NS_12: A-MPR areas

- Proposals: Evaluate NS_12 A-MPR for 7 MHz channel BW considering the following area definition:

  - Option 1 (Apple)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz |  | ≤2.88 | >0 | A1 |

  - Option 2 (Nokia)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | Fc = 820.5 MHz | ≤ 0.72 | ≤ 0.9 | A3 |
|  |  | ≥ 0 | ≥ 1.5*12*SCS*RBstart + 4.8 | A4 |

  - Option 3 (Qualcomm)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | Fc = 817.5 MHz | TBD | TBD | A3 |

- Recommended WF

  - Depending on the agreed Fc value (issue 2-1-1), option 1 (Fc=817.5MHz) or option 2 (Fc=820.5MHz) could be used as starting point.

Issue 2-1-3: NS_15: A-MPR – frequency carrier Fc

- Proposals: Evaluate NS_15 A-MPR for 7 MHz channel BW considering a frequency carrier Fc:

  - Option 1 : 840 < Fc ≤ 845.5 and 837 < Fc ≤ 840 (Apple)

  - Option 2 : Fc = 820.5 MHz (Nokia)

  - Option 3 : Fc = 845.5 MHz (Qualcomm)

- Recommended WF

  - Option1.

820.5 MHz is at the low edge of the band while NS_15 is supposed to protect above band n26.

Issue 2-1-4: NS_15: A-MPR areas

- Proposals: Evaluate NS_15 A-MPR for 7 MHz channel BW considering the following area definition:

  - Option 1 (Apple)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 840 < Fc ≤ 845.5 | ≥4.68 | >0 | A1 |
|  |  | <4.68, ≥2.52 | ≥2.16 | A2 |
|  |  | <1.8 | ≤0.36 | A3 |
|  | 837 < Fc ≤ 840 | ≥0 | ≥2.7 | A7 |

  - Option 2 (Qualcomm)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | Fc = 845.5 MHz | TBD | TBD | A7 |

- Recommended WF

  - This issue is also depending on the agreed Fc value (issue 2-1-3), but option 1 might be used as starting point.

Issue 2-1-5: NS_13

- Proposals: Evaluate A-MPR for NS_13 and for 7 MHz channel BW:

  - Agree (ZTE, Ericsson)

  - Disagree

- Recommended WF

  - Evaluate A-MPR for NS_13 and 7 MHz channel BW.

Issue 2-1-6: NS_14

- Proposals: Evaluate A-MPR for NS_14 and for 7 MHz channel BW:

  - Agree (Ericsson)

  - Disagree (ZTE: no A-MPR is needed for NS_14 and 7 MHz channel BW).

- Recommended WF

  - Check with operators.

### Sub-topic 2-2

Sub-topic description: UE REFSENS

Issue 2-2-1: REFSENS for n5

- Proposals: REFSENS value for 7 MHz channel BW in band n5:

  - Option 1: -96.53 dBm (CATT)

  - Option 2: -96.5 dBm (MediaTek, Ericsson, Qualcomm)

- Recommended WF

  - 7 MHz channel BW REFSENS for n5: -96.5 dBm value (25 RBs uplink configuration).

Issue 2-2-2: REFSENS for n26

- Proposals: REFSENS value for 7 MHz channel BW in band n26:

  - Option 1: -96.09 dBm (CATT)

  - Option 2: -96.0 dBm (MediaTek, Ericsson, Qualcomm)

- Recommended WF

  - 7 MHz channel BW REFSENS for n26: -96.0 dBm value (25 RBs uplink configuration).

### Sub-topic 2-3

Sub-topic description : UE FRCs

Issue 2-3-1: 7 MHz channel BW FRCs

- Proposals: Specify 7 MHz channel BW FRCs

  - Option 1: As specified in R4-2504485 (Qualcomm)

  - Option 2: Other

- Recommended WF

  - To be further discussed

# Topic #3: BS

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2503234 | Nokia | Proposal 1: only conducted BS requirements are covered within this WI. Proposal 2: it is proposed to reuse existing FRCs. |
| R4-2503475 | CATT | Proposal 1: RAN4 to specify REFSENS as the same as that for channel bandwidths 5/10/15MHz. Proposal 2: For conducted Rx dynamic range, RAN4 to specify the interfering signal mean power as -81dBm for WA, -76dBm for MR and -73dBm for LA. Proposal 3: For radiated Rx dynamic range, RAN4 to specify the interfering signal mean power as -81- ΔOTAREFSENS for WA, -76- ΔOTAREFSENS for MR, and -73- ΔOTAREFSENS for LA. |
| R4-2503643 | ZTE Corporation, Sanechips | Proposal1: No need to add any additional restriction for initial BWP size to support of 7 MHz channel BW release independent from Rel-15 in RAN4. Proposal2: Supporting to specify BS OTA requirements in Rel 19. Observation1：The FRCs for NB-IoT operation are defined in TS 36.104, and FRCs for wideband operation are specified in annex A.1 and annex A.2 in TS 38.104. Proposal3: For NB-IoT operation in NR in band, the legacy FRCs defined in TS 36.104 can be reused. Proposal4: For wideband operation in NR in band, to define new FRCs for new added 7MHz/15 k SCS CBW. Proposal5: To define the FRC parameters marked in blue in table A.1-1 for 7 MHz CBW FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity. Proposal6: To define the FRC parameters marked in blue in table A.2-1 for 7 MHz CBW FR1 dynamic range and OTA dynamic range. Proposal7: Supporting to prioritize the conducted Tx/Rx requirements discussion. Proposal8: For ACLR/CACLR, the existing requirements are reused for 7 MHz CBW. Proposal9:To consider the proposals in Table 1 for BS RF requirement for 7 MHz channel bandwidths. |
| R4-2503951 | Ericsson | Proposal1: Do not specify 7 MHz FRC and reuse the existing 5 MHz FRCs to test Rx requirements for 7 MHz channel bandwidth. Proposal2: Specify the total power dynamic range for 7 MHz channel bandwidth and 15kHz SCS with 15.4 dB value. Proposal3: Specify REFSENS for 7 MHz channel BW according to the following table: Proposal4: Specify Dynamic Range for 7MHz channel BW according to the following table:. Proposal5: Specify the ACS interferer frequency offset with ±2.5075 MHz value. Proposal6: Specify the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap with value: ±(360+m*180) kHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24. Proposal7: Specify the interfering signals for the intermodulation requirement according to the following table: Proposal9: Specify the In-channel selectivity requirements for 7 MHz channel BW based on the requirements for 5 MHz channel BW. |
| R4-2504588 | Huawei, HiSilicon | Proposal 1: Introduce complementary OTA requirements for all applicable NR network-nodes specifications under 7 MHz channel bandwidth WI. Proposal 2: Introduce OTA requirements for the following specifications: - Draft CR to TS 37.105: 7MHz channel bandwidth introduction - Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction - Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction - Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction |

## Open issues summary

### Sub-topic 3-1

Sub-topic description: General issues.

Issue 3-1-1: OTA requirements

- Proposals: Specify OTA requirements for 7 MHz channel BW

  - Agree (Huawei).

    - Note: ZTE would agree specifying OTA requirements but as second priority.

  - Disagree (Nokia).

- Recommended WF

Note that if RAN4 agrees to specify OTA requirements for 7 MHz channel BW, the following specifications shall be added to the WID: TS 37.105, TS 37.145-1, TS 37.145-2, TS 37.141-2.

Issue 3-1-2: FRCs for Rx requirements

- Proposals: Specify new FRC for 7 MHz channel BW

  - Agree (ZTE)

  - Disagree and reuse existing ones (Nokia, CATT, Ericsson).

- Recommended WF

  - The majority’s view is that no new FRC will be specified with 7 MHz channel BW. To be checked with ZTE if acceptable.

### Sub-topic 3-2

Sub-topic description: BS RF Tx requirements.

Issue 3-2-1: Output power dynamics

- Proposals:

  - Option1 (Nokia, ZTE, Ericsson)

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 7 | 15.4 | N/A | N/A |

  - Other option

- Recommended WF

  - Option 1

Issue 3-2-2: Output power dynamics – NB-IoT in-band

- Proposals:

  - Option1 (Nokia, ZTE)

| BS channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 7, 10 | Any | +6 |

  - Other option

- Recommended WF

  - Option 1

Issue 3-2-3: ACLR

- Proposals: For 7 MHz channel BW, the ACLR limit, the ACLR limit in non-contiguous spectrum or multiple bands and the CACLR limit shall be the same ones as for 5 MHz channel BW (see R4-2503234).

  - Agree (Nokia, ZTE)

  - Disagree

- Recommended WF

  - For 7 MHz channel BW, the ACLR limit, the ACLR limit in non-contiguous spectrum or multiple bands and the CACLR limit shall be the same ones as for 5 MHz channel BW (see R4-2503234).

### Sub-topic 3-3

Sub-topic description: BS RF Tx requirements.

Issue 3-3-1: REFSENS

- Proposals: Specify REFSENS for 7 MHz channel BW according to the following table:

  - Option1 (Nokia, CATT, Ericsson)

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -101.7 | -96.7 | -93.7 |

  - Option2 (ZTE) – new FRC for 7 MHz channel BW

| FRC for 7 MHz CBW | SNR | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- |
|  |  | For WA | For MR | For LA |
| G-FR1-A1-[X] | -1.23 dB | -100.2 | -95.2 | -92.2 |
| G-FR1-A1-[X’] | -1.27dB | -100.2 | -95.2 | -92.2 |

  - Other option

- Recommended WF

  - Pending on issue 3-1-2, option 1 or option 2.

Issue 3-3-2: Dynamic range

- Proposals: Specify Dynamic Range for 7MHz channel BW according to the following table:

  - Option1 (Nokia, CATT, Ericsson).

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA | WA | MR | LA |
| 7 | 15 | G-FR1-A2-1 (Note 1) | -70.7 | -65.7 | -62.7 | -81.0 | -76.0 | -73.0 |

  - Option2 (ZTE) – new FRC for 7 MHz channel BW

| FRC for 7 MHz CBW | SNR | Pwanted(dBm) |  |  |
| --- | --- | --- | --- | --- |
|  |  | For WA | For MR | For LA |
| G-FR1-A2-[Y] | 8.977 | -69.5 | -64.5 | -61.5 |

Table2.3.4-1 The interfering signal PIntf

| FRC for 7 MHz CBW | PIntf for WA | PIntf for MR | PIntf for LA |
| --- | --- | --- | --- |
| G-FR1-A2-[Y] | -81.0 dBm | -76.0 dBm | -73.0 dBm |

  - Other option

- Recommended WF

  - Pending on issue 3-1-2, option 1 or option 2.

Issue 3-3-3: Dynamic range for NB-IoT operation in NR in-band

- Proposals: Specify Dynamic Range for NB-IoT operation in NR in-band and for 7MHz channel BW according to the following table:

  - Option1 (Nokia).

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 7 | FRC A15-1 in Annex A.15 in TS 36.104 [13] | -99.7 | -81 | AWGN |
| 7 | FRC A15-2 in Annex A.15 in TS 36.104 [13] | -105.6 | -81 | AWGN |

  - Other option

- Recommended WF

  - Option 1

Issue 3-3-4: ACS

- Proposals: Specify ACS for 7MHz channel BW according to the following tables:

  - Option1 (Nokia, Ericsson, ZTE).

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3 | PREFSENS + 8 dB | Wide Area BS: -52 Medium Range BS: -47 Local Area BS: -44 |
| 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB |  |

Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 7, 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |

  - Other option

- Recommended WF

  - Option 1.

Issue 3-3-5: In-band general blocking

- Proposals: Specify In-band general blocking for 7MHz channel BW according to the following table:

  - Option1 (Nokia).

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20 | PREFSENS + x dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |

  - Other option

- Recommended WF

  - Option 1

Issue 3-3-6: Narrowband blocking - limit

- Proposals: Specify narrowband blocking limit for 7MHz channel BW according to the following tables:

  - Option1 (Nokia).

Table 7.4.2.2-2: Base Station narrowband blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area BS: -49 Medium Range BS: -44 Local Area BS: -41 |

Table 7.4.2.2-2a: Base Station narrowband blocking requirement for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50 | PREFSENS + x dB (Note 2) | Wide Area: -49 Medium Range: -44 Local Area: -41 |

  - Other option

- Recommended WF

  - Option 1

Issue 3-3-7: Narrowband blocking - interferer

- Proposals: Specify narrowband blocking interferer for 7MHz channel BW according to the following table:

  - Option1 (Nokia, ZTE).

Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 7, 10 | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |

  - Option2 (Ericsson).

Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 7 | ±(360+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |

  - Other option

- Recommended WF

  - To be discussed

Issue 3-3-8: Intermodulation interferer

- Proposals: Specify intermodulation interferer for 7MHz channel BW according to the following tables:

  - Option1 (Nokia, ZTE, Ericsson).

Table 7.7.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 7 | ±7.45 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

  - Other option

- Recommended WF

  - Option 1

Issue 3-3-9: Narrowband Intermodulation interferer

- Proposals: Specify narrowband intermodulation interferer for 7MHz channel BW according to

  - Option1 (Nokia).

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 7 | ±360 | CW |
|  | ±1160 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

  - Option2 (ZTE).

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 7 | ±355 | CW |
|  | ±1240 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

Moderator’s note: ZTE initially proposed 400kHz but changed their proposal to 355kHz when reviewing this summary.

  - Other option

- Recommended WF

  - To be discussed

Issue 3-3-10: ICS

- Proposals: Specify ICS for 7MHz channel BW according to

  - Option1 (Nokia).

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 7 | 15 | G-FR1-A1-7 | -100.6 | -79.9 | DFT-s-OFDM NR signal, 15 kHz SCS, 14 RBs |

  - Option2 (ZTE) – new FRC for 7 MHz channel BW

| FRC for 7 MHz CBW | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) |  |  | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | for WA | for MR | for LA | for WA | for MR | for LA | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| G-FR1-A1-[X’’] | -99.6 | -94.6 | -91.6 | -79.6 | -74.6 | -71.6 |  |

  - Option3 (Ericsson).

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5, 7 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |

  - Other option

- Recommended WF

Issue 3-3-11: ICS for NB-IoT operation in NR in-band

- Proposals: Specify ICS for NB-IoT operation in NR in-band for 7MHz channel BW according to

  - Option1 (Nokia).

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 7 | FRC A14-1 in Annex A.14 in TS 36.104 [13] | -124.3 | -79.9 | DFT-s-OFDM NR signal, 15 kHz SCS, 14 RBs |
| 7 | FRC A14-2 in Annex A.14 in TS 36.104 [13] | -130.2 | -79.9 | DFT-s-OFDM NR signal, 15 kHz SCS, 14 RBs |

  - Other option

- Recommended WF

  - Pending on issue 3-3-10 (Number of RBs for the interfering signal).

Issue 3-3-12: EVM

- Proposals: Specify EVM window length for 7MHz channel BW according to

  - Option1 (Nokia).

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 7, 10 | 1024 | 72 | 28 | 40 |

  - Other option

- Recommended WF

  - Option 1.

# Topic #4: Draft CR

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| UE |  |  |
| R4-2503476 | CATT | draftCR to TS 38.115-1 on 7MHz channel bandwidth Note: exclusion of 7 MHz channel BW |
| R4-2503276 | Huawei Technologies France | draftCR for Tx RF requirements for introduction of 7MHz Note: Capture RAN4#114 agreements |
| R4-2504475 | Nokia | draft CR to 38.101-1: Introduction of 7 MHz channel bandwidth for n26 and n5 Note: Capture RAN4#114 agreements |
| R4-2504485 | Qualcomm Incorporated | 7 MHz Channel Bandwidth for n5 and n26 Note: Capture RAN4#114 agreements + REFSENS proposal + FRCs proposal |
| BS |  |  |
| R4-2503644 | ZTE Corporation, Sanechips | Draft CR to TS37.104: Introduction of 7 MHz NR FR1 channel bandwidth Note: Some RAN4#114 agreements + ZTE proposals for this meeting |
| R4-2503952 | Ericsson | Draft running CR to TS 38.104 - Introduction of 7 MHz channel BW Note: Capture RAN4#114 agreements |
| R4-2504589 | Huawei, HiSilicon | Draft CR to TS 37.105: 7MHz channel bandwidth introduction |
| R4-2504590 | Huawei, HiSilicon | Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction |
| R4-2504591 | Huawei, HiSilicon | Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction |
| R4-2504592 | Huawei, HiSilicon | Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction |
| R4-2504593 | Huawei, HiSilicon | Draft CR to TS 38.176-2: restriction of 7MHz channel bandwidth introduction Note: exclusion of 7 MHz channel BW |

## Open issues summary

### Sub-topic 4-1

Sub-topic description: draft CRs

Issue 4-1-1: Note used to indicate 7 MHz channel BW is not in the scope

- Proposals: Where should we add the note indicating that 7 MHz channel BW is not in the scope of IAB and NCR specifications:

  - Option1: In the scope and in the “Operating bands and channel arrangement” subclause (Huawei, R4-2504593)

  - Option 2: In the “Operating bands and channel arrangement” only subclause (CATT, R4-2503476)

- Recommended WF

  - To be discussed

Issue 4-1-2: draft CRs to be endorsed

- Proposals: Following draft CRs to be endorsed:

  - R4-2503276 (Huawei)

  - R4-2504475 (Nokia)

  - R4-2504485 (Qualcomm)

  - R4-2503644 (ZTE)

  - R4-2503952 (Ericsson)

  - Others: R4-2503476 (CATT), R4-2504589 (Huawei), R4-2504590 (Huawei), R4-2504591 (Huawei), R4-2504592 (Huawei), R4-2504593 (Huawei)

- Recommended WF

  - R4-2503276, R4-2504475, R4-2503952 might be endorsed (they captured RAN4#114 agreements).

  - R4-2503644 and R4-2504485 should be postponed or revised.

  - R4-2503476 and R4-2504593 are discussed in issue 4-1-1.

  - R4-2504589, R4-2504590, R4-2504591 and R4-2504592 are pending on issue 3-1-1 and should most likely be postponed.

  - Note that, according to the agreed RAN4#114 WF, R4-2503476, R4-2504590, R4-2504591, R4-2504592 and R4-2504593 should have not been submitted in this meeting.