# 3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504906

Wuhan, China, 7th - 11th April 2025

Agenda item:			7.25.3

Source:	Ericsson

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part1

Document for:	Approval

# Introduction

Note: The detail open issue list is captured in the moderator summary R4-2503625, R4-2504920.

# Topic #1: On-demand SSB(OD-SSB) requirements

# OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 1: Lower bound for measurement in FMW

Agreement

Introduce UE capability on whether a lower bound is needed, i.e.,

  - The lower bound is not needed (default value), or,

  - A lower bound is needed and based on OD-SSB periodicity of [10] ms

    - Further confirm whether [10] is feasible, and only one value

Sub-issue 2: CSSF

Agreement

- When NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer

  - Extend the FMW window time by one OD-SSB periodicity

  - Longer measurement delay of other frequency layers with AO-SSBs are expected

Sub-issue 3: OD-SSB in FMW collided with MG

Wayforward

- FFS UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

Sub-issue 5: Searcher assumption

Agreement

- RAN4 shall not change searcher assumption in this work item.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

- Background

  - The deactivated SCell measurement requirement in FR1 is as follow.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

  - The OD-SSB based deactivated SCell measurement procedure is shown as follow.

Sub-issue 1: The start point of T2

Wayforward

The start point of T2 is

- Option 1:

  - The time point of OD-SSB deactivation

- Option 2:

  - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

- Option 2a:

  - The time point of OD-SSB activation

Sub-issue 2: The minimum interval(T2) between multiple FMWs

Wayforward

- Option 1: 5* measCycleSCell

- Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity)

- Option 1b: 5*max (measCycleSCell, DRX cycles)

- Option 2: Minimum measurement period defined in existing spec.

- Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1

Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2

Wayforward

- Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

- Option 2: The UE is assumed to perform fast mode measurement without cell identification period.

- Option 3: No requirement for UE within T2.

Issue 1-1-3: OD-SSB based deactivated SCell measurement – FMW start point

Close the issue. Check the CR directly.

# OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-2-1: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreement

    - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow.

      - max(5*measCycleSCell,  5*DRX cycles) for FR1, or

      - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

    - Otherwise, the same approach in Case 1 can be reused.

    - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation.

    - Note 2: T2 discussion is a separate issue.

Issue 1-2-2: Requirement for RAN1 Alt Time-C1

Agreement:

- RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario.

  - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification.

  - FFS for other procedures.

Issue 1-2-3: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: Define the requirement based on OD-SSB periodicity.

- Option 2: Not to define requirement.

Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

Agreement

RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015.

- Option 1: RAN4 to define requirement for different frequencies

- Option 2: RAN4 not to define requirements for different frequencies

# OD-SSB based SCell activation

Issue 1-3-2: The known cell condition – period of known condition

- Wayforward

Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

Issue 1-3-3: SCell activation requirement – uncertainty time

Close the issue. Check the CR directly.

Issue 1-3-5: Miscellaneous

Issue 1: For the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication, the OD-SSB provided better valid measurement report can be used for SCell activation.

- Wayforward

Withdrawn the issue 1 in 1-3-5.

Issue 1-3-6: SCell activation detail requirement

Close the issue. Check the CR directly.

# OD-SSB general issues

Issue 1-4-1: Additional Processing time for OD-SSB reception – scenario

Issue 1-4-2: Additional Processing time for OD-SSB reception - value

- Background

In RAN4 #113 meeting, RAN4 achieved agreements as follow.

Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

- Information

RAN1 #120-bis Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. - If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ

Agreement

UE applies the additional processing timing only in deactivated SCell measurement.

- Note: The additional UE processing timing is not applied to SCell activation procedure.

The additional processing timing is T_processing, the value of T_processing:

- Introduce different values with UE capability: [2 or 3]ms and 5ms.

Issue 1-4-3: Additional Processing time for OD-SSB deactivation

Wayforward

- FFS RAN4 to consider defining the same timeline for activating and deactivating OD-SSB

Issue 1-4-4: L1 measurement after SCell activation when DRX is configured

Agreement

- For Scenario 2 and Scenario 2a, when DRX is configured, following the legacy requirements based on DRX with the consideration of OD-SSB is not deactivated.

Issue 1-4-5: L1 measurement during and after SCell activation

Merge to issue 1-2-2. Close the issue.

Issue 1-4-6: Scheduling restriction for L1 measurement after SCell activation

Wayforward

- FFS In Case 2 L1-RSRP measurement, no scheduling restriction and measurement restriction are expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

Issue 1-4-7: OD-SSB based Neighbor cell measurement in FR2

Agreement

- Neighbor cell measurements for FR2 should follow legacy requirements and no change in Rx beam sweeping is required.

Issue 1-4-8: OD-SSB based Neighbor cell measurement before OD-SSB activation

Wayforward

- RAN4 to discuss whether to perform measurement on neighbour cells’ in the same frequency of deactivated SCell before OD-SSB activation for Case#1.

Issue 1-4-9: Interruption

Agreement

- New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation.

- New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command.

- The interruption can be applied in both Case 1 and Case 2.

- The detail condition, duration and position of the interruption is FFS.

- The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation.

- No new interruption requirement is defined.

- The interruption requirement follows the same principle of legacy SCell activation.

# Topic #2: General (AI 7.25. 1)

# Sub-topic 2-1: General

Issue 2-1-1: OD-SSB based multiple SCells activation

Wayforward

- Option 1:

  - RAN4 to define OD-SSB based multiple SCells activation delay requirements.

    - The limitation is needed

    - It doesn’t impact the WI’s completion.

- Option 2:

  - Not define multiple OD-SSB carriers requirement in this release.

Issue 2-1-2: Fast SCell Activation

Agreement

- RAN4 to only update the known cell condition considering OD-SSB for Rel-17 fast SCell activation.

  - The requirement is based on A-TRS of Rel-17 fast SCell activation requirement.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

Issue 2-1-3: Deactivated PUCCH SCell activation

Agreement

- RAN4 to define OD-SSB based deactivated PUCCH SCell activation requirement.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

Issue 2-1-4: Rel-18 SCell Activation with the L3 reporting during activation

Agreement

- FFS RAN4 to define OD-SSB based Rel-18 SCell activation with the L3 reporting requirement.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

# Sub-topic 2-2: CR split

Agreement

| Section | Title | Company |
| --- | --- | --- |
| New | Interruptions for OD-SSB transmission | Apple |
| 8.2.2.2.1 | Interruptions at SCell addition/release | Xiaomi |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation | Xiaomi |
| New | OD-SSB based SCell Activation Delay Requirement for Deactivated SCell | Samsung |
| New | OD-SSB based direct SCell Activation at SCell addition | CMCC |
| New | OD-SSB based SCell Activation Delay Requirement for Deactivated PUCCH SCell | Huawei |
| New | OD-SSB based SCell Deactivation Delay Requirement for Activated SCell | LGE |
| New | Fast SCell Activation Delay Requirement for Deactivated SCell with OD-SSB | OPPO |
| New | OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation | Nokia |
| 9.1.5 | Carrier-specific scaling factor | CATT |
| New | OD-SSB based NR intra-frequency measurements · Intra-frequency cell identification · Measurement period | Ericsson |
| New | OD-SSB based NR intra-frequency measurements · Scheduling availability of UE during intra-frequency measurements | Intel |
| 9.5 | L1-RSRP measurements for Reporting - Introduction - L1-RSRP measurement requirements - Measurement restriction for CSI-RS and SSB for L1-RSRP measurement | China Telecom |
| 9.5 | L1-RSRP measurements for Reporting - Scheduling availability of UE during L1-RSRP measurement | ZTE |
| 9.8 | L1-SINR measurements for Reporting - Introduction - L1-SINR measurement requirements, -Measurement restriction for L1-SINR measurement | MTK |
| 9.8 | L1-SINR measurements for Reporting - Scheduling availability of UE during L1-SINR measurement | vivo |