3GPP TSG-RAN WG4 Meeting # 114-bis	             R4-2504912

# Wuhan , CN, 7th – 11st Apr, 2025

Agenda Item:	7.29.5

Source: 	Moderator (CATT)

Title: 	WF on RRM requirements for NR_NTN_Ph3_Part1

Document for:	Approval

0 Notes:

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other options can be proposed.

- < Tentative Agreement > will be changed to < Agreement > if no comments are received.

1 Topic #1: General consideration on RRM requirements

    - Sub-topic 1-1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

Issue 1-1-1: SDT requirements

- Background

RAN4#114: #### Issue 1-1-2: SDT requirements Agreement: - For SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and RAN4 can refer to the requirements on TA validation for the PUR requirements for IoT-NTN, that is, - Define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Further discuss whether to send RAN2 LS

<Way Forward>:

- Proposal 1: For the conclusion on how to define CG-SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2.

  - The agreement reached in the last meeting only focus on RAN4’s requirements.

- Proposal 2: RAN4 to send LS to RAN2 informing them about the RAN4 agreement on the removal of the RSRP change criterion for TA validation in CG-SDT for (e)RedCap UE in FR1-NTN and on the reuse of the TA validation criteria as specified for PUR in IOT-NTN.

Issue 1-1-2: MDT requirements

- Background

RAN4#114: #### Issue 1-1-3: MDT requirements Agreement: - For MDT requirements in RRC_IDLE and RRC_INACTIVE state, - Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.

<Way Forward>:

- Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN.

    - Sub-topic 1-2 Features to be considered for (e)RedCap UE with FR1-NTN

Issue 1-2-1: SMTCs and 'in parallel' related

Issue 1-2-1-1: Feature 25-1(parallelSMTC)

Online agreement

Agreement:

- For Feature 25-1 (parallelSMTC), not differentiate 1Rx and 2Rx RedCap UEs for this UE capability.

- RedCap UE is mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier

    - ‘Parallel’ means the SMTCs are TDM with proximity distance.

Issue 1-2-1-1a: The proximity distance between two SMTCs

<Way Forward>:

- When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

    - the two SMTC occasions are fully or partially overlapping in time domain, or

    - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms.

- Y=5 ms

Issue 1-2-1-2: The number of cells/SSBs/supported satellites

Ad-hoc agreement

Agreement:

- For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for (e)RedCap UEs for both intra-frequency and inter-frequency measurement, and it will be optional with capability signalling.

- For the supported number of cells/SSBs, the following legacy requirement also can be reused for (e)RedCap UEs:

    - For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

- 8 identified cells, and

- 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

- 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

- In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

Issue 1-2-1-3: Feature 25-3 (parallelMeasurementGap)

Online agreement

Agreement:

- Optional feature 25-3 (parallelMeasurementGap-r17) is applicable for (e)RedCap UE and is left to UE implementation, RAN4 to define RRM requirements following the same principle of the existing NR NTN requirements.

- This capability supported by (e)RedCap UEs is not differentiated between 2Rx and 1Rx.

Issue 1-2-2: NCD-SSB

- Background

#### RAN4#112: #### Issue 6-6-1: The impact of NCD-SSB for (e)RedCap UEs with FR1-NTN <Agreement> - For (e)RedCap UEs with FR1-NTN, there is no need to define NCD-SSB specific measurement requirements.

Agreement:

- Use the terminology of SSB without differentiating CD-SSB/NCD-SSB.

- No need to add additional requirement applicability about CD-SSB/NCD-SSB.

- Test cases are defined based on CD-SSB.

Issue 1-2-3: Other features

Online agreement

Agreement:

- SMTC adjustment in idle/inactive is kept [mandatory] for (e)RedCap UEs with NTN.

- From RAN4 perspective, location-based measurement report trigger is kept optional for (e)RedCap UEs with NTN.

2 Topic #2: Reduction in the number of UE Rx branches

    - Sub-topic 2-1: RRC_IDLE/RRC_INACTIVE mode mobility

Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

Ad-hoc agreement

Agreement:

- For the enhanced cell reselection measurement requirements，

    - Enhanced RRM requirements for measurements in IDLE/INACTIVE modes are kept optional for (e)RedCap NTN UE, and RAN4 to reuse the existing requirements.

    - RAN4 only to define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX cycle.

    - Reuse the requirements for normal UE except for Tdetect being extended by [2] DRX cycles.

    - The requirements don’t differentiate 1Rx and 2Rx.

Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

Ad-hoc agreement

Agreement:

- For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN,

    - For 1 Rx and 2 Rx RedCap UEs not configured with eDRX cycle, RAN4 to reuse the existing requirements defined for normal UEs with FR1-NTN (cl. 4.2C.2.11).

    - For 1 Rx and 2 Rx RedCap UEs configured with eDRX cycle, RAN4 to reuse the existing requirements (Table 4.2B.2.5-2) defined for (e)RedCap UE with TN (cl. 4.2B.2.5) as a baseline and adopt NTN principles on how measurements are triggered/etc when implementing the requirement in the specification. Note that only eDRX cycles up to 10.24 sec are applicable for (e)RedCap NR-NTN UEs.

- Note: The eDRX related impact and above principle also apply to other agreed cell re-selection measurement requirements in the previous meeting.

Issue 2-1-3: SDT requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

Ad-hoc agreement

Agreement:

- RAN4 to keep the +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 as used for 1Rx (e)RedCap UE in TN.

- For RA-SDT requirements, the legacy requirements defined for (e)RedCap UE with TN can be reused for (e)RedCap UE with FR1-NTN with the following adaptation:

    - For the SDT requirements based on 2-step RA and 4-step RA, RAN4 to refer to clauses 6.2C.2.3 and 6.2C.2.2 respectively.

    - Sub-topic 2-2: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Issue 2-2-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Ad-hoc agreement

Agreement:

    - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

    - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

      - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

3 Topic #3: The impact of HD-FDD

    - Sub-topic 3-1 General HD-FDD related impact

Issue 3-1-1: General HD-FDD related impact

Online agreement

Agreement:

- The legacy requirements and applicable conditions can be reused as baseline:

    - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

    - Handover interruption time

    - Random access

    - SA: RRC Connection Release with Redirection

    - [Minimum requirement for L1 indication for RLM and LR]

    - Scheduling availability of UE performing intra/inter measurements

RAN4 to further discuss:

  - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD:

    - FFS: Scheduling availability of UE during L1-RSRP measurement

    - Other requirement is not precluded

  - Candidate options for the potential spec impact for each of the above requirements (if needed):

    - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification.

    - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

    - Option c: The requirement applies when there is at least one RS within each measuremen interval.

    - FFS the same or different option for DRX and non-DRX

    - Sub-topic 3-2 The specific impact of HD-FDD on NR Handover

Issue 3-2-1: The impact on NR Handover due to HD-FDD

Online agreement

Agreement:

- Option 1: agree on the principle below and further discuss the wording in the CR:

    - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU

    - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- One SSB is available during T∆

- One SSB is available during TIU

- Option 2: No spec impact

Issue 3-2-2: The impact on other requirements due to HD-FDD

<Way Forward>:

- Proposal 1: Clarify that the measurement requirements apply provided that no RS is overrode by UL transmission during the measurement period. Otherwise, longer delays can be expected.

- Proposal 2: For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured.

- Proposal 3: For scheduling availability of UE during L1-RSRP measurement, update the principle as scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

4 Topic #4: Others

    - Sub-topic 4-1 UE capabilities for (e)RedCap UE with FR1-NTN bands

Issue 4-1-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>:

- Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Proposal 2: Introduce new capability to declare support over RedCap over NTN.

- Proposal 3: RAN4 to not discuss or conclude on UE capability signaling and structure for RedCap NTN, unless specific requests are received from other working groups via LSs.

CR split

| CR index | Clauses | Notes | Company |
| --- | --- | --- | --- |
| 1 | 3.6.Y Applicability of requirements for RedCap UEs with NTN |  | CATT |
|  | 4.1 Cell Selection | Since there is no new clause for both RedCap and NTN in legacy, so only clarify that the offset defined for RedCap with TN is also applicable to RedCap UEs with NTN. |  |
| 2 | 4.2X Cell Re-selection for RedCap UEs with NTN |  | ZTE |
|  | 5.1X Cell Re-selection for RedCap UEs with NTN | Refer to Clause 4.2X |  |
| 3 | 5.5X Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UEs with NTN |  | Nokia |
|  | 5.7X Random access based Small Data Transmissions (RA-SDT) for RedCap |  |  |
| 4 | 6.1X Handover for RedCap UEs with NTN | Include handover，conditional handover and satellite switching with re-synchronization | HW |
| 5 | 6.2X RRC Connection Mobility Control for RedCap UEs with NTN |  | Samsung |
| 6 | 7.1X UE transmit timing for RedCap UEs with NTN | Refer to Clause 7.1C | MTK |
|  | 7.2X UE timer accuracy for RedCap UEs with NTN | Refer to Clause 7.2C |  |
|  | 7.3X Timing advance for RedCap UEs with NTN | Refer to Clause 7.3C |  |
| 7 | 8.1X Radio Link Monitoring for RedCap UEs with NTN |  | QC |
| 8 | 8.5X Link Recovery Procedures for RedCap UEs with NTN |  | OPPO |
| 9 | 8.6X Active BWP switch delay for RedCap UEs with NTN | Refer to Clause 8.6C | VIVO |
|  | 8.10X Active TCI state switching delay for RedCap UEs with NTN | Refer to Clause 8.10C |  |
|  | 8.13X UE-specific CBW change for RedCap UEs with NTN | Refer to Clause 8.13C |  |
|  | 8.14X Pathloss reference signal switching delay for RedCap UEs with NTN | Refer to Clause 8.14C |  |
| 10 | 9.1X General measurement requirement for RedCap UEs with NTN |  | Ericsson |
| 11 | 9.2X NR intra-frequency measurements for RedCap UEs with NTN |  | Ericsson |
| 12 | 9.3X NR inter-frequency measurements for RedCap UEs with NTN |  | CMCC |
| 13 | 9.5X L1-RSRP measurements for Reporting for RedCap UEs with NTN |  | Xiaomi |
| 14 | 9.9X NR measurements for positioning for RedCap UEs with NTN |  | Apple |