3GPP TSG-RAN WG4 Meeting #114bis				                   R4-2504940

Wuhan, China, 7th – 11th April, 2025

Title:	Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18

Response to:	R2-2501392 / R4-2503115

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Ada Wang

ada.wang@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN4 would like to thank RAN2 for the LS on granularity definition of pdcch-RACH-AffectedBandsList-r18. RAN4 discussed the question from RAN2 and reached the following agreement:

- "Understanding 2" in the LS (R2-2501392 / R4-2503115) is what RAN4 intended. And the band in which the PDCCH order is received has no impact on whether and on which band the interruption may occur.

In addition, RAN4 would like to provide one example to clarify the intention behind this capability. For each target band on which the PDCCH order triggered RACH is transmitted, UE indicates on which of the bands with active serving cell(s) UE may cause interruption during the PRACH transmission.

For example, if a UE supports a PDCCH order RACH to an LTM candidate cell in bands A, B, C, D, E, and F, and supports CA with the band combinations of A+B+C and A+B+D+E, the UE may report this capability for the band combination of A+B+C as shown in the table below.

| Target LTM band for PDCCH order RACH | Capability for the band combination of A+B+C (x, y, z) = Interruptions on the active serving cell in bands A, B, and C due to the PDCCH order RACH |
| --- | --- |
| A | (No, No, No) |
| B | (No, No, No) |
| C | (No, No, No) |
| D | (No, No, Yes) |
| E | (No, No, Yes) |

RAN4 would like to kindly note that the above table is only for example, the signalling design on how to capture this principle is up to RAN2.

# 2	Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take into account above information.

# 3	Dates of next TSG RAN WG 4 meetings

TSG RAN WG4 Meeting #115		              19 - 23 May 2025 			Malta

TSG RAN WG4 Meeting #116		              25 - 29 August 2025			Bengaluru, India