# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504954

# Wuhan, China, 7th – 11st April, 2025

Agenda Item:	7.21.1

Source: 	OPPO

Title: 	TP on RAN4 aspects for TR 38.744

Document for:	Approval

- Introduction

In this contribution, we provide the text proposal on the framework of RAN4 part in TR 38.744 to capture the RAN4 agreements.

- Text proposal

< Start of text proposal >

6.2	Interoperability, testability, and RRM requirements

6.2.1 RRM requirements impact for measurement prediction

6.2.1.1 General

RAN4 has studied the impacts of RRM requirements for scenarios 2, 3, 4.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

For scenarios 2, 3 and 4, the following 3 sub-use cases discussed:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

RAN4 studied the impacts on RRM requirements for the following case:

  - L3 cell level RSRP (Point C) prediction

For Rel-19 study, RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

6.2.1.2 Potential impact to RRM requirements

RAN4 to consider both absolute RSRP and relative RSRP accuracy as the candidate metrics for RRM prediction use cases.

Use the following definition for absolute accuracy:

  - Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP

Use the following definition for relative accuracy:

  - For intra-frequency L3-RSRP,

    - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

    - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

      - FFS if this definition will be needed.

  - For inter-frequency L3-RSRP (scenario 3),

    - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

    - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].

      - Depending upon the agreement on ground truth definition, we may revisit relative RSRP accuracy definition.

      - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples.

  - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

For the ground truth definition in FR2, take Option 1-1-FR2 as baseline, and further discuss any additional aspects that need to be considered.

  - Option 1-1-FR2: The ground truth of L3 RSRP is the reported L3 RSRP measurement result under sufficient high SNR in FR2.

For the ground truth definition in FR1, down-select from the following alternatives:

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions.

  - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects:

  - the side condition of frequency prediction (e.g., EPRE difference)

  - the impact of correlation coefficient between measured and predicted frequency layers

  - impact of cluster approach, e.g.,

    - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

    - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

For AI based RRM measurement prediction, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

6.2.2 RRM requirements impact for measurement event prediction

6.2.2.1 General

6.2.2.2 Potential impact to RRM requirements

6.2.3 Testability and Interoperability

6.2.3.1 Testing goal

As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

6.2.4.2 Testing setup

For the number of cells configured in the testing:

  - RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction.

  - FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

<End of text proposal>