# 3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504967

# Wuhan , CN, 7th – 11st Apr, 2025

Agenda Item:	7.29.5

Source: 	Qualcomm Incorporated, CATT

Title: 	Ad-hoc minutes on RRM requirements for NR_NTN_Ph3

Document for:	Information

# 0 Introduction

This document is the Ad-hoc minutes for NR_NTN_Ph3-Core WI with the following topics covered.

[114-bis][221] NR_NTN_Ph3_Part1:

Sub-topic 2-1: Issue 2-1-1/2-1-2/2-1-3

Sub-topic 2-2: Issue 2-2-1

Sub-topic 1-2: Issue 1-2-2/1-2-1-2

1 Topic #2: Reduction in the number of UE Rx branches

    - Sub-topic 2-1: RRC_IDLE/RRC_INACTIVE mode mobility

Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

- Background

The related requirements defined in TS 38.133 are copied as information:

Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh DRX cycle length [s]; Tdetect,NR_Intra_enh [s] (number of DRX cycles); Tmeasure,NR_Intra_enh [s] (number of DRX cycles); Tevaluate,NR_Intra_enh [s] (number of DRX cycles). 0.32; 2.56 x M2 (8 x M2)Note 1; 0.32 x M3 (1 x M3) Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; 5.12 (8); 0.64 (1); 1.92 (3). 1.28; 8.96 (7); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). NOTE 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5.

- Proposals

For how to define the enhanced cell reselection measurement requirements，

  - Proposal 1(Xiaomi, CMCC, CATT, ZTE, Samsung, Nokia): Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional for (e)RedCap NTN UE, and RAN4 to reuse the existing requirements.

    - Proposal 1a (CMCC, CATT, Nokia): RAN4 only to define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX cycle, the requirement defined for normal UEs with FR1-NTN can be reused.

    - Proposal 1b (ZTE): The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

  - Proposal 2 (HW, vivo, QC): To define enhanced cell reselection requirements for RedCap UE, and to re-use the requirements for normal UE except that some extensions for Tdetect / Tevaluate/ Tmeasure.

    - Proposal 2a (HW): Tdetect is extended by [2] samples.

    - Proposal 2b (QC): RAN4 to increase Tdetect so that the value is close to half of Tdetect in the normal requirements.

    - Proposal 2c (vivo): By extending Tmeasure samples from the existing enhanced requirement defined for NTN,

      - Extend Tmeasure from 1 sample to [2] samples, the Tevaluate and Tdetect would also be adjusted to [6] samples and [11] samples accordingly

For whether to differentiate 1Rx and 2Rx (e)RedCap NTN UE,

  - Proposal 1(Xiaomi, CMCC, CATT, ZTE, Samsung, HW, vivo, QC): The requirements don’t differentiate 1Rx and 2Rx.

  - Proposal 2 (Nokia): For 1Rx RedCap UE relaxations, e.g. related to cell reselection margin, can be considered.

- Recommended WF

  - To be discussed.

For how to define the enhanced cell reselection measurement requirements，

    - Option 1:

      - Enhanced RRM requirements for measurements in IDLE/INACTIVE modes are kept optional for (e)RedCap NTN UE, and RAN4 to reuse the existing requirements.

      - RAN4 only to define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX cycle.

    - Option 2: RAN4 to reuse the requirements for normal UE except that some extensions for Tdetect / Tevaluate/ Tmeasure.

      - Alt A: Tdetect is extended by [2] samples.

      - Alt B: Increase Tdetect so that the value is close to half of Tdetect in the normal requirements.

      - Alt C: By extending Tmeasure samples from the existing enhanced requirement defined for NTN. Extend Tmeasure from 1 sample to [2] samples, the Tevaluate and Tdetect would also be adjusted to [6] samples and [11] samples accordingly

For whether to differentiate 1Rx and 2Rx (e)RedCap NTN UE,

    - Option 1: The requirements don’t differentiate 1Rx and 2Rx.

    - Option 2: Some relaxations for 1Rx RedCap UE can be considered.

Agreement:

For the enhanced cell reselection measurement requirements，

- Enhanced RRM requirements for measurements in IDLE/INACTIVE modes are kept optional for (e)RedCap NTN UE, and RAN4 to reuse the existing requirements.

- RAN4 only to define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX cycle.

- Reuse the requirements for normal UE except for Tdetect being extended by [2] DRX cycles.

- The requirements don’t differentiate 1Rx and 2Rx.

Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

- Proposals

  - Proposal 1 (CMCC, CATT, HW, Ericsson, Nokia): For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN,

    - For 1 Rx and 2 Rx RedCap UEs not configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for normal UEs with FR1-NTN (cl. 4.2C.2.11).

    - For 1 Rx and 2 Rx RedCap UEs configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for (e)RedCap UE with TN (cl. 4.2B.2.5).

Note: The eDRX related impact and above principle also apply to other agreed cell re-selection measurement requirements in the previous meeting.

  - Proposal 2 (CATT): RAN4 to reuse the existing margin of RSRP/RSRQ defined for normal UEs with NTN for 1 Rx and 2 Rx (e)RedCap UE.

    - The reselection criteria are met by a margin of at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

  - Proposal 3 (Samsung): For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands, for both 2Rx UE and 1Rx UE, reuse the existing requirements for measurements for inter-RAT E-UTRAN cells with TN carrier.

  - Proposal 4 (ZTE):

    - The cell re-selection inter-frequency and inter-RAT measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs considering TN to NTN and NTN to TN.

    - RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN.

- Recommended WF

  - Recommend to agree on:

    - For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN,

      - For 1 Rx and 2 Rx RedCap UEs not configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for normal UEs with FR1-NTN (cl. 4.2C.2.11).

      - For 1 Rx and 2 Rx RedCap UEs configured with eDRX_IDLE cycle, RAN4 to reuse the existing requirements defined for (e)RedCap UE with TN (cl. 4.2B.2.5).

    - Note: The eDRX related impact and above principle also apply to other agreed cell re-selection measurement requirements in the previous meeting.

Agreement:

- For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN,

  - For 1 Rx and 2 Rx RedCap UEs not configured with eDRX cycle, RAN4 to reuse the existing requirements defined for normal UEs with FR1-NTN (cl. 4.2C.2.11).

  - For 1 Rx and 2 Rx RedCap UEs configured with eDRX cycle, RAN4 to reuse the existing requirements (Table 4.2B.2.5-2) defined for (e)RedCap UE with TN (cl. 4.2B.2.5) as a baseline and adopt NTN principles on how measurements are triggered/etc when implementing the requirement in the specification. Note that only eDRX cycles up to 10.24 sec are applicable for (e)RedCap NR-NTN UEs.

- Note: The eDRX related impact and above principle also apply to other agreed cell re-selection measurement requirements in the previous meeting.

Issue 2-1-3: SDT requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

- Background

Regarding how to define SDT requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN, RAN4 has reached the following agreements in RAN4#113 meeting:

RAN4#113: - Issue 2-1-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN < Agreement >: - For 1Rx (e)RedCap UEs with FR1-NTN bands, - For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE.

- Proposals

  - Proposal 1 (vivo, CATT): For SDT requirements on Redcap NTN in RRC_INACTIVE state, RAN4 to revert the agreement reached in RAN4#113 and modify it as:

    - For SDT requirements on RedCap NTN (including CG-SDT and RA-SDT), the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

  - Proposal 2 (Nokia): For SDT requirements for 1Rx (e )RedCap UE in FR1-NTN, RAN4 to keep the +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 as used for 1Rx (e)RedCap UE in TN.

  - Proposal 3 (CATT): Except for the TA validation and corresponding offset of threshold for CG-SDT and RA-SDT requirements, the legacy requirements defined for (e)RedCap UE with TN can be reused for (e)RedCap UE with FR1-NTN with the following adaptation:

    - For NTA in CG-SDT, it refers to the value specified in clause 7.1C.2.

    - For the SDT requirements based on 2-step RA and 4-step RA, they refer to clauses 6.2C.2.3 and 6.2C.2.2 respectively.

- Recommended WF

Proposal 3 does not conflict with Proposal 1/2. Please firstly check Proposal 3 and further discuss Proposal 1/2.

  - Recommend to agree on:

    - For RA-SDT requirements, except for the corresponding offset of threshold, the legacy requirements defined for (e)RedCap UE with TN can be reused for (e)RedCap UE with FR1-NTN with the following adaptation:

      - For the SDT requirements based on 2-step RA and 4-step RA, RAN4 to refer to clauses 6.2C.2.3 and 6.2C.2.2 respectively.

  - To be discussed.

For SDT requirements on Redcap NTN in RRC_INACTIVE state, keep or revert the agreement on the offset reached in RAN4#113,

    - Option 1: RAN4 to revert the agreement reached in RAN4#113 and modify it as:

      - For SDT requirements on RedCap NTN (including CG-SDT and RA-SDT), the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

    - Option 2: RAN4 to keep the +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 as used for 1Rx (e)RedCap UE in TN.

Agreement:

- RAN4 to keep the +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 as used for 1Rx (e)RedCap UE in TN.

- For RA-SDT requirements, the legacy requirements defined for (e)RedCap UE with TN can be reused for (e)RedCap UE with FR1-NTN with the following adaptation:

  - For the SDT requirements based on 2-step RA and 4-step RA, RAN4 to refer to clauses 6.2C.2.3 and 6.2C.2.2 respectively.

    - Sub-topic 2-2 L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Issue 2-2-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Proposals

  - Proposal 1(Xiaomi, CMCC, CATT, Sumsung, ZTE, HW, Nokia, QC):

    - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

    - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

      - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

- Recommended WF

No different views. Check Proposal 1 is agreeable or not.

  - Recommend to agree on:

    - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

    - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

      - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

Agreement:

    - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

    - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

      - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

2 Topic #1: General consideration on RRM requirements

    - Sub-topic 1-2 Features to be considered for (e)RedCap UE with FR1-NTN

Issue 1-2-2: NCD-SSB

- Background

The agreement in RAN4#112 meeting:

#### RAN4#112: #### Issue 6-6-1: The impact of NCD-SSB for (e)RedCap UEs with FR1-NTN <Agreement> - For (e)RedCap UEs with FR1-NTN, there is no need to define NCD-SSB specific measurement requirements.

- Proposals

  - For CR drafting,

    - Proposal 1 (Sumsung, CATT, Ericsson): Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

      - Proposal 1a (CATT): The wording of the clarification in clause 3 in TS 38.133 can be: The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

    - Proposal 2 (HW): Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and no need to add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

    - Proposal 3 (CMCC): No strong view about whether to add additional requirement applicability about CD-SSB/NCD-SSB.

    - Proposal 4 (Nokia): RAN4 to specify NCD-SSB support as mandatory for 1Rx and 2Rx (e)RedCap UEs in NTN.

- Recommended WF

  - Recommend to agree on:

    - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB.

  - To be discussed.

    - For whether to add additional requirement applicability about CD-SSB/NCD-SSB,

      - Option 1: Add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

- The wording of the clarification in clause 3 in TS 38.133 can be: The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

      - Option 2: No need to add additional requirement applicability about CD-SSB/NCD-SSB.

Agreement:

      - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB.

      - No need to add additional requirement applicability about CD-SSB/NCD-SSB.

      - Test cases are defined based on CD-SSB.

Issue 1-2-1: SMTCs and 'in parallel' related

Issue 1-2-1-2: The number of cells/SSBs/supported satellites

- Proposals

  - Proposal 1 (Xaiomi, CMCC, CATT, ZTE, HW, Ericsson, Nokia, QC ):

    - For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for (e)RedCap UEs for both intra-frequency and inter-frequency measurement, and it will be optional with capability signalling.

  - Proposal 2 (Xaiomi, CMCC, CATT, ZTE, HW, Ericsson, Nokia):

    - For the supported number of cells/SSBs, the following legacy requirement also can be reused for (e)RedCap UEs:

      - For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

- 8 identified cells, and

- 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

- 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

- In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

- Recommended WF

No different views. Check Proposal 1 and Proposal 2 are agreeable or not.

  - Recommend to agree on:

    - For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for (e)RedCap UEs for both intra-frequency and inter-frequency measurement, and it will be optional with capability signalling.

    - For the supported number of cells/SSBs, the following legacy requirement also can be reused for (e)RedCap UEs:

      - For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

- 8 identified cells, and

- 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

- 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

- In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

Agreement:

    - For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for (e)RedCap UEs for both intra-frequency and inter-frequency measurement, and it will be optional with capability signalling.

    - For the supported number of cells/SSBs, the following legacy requirement also can be reused for (e)RedCap UEs:

      - For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

- 8 identified cells, and

- 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

- 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

- In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.