3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2505171

Wuhan Meeting, Apr. 7th – Apr. 11th, 2025

Title:	TP to TR 38.719-02-01 CA_n48-n66

Source:	Nokia, Verizon

Agenda item:	6.3.3

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.719-02-01 to add CA_n48B-n66A with CA_n48B-n66A in uplink BCS 4&5. Note that this TP updates the existing entry in the TR since UL configuration CA_n48B-n66A is added.

Note that only CA_n2A-n77C as UL configuration for BCS 4 and 5 are new. However, in the TP the currently supported UL configurations supported are included for completeness.

************************************* Start of TP*****************************************

## 5.7		CA_n48-n66

### 5.7.1	Common for 1 band UL and 2 bands UL CA

#### 5.7.1.1	Operating bands for CA

Table 5.7.1.1-1:  CA band combination of band n48+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 5.7.1.2	Channel bandwidths per operating band for CA

Table 5.7.1.2-1: Supported bandwidths per CA band combination of band n48+n66

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48A-n66A | CA_n48A-n66A | n48 | See n48 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| CA_n48B-n66A | CA_n48B CA_n48A-n66A CA_n48B-n66A | n48 | CA_n48B_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48B-n66(2A) | CA_n48B CA_n48A-n66A CA_n48B-n66A | n48 | CA_n48B_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

#### 5.7.1.3	UE co-existence studies for 1 band UL

Table 5.7.1.3-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n48-n66.

Table 5.7.1.3-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n66 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| n48 | fLow | fHigh | 1780 | 3560 | 5340 | 7120 | 8900 |  |
| DL1 | 3550 | 3700 | N/A | D |  |  |  | UL Harmonic |
| DL2 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 10650 | 11100 |  |  | N/A |  | N/A |  |
| DL4 | 14200 | 14800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 17750 | 18500 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is a direct hit from the band n66 2nd harmonic to the band n48 |  |  |  |  |  |
| UL/DL |  | n48 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n66 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 2110 | 2200 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 4220 | 4400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 6330 | 6600 |  |  | N/A |  | N/A |  |
| DL4 | 8440 | 8800 |  | N/A | N/A |  | N/A |  |
| DL5 | 10550 | 11000 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing |  |  |  |  |  |

Based on Table 5.7.1.3-1:

For the uplink n66 2nd harmonic falling to the band n77, the MSD value has been defined in Table 7.3A.4-1 of 38.101-1. In fact, the band n66 channel bandwidth for BCS 4/5 is the same as BCS 2. Therefore, there is no newer requirement needed.

Table 5.7.1.3 2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.7.1.3-2: Cross-band isolation analysis

| Bands | n48 |  | n66 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 3550 | 3700 | 1710 | 1780 |
| fDL (MHz) | 3550 | 3700 | 2110 | 2200 |
| CBW (MHz) | 5 | 100 | 5 | 45 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 3450 | 3800 | 1665 | 1825 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 3350 | 3900 | 1620 | 1870 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 3250 | 4000 | 1575 | 1915 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 3150 | 4100 | 1530 | 1960 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 3050 | 4200 | 1485 | 2005 |
| Analysis | There is no overlap of the band n48 ACLR range with the band n66 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n66 so no cross-band MSD is needed. |  | There is no overlap of the band n66 ACLR range with the band n48 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n48 so no cross-band MSD is needed. |  |

Based on Table 5.7.1.3-2:

There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

#### 5.7.1.4	∆TIB,c and ∆RIB,c values

For CA_n48-n66, the ∆TIB,c and ∆RIB,c values have already been specified.

#### 5.7.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

#### 5.7.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

### 5.7.2	Specific for 2 bands UL CA

#### 5.7.2.1	Maximum output power for inter-band CA

Table 5.4.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n48B-n66A | 23 | +2/-3 |

#### 5.7.2.2	UE co-existence studies for 2 bands UL

Table 5.7.2.2-1 lists Band n48 + Band n66 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.7.2.2-1: Band n48 and Band n66 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1770 - 1990 |  | 5260 - 5480 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5320 - 5690 |  | 10 - 280 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8810 - 9180 |  | 6970 - 7260 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8870 - 9390 |  | 1430 - 1790 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3540 - 3980 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 12360 - 12880 |  | 8680 - 9040 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10520 - 10960 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3140 - 3570 |  | 12420 - 13090 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1760 - 2270 |  | 7090 - 7680 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10390 - 10820 |  | 15910 - 16580 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12230 - 12740 |  | 14070 - 14660 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The requirements are already found in the specification.

Table 5.7.2.2-2 lists Band n48 + Band n66 2 bands UL CA(3CC) triple beat IMD analysis for Band n48- Band n66

Table 5.7.2.2-2: Band n48 and Band n66 for 3CC UL IMD products

| Band / CA1 | n66 |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1710-1780 |  | 3550-3700 |  |
| fDL | 2110-2200 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -100 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1610-1710 |  | 1780-1880 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### Based on Table 5.7.2.2-2:

The bands are not of the same or adjacent band groups, so Note 1 applies.

#### 5.7.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

************************************* End of TP******************************************