3GPP TSG-RAN WG4 Meeting #114bis	R4-2505220

Wuhan, China, April 7 – 11, 2025

Title: 	WF on Tx switching for TEI19

Agenda Item:	5.3.1

Source: 	MeidaTek inc.

Document for:	Approval

Issue 1: Introduction of Tx switching enhancements in TEL-19.

Background

The following scenarios of further Tx switching enhancement for 2 configured UL bands were discussed in RAN4#114b meeting:

- Scenario 1. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to 'dualUL’

  - Case 1-1: Two antenna ports on Band A and one antenna port on Band B

  - Case 1-2: One antenna port on Band A and two antenna ports on Band B

- Scenario 1a. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’

  - Case 2-1: Three antenna ports on Band A and zero antenna port on Band B

  - Case 2-2: Zero antenna port on Band A and three antenna ports on Band B

- Scenario 2. Switching between the following 2 cases for 4Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’

  - Case 3-1: Four antenna ports on Band A and zero antenna port on Band B

  - Case 3-2: Zero antenna port on Band A and Four antenna ports on Band B

Agreement:

- Support scenario 1 in Rel-19 TEI

- Focus on scenario 1 in RAN4 #115 meeting:

  - Inform RAN1 that RAN4 is considering whether to support scenario 1a and scenario 2 in Rel-19 TEI and check the possibility to support scenario 1a/2 from RAN1 perspective in addition to scenario 1.

    - FFS whether need to further discuss scenario 1a and scenario 2 in RAN4 which is pending on RAN1 decision on supporting above scenarios in May meeting.

Issue 2: Applicable duplexing mode for the 2 configured UL bands (focus on scenario 1 in RAN4#115 meeting)

Wayforward:

- FFS any down selection among {FDD+TDD, FDD+FDD, TDD+FDD} is needed.

  - Companies are encouraged to provide the justifications, such as commercial demand or problem identified in scheduling.

Issue 3: Switch period (focus on scenario 1 in RAN4#115 meeting)

Wayforward:

- FFS whether and which existing values can be re-used, or any new values need be introduced.

  - The final conclusions will be captured in the Rel-19 RAN4 feature list to RAN1 and RAN2

Issue 4: UE band combo report (focus on scenario 1 in RAN4#115 meeting)

Wayforward:

- FFS whether and which existing UE capabilities can be re-used, or any new UE capabilities need be introduced.

  - The final conclusions will be captured in the Rel-19 feature list to RAN1 and RAN2

# Annex: Online agreements on Thursday

Issue 3-1-1: Tx switching

Agreement:

- Discuss and down-select the following for Rel-19 TEI work

  - Scenario 1: UL Tx switching between 2UL bands for 3Tx UE for the cases as following

    - Scenario 1a: Between 2Tx on band A + 1Tx on band B and 1Tx on band A + 2Tx on band B for FDD+TDD band combinations.

    - Scenario 1b: Between 3Tx on band A + 0Tx on band B and 0Tx on band A + 3Tx on band B for FDD+TDD band combinations.

  - Scenario 2: UL Tx switching between 2UL bands for 4Tx UE for the cases as following

    - Between 4Tx on band A + 0Tx on band B and 0Tx on band A + 4Tx on band B for TDD+TDD band combinations.

  - All the scenarios will have RAN1 and RAN2 impacts