3GPP TSG-RAN WG4 Meeting #114bis	R4-2505222

Wuhan, China, 7th – 11st April, 2025

Title: 	WF on PHS protection update

Agenda Item:	5.1.1

Source: 	KDDI

Document for:	Approval

- Introduction

The moderator copies agreements and approved way forwards during the online session into this contribution in agenda 5.1.1. On top of that, we capture additional way forwards into this contribution.

Topic #2: PHS protection update

Sub-topic 2-1: PUCCH transmissions for meeting limits without A-MPR

< Issue 2-1-1: PUCCH transmissions for meeting limits without A-MPR >

Agreement:

- Encourage companies to evaluate the feasibility of the following:

  - Split the agreed Region A2 into two new regions

Sub-topic 2-2: NS_05 PC2

< Issue 2-2-1: A-MPR region and value for NS_05 PC2 >

Way forward:

- Continue further discussion based on more inputs in next meeting.

- Encourage companies to submit simulation results in next meeting.

Sub-topic 2-3: NS_05U PC3

< Issue 2-3-1: A-MPR region and value for NS_05U PC3 >

Agreement:

- Taking “Avg Rdup” approach to decide MPR values as captured in slide 6,8.10,12 from the WF [1].

Way forward:

- A-MPR region definition and values for NS_05U PC3 are as follows.

Table 1: A-MPR regions for NS_05U (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  | Region A3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBend |
| 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 2.52MHz/12/scs | ≥ 0 | ≤ 2 .16MHz/12/scs | ≤ 1.8MHz/12/scs | ≤ 1.35*RBstart + 2.52MHz/12/scs ≥ 2.34MHz/12/scs | ≥ 8.1MHz/12/scs |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ 1.2*RBstart + 3.6MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs | ≤ 1.2*RBstart + 3.6MHz/12/scs ≥ 2.88MHz/12/scs | ≥ 12.24MHz/12/scs |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 1.45*RBstart + 2.16MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs | ≤ 1.45*RBstart + 2.16MHz/12/scs ≥ 2.88MHz/12/scs | ≥ 12.24MHz/12/scs |
| 20 | < 1945.7 | ≥ 1.5*RBstart + 4.14MHz/12/scs | ≥ 0 | ≤ 5.76MHz/12/scs ≤ RBstart*1.5 + 4.14MHz/12/scs | ≤ 4.68MHz/12/scs | ≤ 1.5*RBstart + 4.14MHz/12/scs ≥ 2.88MHz/12/scs | ≥ 16.2MHz/12/scs |

Table 2: A-MPR values for NS_05U (Power Class 3, 10MHz, Fc<1930.7MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3 | ≤ 2.5 | ≤ 2.5 |
|  | QPSK | ≤ 4 | ≤ 3.0 | ≤ 2.5 |
|  | 16 QAM | ≤ 4 | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 3.5 | ≤ 3.5 |
|  | 256 QAM | ≤ 5 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 4.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 6.0 |

Table 3: A-MPR values for NS_05U (Power Class 3, 15MHz, 1927.5≤fc<1932.5)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 2.5 | ≤ 2.5 |
|  | QPSK | ≤ 4.5 | ≤ 3.5 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 4.0 | ≤ 4.0 |
|  | 256 QAM | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 5.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 5.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 5.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.0 | ≤ 6.5 |

Table 4: A-MPR values for NS_05U (Power Class 3, 15MHz, 1932.5≤Fc<1938.2MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 |
|  | QPSK | ≤ 3.0 | ≤ 3.0 | ≤ 2.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3.5 | ≤ 3.0 |
|  | 64 QAM | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 |
|  | 256 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 5.5 | ≤ 6.5 |

Table 5: A-MPR values for NS_05U (Power Class 3, 20MHz, Fc<1945.7MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 3.0 | ≤ 3.0 |
|  | QPSK | ≤ 4.5 | ≤ 3.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 5.0 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.0 | ≤ 4.0 | ≤ 4.0 |
|  | 256 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 5.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 6.0 | ≤ 6.5 |

Sub-topic 2-4: NS_05U PC2

< Issue 2-4-1: A-MPR region and value for NS_05U PC2 >

Way forward:

- Continue further discussion based on more inputs in next meeting.

- Encourage companies to submit simulation results in next meeting.

Sub-topic 2-5: modifiedMPR-Behaviour

< Issue 2-5-1: modifiedMPR-Behaviour >

Way forward:

- Continue further discussion based on more inputs in next meeting.

Sub-topic 2-6: NS_05 PC3

< Issue 2-6-1: A-MPR region and value for NS_05 PC3 >

Agreement:

- Update the last meeting agreements for NS_05 PC3 as follows.

Table 1: A-MPR regions for NS_05 (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs |
| 15 | 1927.5 ≤ fc < 1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs |
| 20 | < 1945.7 | ≥ RBstart*1.5 + 4.14MHz/12/scs; | ≥ 0 | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | ≤ 4.68MHz/12/scs |

Reference

[1] R4-114bis_AMPR region and value for NS_05U,NS_05 PC3_v3.1_after ad-hoc.pptx, KDDI, RAN4#114bis