3GPP RAN WG4 Meeting #115																	R4-2505412

Malta, Malta, 19th – 23rd May 2025

Agenda item:	5.2

Source:	Apple

Title:	(IoT_NTN_FDD_LS_band-Core) Power back-off simulations for a UE operating in 1610-1626.5 MHz frequency range

WI/SI:	IoT_NTN_FDD_LS_band-Core

Release:	Rel-18

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. After the RAN#98 meeting new Rel-18 spectrum WIs were agreed with the objective of adding support for new bands where the UL is on the L-band and the DL is on the S-band [2] [3], which finally was specified as the NTN bands 254/n254.

Upon completion of the NTN IOT band 254 specification work some concerns were raised on potentially high AMPR to comply with the existing ETSI regulations, and thus some AMPR values were left as TBD in TS 36.102. In fact, uncompleted work for band 254 also impacts another spectrum WI [4], which will add the IOT TDD band operating in the same frequency range as the IOT band 254. Thus, in this discussion paper we provide our further technical input on required power back-off values aiming to complete band 254 specifications.

# 2	Power back-off for band 254

Table 2-1 below presents a summary of required A-MPR for different NB-IOT configurations, such as 12, 6, and 3 tone allocations under the existing FCC and ETSI regulations. The full RB 12-tone allocation can be viewed as a reference point requiring 0dB A-MPR, in addition to which we also present results for the 6- and 3-tone allocations for the edge allocations as a worst-case scenario

So as can be seen from the table, no AMPR is needed if a NB-IOT UE operates under the FCC rules. As for the ETSI regulatory domain, some A-MPR is needed for 6- and 3-tone allocations, whereupon the second ETSI sub-range requires slightly higher A-MPR because of more stringent in-band emission mask. It is also worth noting that for the 1-tone allocation we apply the fixed 2dB A-MPR, which is due to the existing -15dBW/4kHz (i.e. 21.3dBm/15kHz) PSD threshold that will always limit the maximum output power.

Table 2-1: Summary of A-MPR values

| Number of tones | FCC 1610-1626.5 MHz | ETSI |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | 1610-1618.25 MHz | 1618.25-1626.5 MHz |  |
| 12 | 0 | 0 | 0 | Full RB 12-tone allocation |
| 6 | 0 | 0 | 1 | 0-5 and 6-11 tones |
| 3 | 0 | 0.5 | 1.5 | 0-2 and 9-11 tones |
| 1 | 0 | 2 | 2 | 2dB A-MPR is applied to ETSI because of the existing -15dBW/4kHz limit, i.e. 21.3dBm/15kHz |

And for the sake of completeness, Figure 2-1 below also presents the corresponding spectrum plots for FCC and ETSI with the corresponding in-band emission masks.

Figure 2-1: 12-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure 2-2: 6-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure 2-3: 3-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Based on the presented power back-off results we suggest defining the corresponding A-MPR values for the NB-IOT operation in band 254. As for the 1-tone allocations, 2dB A-MPR can be revised if 3GPP concludes that -15dBW/4kHz does not have to be always applied, as discussed in [5].

Proposal 1:	Adopt A-MPR values for band 254 based on results presented Table 2-1.

Proposal 2:	1-tone A-MPR can be revised further if 3GPP concludes that we do not need to follow (always) the existing -15dBW/4kHz PSD threshold.

# 3	Conclusions

In this discussion paper we presented our power back-off simulation results for the NTN NB-IOT operating in the band 254 frequency range. According to our results, some A-MPR is needed for a case when a UE operates under ETSI regulations and is scheduled with fewer than 12 tones, whereupon ETSI sub-range 2 requires slightly higher A-MPR because of a more stringent in-band emission mask. As for the 1-tone allocation, fixed 2dB A-MPR is applied due to the existing -15dBW/4kHz PSD limit, subject for further discussions in 3GPP.

Proposal 1:	Adopt A-MPR values for band 254 based on results presented Table 2-1.

Proposal 2:	1-tone A-MPR can be revised further if 3GPP concludes that we do not need to follow (always) the existing -15dBW/4kHz PSD threshold.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-230792, "New WID: FDD LTE band (L+S band) for IoT NTN operation", MediaTek

RP-242415, "New WID for introduction of IoT-NTN TDD mode", Iridium Satellite LLC

R4-2505407, "Clarification for the PSD limits for a UE operating in 1610-1626.5 MHz frequency range", Apple, Globalstar