3GPP TSG-RAN4 Meeting #115	R4-2505512

# Malta 19th – 23rd May, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for low band carrier aggregation via switching

Agenda item:	7.7.4

Document for:	Discussion

# Introduction

3GPP introduced a new work item on low band carrier aggregation via switching in RAN #106 [1]. We discuss some relevant RRM requirements for this topic.

# Past Agreements

RAN4 RRM session discussed this topic during the past meeting. The agreements and open issues are shown below:

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4. - Switch delay requirements Agreement: - The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session. RAN4 RRM not to define additional switch delay in RRM. - Synchronization requirements for FDD and synchronization accuracy at BS antenna connectors or radiated interface boundaries Agreement: - No need to define new cell phase synchronization requirement in RRM spec for R19 LB CA via switching. - SDL SCC related RRM requirement - activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - de-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used. - other SDL SCell requirements Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA Agreement: - Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement - FFS on the use case for MG application. - Potential impacts to the PCell related requirement - FFS - FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us

# Discussion

According to [2], low band spectrum carries significant traffic volume in urban indoor and rural areas. However, SDL remain under-utilized since they cannot be paired with UL. That’s why, [2] proposed to introduce a network configured switching pattern in low band carrier aggregation via switching

Figure 1: RRC configured switching pattern in low band carrier aggregation via switching [2]

Figure 1 shows how network could configure this switching pattern to enable this low band carrier aggregation. This switching pattern would instruct UE to alternate between FDD DL+UL band and SDL band in a periodic manner. We discuss different RRM requirements of SCells and PCells in this scenario.

## Activation of Switching Pattern

Figure 2: Transmission of CQI report and application of switching pattern during SCell activation based on latest RAN4 agreement

RAN4 made the following agreement during the last meeting:

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4.

Above agreement shows that switching pattern for SDL SCell only applies for the active SCell, i.e., after UE reports valid CQI for SCell activation. RAN4 has allowed reduction of SCell activation timeline by not applying switching during activation procedure and SSBs of SCells may not always overlap with the ON duration of SCells.

Legacy requirements allow network to trigger CSI report via DCI during SCell activation. The combination of legacy requirements and recent RAN4 agreement mean that UE’s first application of switching pattern will be dynamically triggered via DCI.

Unless periodic CSI report is not configured, network can send DCI via PCell at any time to configure CSI-RS in SCell and CSI report in PCell after UE has measured enough SCell SSB samples. The time separation between CQI report and applicability of switching pattern can be as small as zero. This goes against the spirit of this Rel-19 work item that only focuses on RRC configured semi-static switching pattern. Figure 2 displays an example of this scenario.

Observation 1: RAN4 has agreed that switching pattern for SDL SCell is not applied when the SCell is in activation procedure. Switching pattern for SDL SCell applies for the active SCell (i.e. after UE reports valid CQI for SCell activation).

Observation 2: Legacy requirements allow network to trigger CSI report for SCell activation via DCI.

Observation 3: Combination of legacy requirements and latest RAN4 agreements mean that the time separation between DCI triggered CQI reporting and the activation of switching pattern can be as small as zero. This goes against the scope of the work item that only focuses on semi-static switching pattern.

This potential zero-time separation between CQI report and activation of switching pattern can be quite challenging for the UE. UE needs a minimum amount of time separation between sending CQI report and applying switching pattern.

Note that, if RAN4 had allowed switching pattern to be applicable during SCell activation procedure, then switching pattern would have become active via MAC-CE or RRC configured SCell activation command. UE gets 3ms or 10ms after sending HARQ A/N with MAC-CE or RRC based activation command respectively to start SCell activation procedure. That means, after sending HARQ A/N corresponding to the SCell activation command, UE would have gotten 3ms or 10ms to apply switching pattern in MAC-CE or RRC based activation. Hence, it is reasonable to allow a similar amount of minimum time separation between valid CQI report and activation of switching pattern.

Zero-time separation between CQI report and switching pattern activation is not a major issue with periodic or semi-persistent CQI reports. UE knows the configuration of periodic or semi-persistent switching pattern beforehand and can be prepared to activate the switching pattern even if it occurs immediately after the CSI report. However, it is better to introduce a same duration of minimum time separation between CQI report and switching pattern activation irrespective of the nature (periodic or DCI triggered) of the report for consistency.

Observation 4: For aperiodic DCI based CSI report trigger, UE needs a minimum amount of time separation between sending valid CQI report and applying switching pattern.

Proposal 1: At the end of SCell activation (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), spec allows UE a minimum time separation of X ms between transmitting CQI report and applying the semi-statically configured switching pattern.

- Value of X is 3ms

## Deactivated SCell Measurement Requirement

Measurement of deactivated SCell in LB-LB CA via switching scenario leads to throughput loss since UE cannot simultaneously communicate in the FDD and SDL band.

Measuring added but deactivated SCell increases the probability of the SCell to be known during next SCell activation. This, in turn, potentially reduces the SCell activation time. In legacy scenarios, network adds SCell before activating it because network wants to activate a SCell quickly based on dynamic traffic pattern. LB-LB CA via switching was not introduced to increase individual UE’s throughput. Rather, it was introduced to balance DL load properly among different UEs in the cell while ensuring that each UE gets the chance to communicate in UL. Hence, the benefits of deactivated SCell measurement don’t seem to outweigh its pitfalls in LB-LB CA via switching.

Observation 5: In legacy scenarios, network adds SCell before activating it because network wants to activate a SCell quickly based on dynamic traffic pattern.

Observation 6: LB-LB CA via switching was not introduced to increase individual UE’s throughput. Rather, it was introduced to balance DL load properly among different UEs in the cell while ensuring that each UE gets the chance to communicate in UL.

Proposal 2: RAN4 does not define any requirement for deactivated SCell measurement in LB-LB CA.

RAN4 legacy spec allows UE to measure SCell in CA scenario. UE is allowed to interrupt PCell when measCycleSCell is greater than or equal to 640ms. This requirement was derived assuming that UE would keep its RF circuits ON to receive both PCell and deactivated SCell when measCycleSCell is less than 640ms. If measCycleSCell is greater than 640ms, UE can turn off a portion of its RF and not receive SCell in RF. Hence, when UE activates its RF to measure SCell, it causes interruption to PCell.

In this scenario, UE cannot simultaneously measure SCell and PCell. Hence, UE will cause interruption to PCell whenever it measures SSB of SCells. This interruption will happen irrespective of measCycleSCell periodicity which can range from 160ms to 1280ms. The required amount of interruption will vary based on measCycleSCell periodicity, SMTC duration and number of SSBs that need to be measured. Any potential deactivated SCell measurement needs to define interruption requirements suitably considering those factors.

Observation 7: RAN4 has to consider measCycleSCell periodicity, SMTC duration and number of SSBs that need to be measured to define interruption requirements if RAN4 defines deactivated SCell measurement.

## Side Conditions between PCell and SCell during SCell Activation

As mentioned before, the absence of deactivated SCell measurement can potentially increase the SCell activation timeline since SCell could be “unknown” during activation timeline. During legacy “unknown” FR1 SCell activation, UE needs to measure multiple SSB samples of SCell for following purposes:

- Coarse AGC acquisition

- Fine AGC acquisition

- SSB index detection

- Fine time/frequency acquisition

Since FDD and SDL cells are co-located and synchronous in LB-LB CA via switching scenario, UE can use some properties of PCell measurement to estimate the corresponding properties of SCell during SCell activation. This would be possible if MRTD and reception power difference between the cells remain small.

RAN4 has already started discussing MRTD between FDD and SDL cells. reception power difference between the cells needs to be contained within a threshold, too. Note that, the guarantee of small MRTD and reception power difference should not tie this feature with SSB-less SCell. In other words, a UE that does not support SSB-less SCell feature should still be allowed to support LB-LB CA via Switching feature.

Proposal 3: Spec guarantees small MRTD and reception power difference between FDD and SDL cells in LB-LB CA via switching scenario so that UE can use the measurement of FDD PCell to reduce the activation timeline of SDL SCell that it has not measured recently.

- Note: Support of LB-LB CA via switching feature does not depend on “SSB-less SCell” feature. Above spec guarantee for small MRTD and RPD difference are needed just to reduce SCell activation time for UEs.

Proposal 4: If RAN4 cannot reach a consensus to enforce small reception power difference between FDD PCell and SDL SCells, RAN4 should first focus on designing a basic known/unknown SCell activation requirement while utilizing the already agreed side condition on MRTD.

## L1 Measurement of Activated SCells

RAN4 legacy spec defines requirements for different types of L1 measurements, e.g.,

- L1-RSRP measurement

- Beam failure detection (BFD) and candidate beam detection (CBD) measurements

L1-RSRP measurements are essential to operate in SCell. RAN4 needs to define L1-RSRP measurement requirements for active SCells.

Proposal 5: RAN4 defines L1-RSRP measurement requirement for activated SCells.

## Interruption and Scheduling Restriction to FDD and SDL Bands in LB-LB CA

RAN1 made the following agreement during the last meeting [4]:

RAN1 Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA FFS: whether it has specification impact

RAN1 has already agreed that the switching gap is always assumed at the SDL carrier during switching from SDL carrier to FDD carrier. RAN1 also agreed that this switching gap would include UL TA and switching period. Hence, no UL interruption requirement is needed due to switching from SDL to FDD.

RAN1 is still discussing the location of switching gap during switching from FDD to SDL carrier.

Observation 8: RAN1 agreed that the switching gap is always assumed at the SDL carrier during switching from SDL carrier to FDD carrier. RAN1 also agreed that switching gap includes UL TA and switching period.

Observation 9: RAN1 is still discussing the location of switching gap during switching from FDD to SDL carrier.

Proposal 6: RAN4 does not define interruption requirements to FDD UL due to switching from SDL to FDD. RAN4 waits until RAN1’s decision regarding switching location during FDD to SDL switching before deciding corresponding interruption requirements.

## Interruption to Other Carriers/Bands

RAN4 RF session has focused on only two bands during this Rel-19 work item. RF session may focus on other bands, including switching between more than two bands, in the future. Since there is no 3rd band in the current release, RRM session does not need to specify interruption to 3rd band in Rel-19.

Also, this Rel-19 work item is primarily focusing on only one carrier in each band. UE cannot simultaneously operate in the relevant FDD and SDL bands. Hence, if network activates multiple carriers in FDD PCells, they will have to be turned off when UE switches to SDL band. System throughput may get impacted if network switches UE, operating with FDD bands containing multiple carriers, to SDLSCell band. Hence, RAN4 does not need to define interruption requirements to other carriers in the same band since: I) it does not seem to be within Rel-19 scope and 2) its use-case is unlikely.

Observation 10:  RAN4 RF session has focused on only two bands during this Rel-19 work item.

Observation 11: Multiple carriers in FDD band and one carrier in SDL band don’t seem to be within the scope of work item. Network is unlikely to deploy this because the switch to SDL band may impact system throughput in this scenario.

Proposal 7: RAN4 does not define interruptionr equirements to other bands/carriers in Rel-19.

## Measurement of PCells

PCell measurement opportunities would be impacted due to the switching pattern. Figure 4 shows a reference switching pattern. Assuming that the SSBs of PCell and SCell overlap in time and are configured with 20ms periodicity, UE will miss every other SSB of PCell and SCell due to the reference switching pattern shown in Figure 4. Hence, PCell measurement periods need to be extended to accommodate the switching pattern at the UE.

Proposal 8: RAN4 extends all relevant measurement periods of PCell to accommodate switching pattern in LB-LB CA via switching.

# Conclusions

Observation 1: RAN4 has agreed that switching pattern for SDL SCell is not applied when the SCell is in activation procedure. Switching pattern for SDL SCell applies for the active SCell (i.e. after UE reports valid CQI for SCell activation).

Observation 2: Legacy requirements allow network to trigger CSI report for SCell activation via DCI.

Observation 3: Combination of legacy requirements and latest RAN4 agreements mean that the time separation between DCI triggered CQI reporting and the activation of switching pattern can be as small as zero. This goes against the scope of the work item that only focuses on semi-static switching pattern.

Observation 4: For aperiodic DCI based CSI report trigger, UE needs a minimum amount of time separation between sending valid CQI report and applying switching pattern.

Observation 5: In legacy scenarios, network adds SCell before activating it because network wants to activate a SCell quickly based on dynamic traffic pattern.

Observation 6: LB-LB CA via switching was not introduced to increase individual UE’s throughput. Rather, it was introduced to balance DL load properly among different UEs in the cell while ensuring that each UE gets the chance to communicate in UL.

Observation 7: RAN4 has to consider measCycleSCell periodicity, SMTC duration and number of SSBs that need to be measured to define interruption requirements if RAN4 defines deactivated SCell measurement.

Observation 8: RAN1 agreed that the switching gap is always assumed at the SDL carrier during switching from SDL carrier to FDD carrier. RAN1 also agreed that switching gap includes UL TA and switching period.

Observation 9: RAN1 is still discussing the location of switching gap during switching from FDD to SDL carrier.

Observation 10:  RAN4 RF session has focused on only two bands during this Rel-19 work item.

Observation 11: Multiple carriers in FDD band and one carrier in SDL band don’t seem to be within the scope of work item. Network is unlikely to deploy this because the switch to SDL band may impact system throughput in this scenario.

Proposal 1: At the end of SCell activation (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), spec allows UE a minimum time separation of X ms between transmitting CQI report and applying the semi-statically configured switching pattern.

- Value of X is 3ms

Proposal 2: RAN4 does not define any requirement for deactivated SCell measurement in LB-LB CA.

Proposal 3: Spec guarantees small MRTD and reception power difference between FDD and SDL cells in LB-LB CA via switching scenario so that UE can use the measurement of FDD PCell to reduce the activation timeline of SDL SCell that it has not measured recently.

- Note: Support of LB-LB CA via switching feature does not depend on “SSB-less SCell” feature. Above spec guarantee for small MRTD and RPD difference are needed just to reduce SCell activation time for UEs.

Proposal 4: If RAN4 cannot reach a consensus to enforce small reception power difference between FDD PCell and SDL SCells, RAN4 should first focus on designing a basic known/unknown SCell activation requirement while utilizing the already agreed side condition on MRTD.

Proposal 5: RAN4 defines L1-RSRP measurement requirement for activated SCells.

Proposal 6: RAN4 does not define interruption requirements to FDD UL due to switching from SDL to FDD. RAN4 waits until RAN1’s decision regarding switching location during FDD to SDL switching before deciding corresponding interruption requirements.

Proposal 7: RAN4 does not define interruptionr equirements to other bands/carriers in Rel-19.

Proposal 8: RAN4 extends all relevant measurement periods of PCell to accommodate switching pattern in LB-LB CA via switching.

# References

[1] RP-243317, New WID on low band carrier aggregation via switching, 3GPP TSG RAN #106.

[2] RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, 3GPP TSG RAN #106.

[3] R4-2502588, WF on RRM requirements for NR_LBCA_Sw, 3GPP TSG-RAN WG4 Meeting#114.

[4] 3GPP TSG RAN WG1 Chairman’s Notes, RAN1 #120bis.