3GPP TSG-RAN WG4 Meeting # 115	R4-2505536

# Malta, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | DraftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-5 to introduce ACS and blocking characteristics for Ku band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-04-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce ACS and in-band blocking requirements for FR1-NTN and FR2-NTN Ku bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce ACS and in-band blocking requirements for FR1-NTN and FR2-NTN Ku bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no ACS and in-band blocking requirements for FR1-NTN and FR2-NTN Ku bands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.1, 10.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-5>>

10.5	Adjacent channel selectivity

10.5.1	Minimum requirement for Mobile VSAT

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal and the interfering signal are both from the same direction.

For mobile VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2.1.2 and A.3.2.1.3 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 10.5.1-1, Table 10.5.1-1a, Table 10.5.1-2 and Table 10.5.1-3. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.5.1-1: Adjacent channel selectivity for FR2-NTN

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511, n510 | dB | 25 | 25 | 25 | 25 |
| n509, n508 | dB | TBD | TBD | TBD | TBD |

Table 10.5.1-1a: Adjacent channel selectivity for FR1-NTN (above 10GHz)

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |
| --- | --- | --- |
|  |  | 10MHz, 15MHz, 20MHz, 25MHz, 35MHz, 50MHz, 70MHz, 100MHz |
| n248, n247 | dB | TBD |

Table 10.5.1-2: Adjacent channel selectivity test parameters for FR2-NTN

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) NOTE 5 |  |  |  |
| PInterferer for band n512, n511, n510 | dBm | EISREFSENS_50M + 28.7 + 10log10(NRB x SCS x 12 / factor) NOTE 5 |  |  |  |
| PInterferer for band n509, n508 | dBm | EISREFSENS_50M + TBD + 10log10(NRB x SCS x 12 / factor) NOTE 5 |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 [15] with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 [15] and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: The transmitter shall be set to same as the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 5: SCS is in kHz, the “factor” represents the normalized factor to scale wanted signal and interference level for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

Table 10.5.1-3: Adjacent channel selectivity test parameters for FR1-NTN (above 10GHz)

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 10MHz, 15MHz, 20MHz, 25MHz, 35MHz, 50MHz, 70MHz, 100MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) NOTE 5 |
| PInterferer for band n248, n247 | dBm | EISREFSENS_50M + TBD + 10log10(NRB x SCS x 12 / factor) or EISREFSENS_10M + TBD + 10log10(NRB x SCS x 12 / factor) NOTE 5 |
| BWInterferer | MHz | Channel bandwidth |
| FInterferer (offset) | MHz | Channel bandwidth / - Channel bandwidth (NOTE 3) |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 [15] with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 [15] and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: The transmitter shall be set to same as the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 5: SCS is in kHz, the “factor” represents the normalized factor to scale wanted signal and interference level for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz or 24 RBs x 30 kHz SCS x 12, i.e. 8640 kHz. |  |  |

<<Next of Change>>

10.6	Blocking characteristics

10.6.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal and the interfering signal are both from the direction where peak gain is achieved.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

10.6.2	Minimum requirement for Mobile VSAT

In-band blocking is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an interferer at a given frequency offset from the centre frequency of the assigned channel.

For mobile VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2.1.2 and A.3.2.1.3 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 10.6.2-1 and Table 10.6.2-2. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.6.2-1: In band blocking requirements for FR2-NTN

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n512, n511 | dBm | EISREFSENS_50M + 28.7 + 10log10(NRB x SCS x 12 / factor) |  |  |  |
| PInterferer for band n509, n508 | dBm | EISREFSENS_50M + TBD + 10log10(NRB x SCS x 12 / factor) |  |  |  |
| FInterferer (offset) | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 [15] with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 [15] and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG pattern as described in Annex A.5.1.1 and set-up according to Annex C. NOTE 4: Void NOTE 5: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. NOTE 7: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 8: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

Table 10.6.2-2: In band blocking requirements for FR1-NTN (above 10GHz)

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 10MHz, 15MHz, 20MHz, 25MHz, 35MHz, 50MHz, 70MHz, 100MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) NOTE 8 |
| BWInterferer | MHz | Channel bandwidth |
| PInterferer for band n248, n247 | dBm | EISREFSENS_50M + TBD + 10log10(NRB x SCS x 12 / factor) or EISREFSENS_10M + TBD + 10log10(NRB x SCS x 12 / factor) NOTE 8 |
| BWInterferer | MHz | Channel bandwidth |
| FInterferer (offset) | MHz | ≤ - Channel bandwidth / 2 & ≥ Channel bandwidth / 2 (NOTE 5) |
| FInterferer | MHz | FDL_low + Channel bandwidth / 2 to FDL_high - Channel bandwidth / 2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 [15] with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 [15] and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG pattern as described in Annex A.5.1.1 and set-up according to Annex C. NOTE 4: Void NOTE 5: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. NOTE 7: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 8: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz or 24 RBs x 30 kHz SCS x 12, i.e. 8640 kHz. |  |  |

<<End of Change>>