3GPP TSG-RAN WG4 Meeting #115	                                                                                      R4-2505698

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.23.4

Source: 	OPPO

Title: 	Study of testability and interoperability for AI mobility

Document for:	Approval

- Introduction

In RAN#102 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR is approved. In RAN#105 meeting, the related work plan in RAN4 has been further checked and updated as follows:

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In last meeting, the testability issues for AI mobility were discussed and the agreements were captured in WF [1]. In this contribution, we will further discuss the RAN4 impacts on testability based on the open issues captured in [2].

- Discussion

Testing in FR2:

For FR2 RRM measurement prediction, in which both temporal and spatial characteristics need to be considered, conclusions that RAN4 made for AI BM can be reused. According to the progress of AI BM, the testing setup is still under discussion and we can wait for more progress and reuse the outcome as baseline.

Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

Proposal 1: For FR2 RRM measurement prediction, reuse the outcome of AI BM testability as baseline.

Testing for inter-carrier scenario:

For AI mobility, an important feature is the introduction of an inter-carrier scenario, e.g., inter-frequency inter-cell measurement prediction. Based on RAN2 agreements, co-located scenario is used as starting point for inter-frequency prediction. And for single cell prediction, RAN2 focus on the case where cell for measurement and cell for prediction are in the same sector.

In such cases, the following issues should be considered:

- Spatial modeling:

How to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighboring site. For instance, if we're using conductive testing in FR1, how do we reflect the assumption of co-location among different cells? For example, are there any new designs and limitations needed during channel modeling?

- Temporal modeling:

Furthermore, it's important to consider how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship. For example, if we consider predicting cell B's RSRP at time T based on cell A's RSRP at time T, we need to consider how to construct and obtain both cell A's RSRP and cell B's RSRP corresponding to the same time T in the testing environment. Building a stable time-domain RSRP signal for a single test might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time.

- Channel modeling:

Based on RAN2 evaluation, the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. To ensure the testability of measurements for inter-cell prediction, RAN4 need to discuss how to model the proper correlation between two frequency layers.

For FR1, considering the test environment is conducted testing, RAN4 should re-evaluate the selection of channel models. Option 1: UMA channel, Option 2: CDL channel

Proposal 2: For inter-carrier scenario (inter-frequency inter-cell prediction), the following aspects need to be discussed:

- how to model co-located or co-site scenario for the cell to be measured and the cell to be predicted.

- how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship.

- channel modelling used for inter-cell prediction

Testing L3-RSRP measurements instead of L1-RSRP

Regarding L3 RSRP, compared with L1 RSRP, the key difference lies in the L3 filter which may need to be included as part of the testing side conditions.

Proposal 3: RAN4 to consider including L3 filter coefficient as part of the testing side conditions.

Testing setup

For each scenario, further consideration on the measurement configurations is needed on how to define it in RAN4 tests. This includes the configuration of the observation window (e.g., window length, measurement sample configuration within the window), the configuration of the prediction window (e.g., window length, samples to be predicted within the window), and other test settings/conditions. The above aspects can be referenced based on the progress of RAN2's research, particularly the evaluation assumptions.

According to our companion paper on the requirements impact of AI mobility, the above configurations may have impacts on the core and performance requirements. How to setup these configurations can be discussed based on the requirements definition.

Proposal 4: RAN4 to consider how to setup the measurement configurations (e.g., OW, PW, MRRT, etc.) in the tests based on the sets of requirements.

For other testing setup, we think the legacy configurations can be used as baseline and can be updated if needed based on the above aspects (e.g., cell number, channel modelling, beam number, frequency correlation, OW, PW, MRRT, etc.). One clear difference is the channel model. In legacy testing, AWGN channel is used. But for AI based mobility, to reflect the time variation, we need to consider fading channel for testing.

Regarding the number of cells, for inter-frequency measurement, if single cell approach is considered, 2 or 3 cells are enough. And if cluster approach is used, more cells is needed which depends on the requirements definition and can be discussed in WI stage.

Proposal 5: For intra and inter frequency measurement prediction, RAN4 to use legacy testing setup as baseline but to consider fading channel for testing.

Proposal 6: For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed and the exact number can be decided in WI stage according to the requirements discussion.

Ground Truth derivation

Regarding the definition of prediction accuracy, the following agreements were reached in RAN4#113 meeting:

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2.

According to our companion paper on the requirements impact of AI mobility, the ground truth in accuracy requirements is defined as the actual L3-RSRP measurement results at point C based on legacy measurement procedure. And we think it should also be used in the testing.

Proposal 7: Regarding the ground truth derivation in tests, the actual measurement report based on legacy procedure can be used.

Testing goal

Regarding the testing goal, the following agreement was reached in previous meeting.

- Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

For intra and inter measurement prediction, similar as legacy, the testing goal is to verify the measurement reported in OW meets the new requirements if defined and to verify the predicted measurement in PW meets the accuracy requirements under certain side conditions.

Proposal 8: The testing goal for measurement prediction is to verify:

- the measurement reported in OW meets the new requirements if defined;

- the predicted measurement in PW meets the accuracy requirements under certain side conditions.

Testing of Generalization aspects

For the generalization of test, we understand it is a common issue for AI based use cases. The issue is still discussing in AI/ML for NR Air Interface without consensus and we think the methodologies can be reused for mobility use case.

From RAN4 perspective, during WI stage, RAN4 need to discuss how to set the different configurations of UE speed, frequencies and cell configuration, e.g., dynamic configuration or fixed multiple sets of configurations. The same discussion is under AI/ML for NR Air Interface. The conclusions can be referred when WI for AI mobility starts.

Proposal 9: RAN4 to discuss the testing configurations for generalization in WI stage and the methodology discussed in NR_AIML_Air WI can be used as starting point.

Testing for event prediction

In our companion paper, we discussed the possible requirements and performance metrics for measurement event prediction. Correspondingly, we need to consider the testability for the performance metrics. Based on RAN2 discussion, some KPIs are related to the real event, e.g., for the calculation of F1 score, the true event prediction is when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa. But it seems hard to get the real event in the test if the handover is based on the predicted event. This aspect needs to be considered for testing.

Proposal 10: For the testing of event prediction, RAN4 to study how to know the real event occurs.

- Conclusions

In this contribution, we discussed RAN4 impacts on testability for AI mobility and got the following proposals:

Proposal 1: For FR2 RRM measurement prediction, reuse the outcome of AI BM testability as baseline.

Proposal 2: For inter-carrier scenario (inter-frequency inter-cell prediction), the following aspects need to be discussed:

- how to model co-located or co-site scenario for the cell to be measured and the cell to be predicted.

- how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship.

- channel modelling used for inter-cell prediction

Proposal 3: RAN4 to consider including L3 filter coefficient as part of the testing side conditions.

Proposal 4: RAN4 to consider how to setup the measurement configurations (e.g., OW, PW, MRRT, etc.) in the tests based on the sets of requirements.

Proposal 5: For intra and inter frequency measurement prediction, RAN4 to use legacy testing setup as baseline but to consider fading channel for testing.

Proposal 6: For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed and the exact number can be decided in WI stage according to the requirements discussion.

Proposal 7: Regarding the ground truth derivation in tests, the actual measurement report based on legacy procedure can be used.

Proposal 8: The testing goal for measurement prediction is to verify:

- the measurement reported in OW meets the new requirements if defined;

- the predicted measurement in PW meets the accuracy requirements under certain side conditions.

Proposal 9: RAN4 to discuss the testing configurations for generalization in WI stage and the methodology discussed in NR_AIML_Air WI can be used as starting point.

Proposal 10: For the testing of event prediction, RAN4 to study how to know the real event occurs.

- References

R4-2504901 WF on RRM requirements for FS_NR_AIML_Mob

R4-2503620 Topic summary for [114bis][210] FS_NR_AIML_Mob