# 3GPP TSG-RAN WG4 Meeting #115	                                                	     R4-2505874

# Malta, MT, 19 – 23 May, 2025

Agenda Item:	7.25.3.3

Source: 	Apple

Title:	Discussion on on-demand SIB1

Document for:	Discussion

# Introduction

RAN4 requirements for on-demand SIB1 was initially discussed in the previous RAN4 meeting. The latest agreements can be found in [1].

In this contribution we provide further discussion on the open issue. After discussion, corresponding proposals are provided.

# Discussion

### Sub-topic 2-1 RRM requirements impacts for OD-SIB1

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Way forward:

    - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo)

    - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE)

    - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time. (E///, LGE, MTK)

Potential RRM impact is on paging interruption.

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

As can be seen, when UE is reading SI from target cell UE may miss paging from serving cells. In OD-SIB1 scenario, based on RAN1 and RAN2 design, UE needs to transmit UL WUS and wait for RAR in order to get OD-SIB1 from target cell. Therefore, if the transmission of UL WUS or reception of RAR from target cell collides with paging reception in serving cell, UE may also fail to receive paging. This needs to be reflected in RRM requirements of Maximum interruption in paging reception.

Proposal 1: potential paging interruption due to UL WUS transmission and RAR reception for OD-SIB1 acquisition needs to be reflected in RRM requirements of Maximum interruption in paging reception.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for on0demand SIB1. After discussion, the following conclusions are provided:

Proposal 1: potential paging interruption due to UL WUS transmission and RAR reception for OD-SIB1 acquisition needs to be reflected in RRM requirements of Maximum interruption in paging reception.

# Reference

- R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei