3GPP TSG-RAN WG4 Meeting #115						R4-2505901

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.16.3

Source:	Apple

Title:	Discussion on Spatial Channel Modeling

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#114bis spatial channel model for demodulation requirements was discussed and way forward [1] was approved. In this contribution we share our views on the study for spatial channel models for demodulation and CSI reporting requirements in RAN4.

# 2. Discussion

## General

In RAN4#114bis the following were discussed:

Prioritisation on SU and MU MIMO

Way Forward

SU-MIMO is prioritised during RAN4#114-bis

FR2 Aspects

Way Forward

FR1 is prioritised during RAN4#114-bis, any removal of aspects from SI to be discussed at RAN Plenary.

The study so far has focused on SU-MIMO scenarios in FR1. We have not concluded on aspects of alignment and comparison yet for SU-MIMO. We don’t see it feasible to cover MU-MIMO and FR2 in the remaining time for this study. Although this is something to be discussed in RAN plenary to remove aspects of the SI, we think we should have a discussion and agreement in RAN4 before it is brought up in the plenary.

- Study so far is focused on SU-MIMO in FR1, with no conclusions yet.

- It would not be feasible to include MU-MIMO and FR2 in the remaining time.

- De-prioritization of MU-MIMO and FR2 needs to be discussed and agreed in RAN4 before it is brought up in RAN plenary for SI scope update.

De-prioritize MU-MIMO and FR2 for Rel-19 SCM study.

## Common for all methodologies

The following was discussed in RAN4#144bis -

PDSCH ‘Fixed’ PMI Selection

Way Forward

For alignment a random PMI will be used, for comparison cases with a ‘fixed’ PMI FFS on approach.

PMI Granularity in PMI reporting cases

Way Forward

RAN4 to use wideband PMI for Type 1 Follow PMI cases

RAN4 to use wideband PMI for Type 1 Random PMI cases

Note: Previous requirements have used both wideband and sub-band PMI

Precoder Granularity for PDSCH cases

Way Forward

Same assumption as PDSCH demodulation requirements for random PMI

For number of TX > 1:Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12].

For number of Tx>2 and Rank=1 or 2, Set “codebookMode” to 1 as defined in section 5.2.2.2.1 of TS 38.214 [12]

RAN4 to use wideband PMI for Type 1 Fixed PMI cases

We prefer to use the same test setup/ parameters for alignment and comparison to avoid duplication of work if possible. We don’t see the necessity to use fixed PMI for comparison purposes either. The exercise of fixing PMI is very cumbersome and requires a 2 step-process. We think it is okay to live with the limitation that maximum TP might not be achieved with random PMI with 8RX 8-layers. We aren’t defining requirements here and this is a study.

For PMI test case, we have agreed to use wideband PMI for Type I at least for alignment. When it comes to comparison between eType II and Type I performance, there might be an issue raised that we are comparing SB (eType II) to WB (Type I) performance, and we might need to use a different assumption for comparison. Since we have already agreed to use WB precodgin granularity for Type I codebook for SU-PMI, we propose to agree to use the same for comparison as well.

- Same test parameters/ set up should be used for alignment and comparison to avoid duplication of work.

- It is very cumbersome to fix PMI for PDSCH test cases and requires a 2-step process

- For the study, it should be okay if max TP is not achieved with random PMI since requirements are not being defined here.

Only use random PMI for PDSCH test cases with 4RX and 8RX for alignment and comparison.

- We have agreed to use WB PMI for SU-PMI test cases with Type I CB at least for alignment, while eType II is always SB granularity.

Use WB recoding granularity for Type I even for comparison, even though not an equivalent comparison.

## CDL based methodologies

In RAN4#114bis it was agreed to use a truncated CDL channel model ‘A4’ for alignment.

CDL Table Truncation

Way Forward

As a starting point, a single CDL Table taken forward to include truncation and cluster combining (details included in Annex A). The number of clusters used will be 12.

This will be assessed during RAN4#115 if it can support 8 layers; and if a untruncated option is required.

Truncated CDL Model

Agreement

The configuration table of the truncated CDL model (commonly referred to as ‘A4’) is derived by the procedure detailed in Annex A, and includes truncation, as well as, combining of clusters with equal angular profiles under the earliest delay. The configuration is based on 38.901 CDL link level tables after delay and angle scaling.

The implementation of the truncated CDL model treats all clusters the same, and uses 20 rays per cluster (i.e., no ray splitting or sub-clustering is applied)

Note: Companies welcome to bring results for both truncated and untruncated results.

During post RAN4#114bis email discussion the truncated channel models were discussed. We have one truncated channel model for AAV(1,1)/AAV3 and one for AAV(8,1)/AAV1Y. The assumptions of antenna radiation pattern, AAV, BS down tilt need to be made for the truncation. Hence the truncated channel model is only applicable for that scenario. It is fine for the purpose of this study, but it would not be a channel model that can be used with any configuration. Also, the angular scaling was done with the assumption of UMa scenario. With the truncation, it is worth pointing out that the angular spreads are no longer the same as the desired value used for UMa scenario.

- The truncated channel model is only applicable to the configuration assumed for truncation - antenna radiation pattern, AAV, BS down tilt and needs to be re-derived based on the scenario/ configuration

- The mean angle spreads are not the same as the desired angle spread used for UMa scenario.

For truncated channel model after the angle and delay scaling, the delays need to be quantized to the delay resolution (2ns for FR1) and the angles need to be rounded to 1 decimal point. We propose to round the angles to 1 decimal.

In the final CDL table, round the angles to 1 decimal point.

In case the truncated channel model A4 is not suitable for the study, we propose to use 20 cluster model variant of A2, where the clusters that are derived from strong clusters are combined and each cluster gets 20 rays.

We provide the channel models A2-20 and A4 below.

Table 1: UMa-CDL-C A2-20 Channel model table

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 76 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 80 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 232 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 240 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 290 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 300 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 340 | -14.1 | -73.0 | 43.1 | 92.3 | 50.8 |
| 9 | 448 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 10 | 476 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 11 | 792 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 12 | 988 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 13 | 1552 | -16.9 | 62.2 | -22.6 | 94.0 | 53.9 |
| 14 | 1678 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| 15 | 2002 | -18.8 | -76.3 | -19.8 | 90.7 | 37.5 |
| 16 | 2044 | -20.1 | 57.4 | 4.5 | 108.2 | 53.1 |
| 17 | 2300 | -19.0 | 58.1 | 0.2 | 91.7 | 35.8 |
| 18 | 2420 | -18.7 | 67.3 | -4.9 | 91.6 | 52.9 |
| 19 | 2568 | -24.6 | 76.2 | -28.3 | 106.4 | 46.4 |
| 20 | 3154 | -25.8 | -91.4 | 29.3 | 109.5 | 44.7 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

Table 2: UMa-CDL-C A4 Channel model table

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

On AAV configuration, we have AAV3/(1,1) – pass through which is agreed for alignment. For comparison in addition we also have AAV1Y/(8,1) – sub-array configuration.

AAV Configuration for alignment

Agreement

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

AAV Configuration for comparison

Way Forward

Options:

- Option 1 (previously 1Y): with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

  - For interested companies: (M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only

- Option 3: with AAV with the following configuration

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

We understand that AAV with sub-array is used in deployment, but we don’t see the need to study this and have it in our CDL channel model configuration. From UE perspective nothing different is tested, as this is transparent to the UE. Using the sub-array config needs extra normalization step for SNR which increased overhead. At least from our simulation results for SU-MIMO, we do observe some gain with sub-array AAV, but don’t see the necessity of introducing this complexity.

- From UE perspective nothing different is tested with sub-array AAV.

- Using sub-array AAV required extra SNR normalization step that adds overhead.

Also, in [2] with SU-PMI we observe gains with eType II over Type I without using sub-array AAV by increasing the number of TX ports.

- For SU-PMI we observe gains with eType II over Type I with larger number of TX ports without sub-array AAV.

Based on the above observations we propose on only retain AAV3 going forward in this study.

For CDL based methodologies, only AAV3/AAV(1,1)/pass-through is retained for the study.

Frequency Bands

The current models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions on RMS delay spread, UE velocity, etc. RAN4 defines band agnostic demodulation and CSI reporting requirements.

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

It would be very important to study if the spatial channel model can be defined for all applicable carrier frequencies/ bands for each FR.

RAN4 to evaluate if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

On results collection for alignment and comparison

Currently we have several spreadsheets with multiple sheets within to collect results for the different options. The task of data entry itself is cumbersome and time consuming. We suggest that we simplify the alignment and comparison by collecting the following metrics instead of full TP curves, as was done in [3]:

SU-MIMO PDSCH

SNR at 70%, 30% Max TP

SU-PMI

SNR at 90 % Max TP for follow Type I PMI

Gamma at 90% Max TP with follow Type I PMI

SNR at 90 % Max TP for follow eType II PMI

Gamma at 90% Max TP with follow eType TI PMI

- The task of data entry and collection for results alignment and comparison is very cumbersome with raw data (SNR and rel TP values)

Simplify the process of collecting results by collecting the following metrics:
SU-MIMO PDSCH
     SNR at 70%, 30% Max TP
SU-PMI
     SNR at 90 %, 70% Max TP for follow Type I PMI
     Gamma at 90% Max TP with follow Type I PMI
     SNR at 90 % , 70%Max TP for follow eType II PMI
     Gamma at 90% Max TP with follow eType TI PMI

## Comparison of methodologies

Comparison with Legacy TDL

Agreement (From RAN4#114):

The following Legacy TDL models for comparison:

- Option 1: TDLC 300-10/100 with following correlation:

  - Option 1a: Low

  - Option 1b: XP Medium

  - Option 1c: XP High

  - Option 1d: Medium

Comparison with legacy TDL models is necessary in the study to understand the limitations of current methodology and also for comparison. The channel models and correlation should be chosen carefully, as not all corr levels are suitable for all antenna configs. We propose the following:

For SU-MIMO PDSCH:

4x4:  TDL with low correlation, and XP-Med.

8x8: TDL with low correlation, XP-MedA (variant of XP-Med with smaller a, b)

For SU-MIMO PMI:

8x4: TDL XP-Med, and XP-High.

32x4: TDL XP-Med or variant with smaller a

For legacy TDL, the following antenna correlation can be used: 
For SU-MIMO PDSCH:
     4x4:  TDL with low correlation, XP-Med.
     8x8: TDL with low correlation, XP-MedA (variant of XP-Med with smaller a, b)
For SU-MIMO PMI:
     8x4: TDL XP-Med, and XP-High.
     32x4: TDL XP-Med or variant with smaller a

# 3. Conclusion

In this paper, we provide our views on scope of study for spatial channel models for defining demodulation and CSI reporting requirements in RAN4. Our observations and proposals are captured below:

General

- Study so far is focused on SU-MIMO in FR1, with no conclusions yet.

- It would not be feasible to include MU-MIMO and FR2 in the remaining time.

- De-prioritization of MU-MIMO and FR2 needs to be discussed and agreed in RAN4 before it is brought up in RAN plenary for SI scope update.

- De-prioritize MU-MIMO and FR2 for Rel-19 SCM study.

Common for all methodologies

- Same test parameters/ set up should be used for alignment and comparison to avoid duplication of work.

- It is very cumbersome to fix PMI for PDSCH test cases and requires a 2-step process

- For the study, it should be okay if max TP is not achieved with random PMI since requirements are not being defined here.

Only use random PMI for PDSCH test cases with 4RX and 8RX for alignment and comparison.

- We have agreed to use WB PMI for SU-PMI test cases with Type I CB at least for alignment, while eType II is always SB granularity.

Use WB recoding granularity for Type I even for comparison, even though not an equivalent comparison.

CDL based methodologies

- The truncated channel model is only applicable to the configuration assumed for truncation - antenna radiation pattern, AAV, BS down tilt and needs to be re-derived based on the scenario/ configuration

- The mean angle spreads are not the same as the desired angle spread used for UMa scenario.

In the final CDL table, round the angles to 1 decimal point.

- From UE perspective nothing different is tested with sub-array AAV.

- Using sub-array AAV required extra SNR normalization step that adds overhead.

- For SU-PMI we observe gains with eType II over Type I with larger number of TX ports without sub-array AAV.

For CDL based methodologies, only AAV3/AAV(1,1)/pass-through is retained for the study.

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

RAN4 to evaluate if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

- The task of data entry and collection for results alignment and comparison is very cumbersome with raw data (SNR and rel TP values)

Simplify the process of collecting results by collecting the following metrics:
SU-MIMO PDSCH
     SNR at 70%, 30% Max TP
SU-PMI
     SNR at 90 %, 70% Max TP for follow Type I PMI
     Gamma at 90% Max TP with follow Type I PMI
     SNR at 90 % , 70%Max TP for follow eType II PMI
     Gamma at 90% Max TP with follow eType TI PMI

Comparison of methodologies

For legacy TDL, the following antenna correlation can be used: 
For SU-MIMO PDSCH:
     4x4:  TDL with low correlation, XP-Med.
     8x8: TDL with low correlation, XP-MedA (variant of XP-Med with smaller a, b)
For SU-MIMO PMI:
     8x4: TDL XP-Med, and XP-High.
     32x4: TDL XP-Med or variant with smaller a

# Reference

- R4-2504756, “Way Forward for [114bis][320] NR_SCM”, Nokia.

- R4-2505902, “Simulation results for Spatial Channel Modeling study”, Apple

- R4-2504698, “Simulation summary for Alignment for NR SCM”, Apple, Ericsson.