3GPP TSG-RAN WG4 Meeting #115				                                                R4-2505998

Saint Julian’s, Malta, May 19th – May 23rd, 2025

Title:	Discussion on performance requirements of fast SCell activation for UE supporting R18 EMR

Source:	CATT

Agenda Item:	7.18.6

Document for:	Discussion

1 Introduction

RAN4 started discussing the performance requirements of fast SCell activation for UE supporting R18 EMR in April meeting. In this contribution, we will present our views on the relevant open issues.

2 Discussion

General principle for defining test case

Regarding the test cases for this new feature, considering that RAN4 have defined test cases for Rel-18 eEMR measurement and SCell activation separately in previous releases, RAN4 can use those existing test cases as a starting point. For example, the test cases in A.6.6.9/A.7.6.23 for IDLE mode fast CA/DC cell reselection measurement with valid reporting and the test case in A.6.5.3/A.7.5.3 for SCell activation and deactivation.

Next we will discuss how to combine the test cases for Rel-18 eEMR and SCell activation.

Firstly, before UE is indicated to activate target SCell, UE needs to perform eEMR measurement on the target SCell and report a valid result, which, in our understanding, is exactly the same as that in the test case where valid reporting is included. RAN4 can discuss which one can be reused.

Observation 1: The contents in the legacy Rel-18 eEMR test case with valid reporting can be reused.

After UE reports a valid EMR result of target SCell during RRC connection setup as requested by network, UE can be signaled to activate the target SCell. But note that the goal of this feature to shorten the SCell activation delay for unknown SCell, i.e., the target SCell shall be unknown, which means this target SCell needs to meet the unknown SCell conditions. In TS 38.133 clause 8.3.2, the known conditions are defined as follows [1]:

FR1 SCell known/unknown conditions: SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. FR2 SCell known/unknown conditions: For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown.

In order to make the target SCell unknown, a simple way is to extend the time gap between early measurement report transmission and SCell activation command reception. Specifically, the time gap needs to be larger than max(5*measCycleSCell,  5*DRX cycles) for FR1 SCell activation case, and 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4 for FR2 SCell activation case.

Observation 2: Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions, which ensures the target SCell is unknown.

After UE receives SCell activation command from TE, it shall start to activate the target SCell and send the first CSI report in the first available UL resource after at least one CSI-RS occasion and satisfy the activation delay requirement defined in clause 8.3.2A. Some contents in the legacy SCell activation delay test case can be reused. RAN4 can discuss which one can be reused. In Figure 1, we provide an example of high-level test procedure to demonstrate the whole idea.

Figure 1-Illustration of test procedure for fast SCell activation for UE supporting Rel-18 eEMR

Proposal 1: Reuse the contents in the legacy test cases for Rel-18 eEMR measurement with valid result reporting and SCell activation.

Proposal 2: Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions:

-	For target SCell in FR1, time gap needs to be larger than max(5*measCycleSCell,  5*DRX cycles);

-	For target SCell in FR2, time gap needs to be larger than4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4.

Number of test cases

In RAN4#114bis meeting, proposals of scope of test cases were brought up and a recommended way forward was provided for further discussion based on relevant proposals [2]:

Issue 2-2-1: Scenarios of test cases - Recommended WF - RAN4 to define test cases for eEMR-based fast SCell activation based on the following aspects: - Scenarios: - Normal SCell activation - Direct SCell activation at SCell addition - PUCCH SCell activation - Frequency range: - FR1 and FR2 - UE capabilities and/or network configurations: - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode - Discuss whether to choose a subset of combinations of the above aspects to define test cases. - Discuss the applicability rule whether UE is required to pass all of test cases.

Firstly, all three types of SCell activation procedure should be covered:

      - Normal SCell activation (i.e., MAC CE based Single SCell Activation)

      - Direct SCell activation at SCell addition

      - PUCCH SCell activation

Secondly, test cases that cover different UE capabilities and NW configurations should also be considered:

      - Combination 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

      - Combination 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

      - Combination 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Regarding the selection of frequency range, RAN4 can determine a subset to control the number of test cases. On top of that, RAN4 can also focus on CA scenario by following the legacy test case configurations. In addition, RAN4 need to discuss whether to configure DRX in test cases or not.

Proposal 3: Test cases need to cover each scenario and UE capabilities and NW configuration combination:

    - Scenarios:

      - Normal SCell activation

      - Direct SCell activation at SCell addition

      - PUCCH SCell activation

    - UE capabilities and/or network configurations:

      - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

      - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

      - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Proposal 4: Test cases focus on CA scenario only, excluding

  - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA)

  - SCell being in the same band as PCell in FR2 (i.e. intra-band CA)

Proposal 5: RAN4 do not define the same test case for both FR1 and FR2.

Proposal 6: RAN4 to discuss whether to configure DRX in the test cases or not.

3 Conclusion

In this paper, we discussed performance requirements of fast SCell activation for UE supporting R18 EMR, and the following proposals are given:

Observation 1: The contents in the legacy Rel-18 eEMR test case with valid reporting can be reused.

Observation 2: Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions, which ensures the target SCell is unknown.

Proposal 1: Reuse the contents in the legacy test cases for Rel-18 eEMR measurement with valid result reporting and SCell activation.

Proposal 2: Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions:

-	For target SCell in FR1, time gap needs to be larger than max(5*measCycleSCell,  5*DRX cycles);

-	For target SCell in FR2, time gap needs to be larger than4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4.

Proposal 3: Test cases need to cover each scenario and UE capabilities and NW configuration combination:

    - Scenarios:

      - Normal SCell activation

      - Direct SCell activation at SCell addition

      - PUCCH SCell activation

    - UE capabilities and/or network configurations:

      - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

      - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

      - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Proposal 4: Test cases focus on CA scenario only, excluding

  - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA)

  - SCell being in the same band as PCell in FR2 (i.e. intra-band CA)

Proposal 5: RAN4 do not define the same test case for both FR1 and FR2.

Proposal 6: RAN4 to discuss whether to configure DRX in the test cases or not.

4 References

- 3GPP TS 38.133, NR; Requirements for support of radio resource management, V18.8.0.

- R4-2503619, Topic summary for [114bis][209] NR_RRM_Ph5_Part2, RAN4#114bis.