3GPP TSG RAN WG4 Meeting #115		R4-2506072

St Julian, Malta, May 19th – 23rd , 2025

Title: 	TP to TR 38.768 on clause 4 (Scenarios)

Agenda Item:	7.7.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, TR 38.768 has further been updated to v0.0.3 by capturing TPs approved in this meeting [1].

Clause 4 lists several operator-provided scenarios intended for this feature. However, this Work Item focuses solely on the CA_n5A-n29A band combination as the example used to specify the feature. Other similar band combinations may be addressed in a separate Work Item once the feature is defined. Therefore, the clarification proposed in this contribution offers a more accurate description.

# 2 Text Proposal

--- < Start of change> ---

# 4	Low NR band aggregation scenarios

<Editor’s note: capture operator-requested scenarios in this clause>

Operator interest for aggregating low band spectrum has always been high during the entire history of the carrier aggregation feature, spanning LTE and NR specifications.  Considering the practicalities of handset RF front end architectures available in the market today, the operators have requested 3GPP to consider specifying a solution based on a switching scheme which can potentially accommodate the related limitations.

The following observations motivate the effort undertaken by this Work Item:

-	The amount of mid-band spectrum an operator holds is typically 10-20 times greater than their low-band spectrum holdings.

-	Mid-band spectrum is more effective closer to cell sites, while low-band spectrum propagates farther, making it more useful at greater distances from sites.

-	Low-band spectrum carries significant traffic volumes in urban (indoor) and rural areas, leading to congestion that severely degrades customer experience.

-	This issue is exacerbated by the fact that low-band SDL bands, which reach most poor coverage areas, cannot be paired with an UL, rendering them useless.

-	Utilization of SDL bands via Low-Low band CA, where one low band serves as an SDL, could potentially solve this problem. However, this solution doesn't exist due to OEM challenges in supporting it.

In the context of the scope of this Work Item where only CA_5A-n29A is handled as the example band combination, Table 4-1 below also illustrates other applicable operator-provided scenarios which can be specified in a separate basket work item once the feature is completed.

Table 4-1: Operator-provided scenarios for NR low band aggregation via switching

| Configuration | Illustration | Notes |
| --- | --- | --- |
| CA_n5A-n29A |  | (1), (2) |
| CA_n12A-n29A |  | (3) |
| CA_n14A-n29A |  |  |
| CA_n28A-n67A |  | (4), (5) |
| CA_n29A-n71A |  | (6), (7) |
| NOTE 1: CA_n5A-n29A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 2: Fractional bandwidth of a single antenna to support this combination is 22% and represents a practical implementation challenge NOTE 3: There is no incumbent narrowband service in the band gap between n29 and n12 DL (728~729 MHz) in Canada NOTE 4: In Europe band n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz DL NOTE 5: 3GPP will introduce requirements for band n28 assuming full band duplexer architecture in Rel-19 NOTE 6: CA_n29A-n71A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 7: Fractional bandwidth of a single antenna to support this combination is 16.5% and represents a practical implementation challenge |  |  |

--- < End of change> ---

# Reference

[1] R4-2505146, “TR 38.768 v0.0.3”, Apple