3GPP TSG-RAN4 Meeting #115	R4-2506145

# Malta, Malta, 19th May 2025 - 23rd May 2025

Source:			Ericsson

Title:			NTN Ku bands SAN RF

Agenda item:		7.11.6

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised [1]) including support for 15 kHz SCS.

In last RAN4#114bis meeting, RAN4 made additional agreements on SAN RF requirements with focus on the FR1-NTN frequency range ([2]). We also proposed ([4]) to introduce a new SAN type for FR1-NTN range.

Some companies were reluctant with this proposal, but we would like to discuss it further in this contribution.

# Discussion

## SAN OTA requirements, FR1-NTN and FR2-NTN

As mentioned in the introduction, we proposed in last RAN4#114bis meeting ([4]) to introduce a new SAN type (i.e. “type 1-A-O”) for FR1-NTN range which would help specifying SAN RF requirements for the NTN Ku-bands in FR1-NTN. Some companies were not comfortable with the proposed naming for this new SAN type so, in the following, we will name it “type 1-XXX” and RAN4 could decide later how to name it if RAN4 agreed to introduce such new SAN type.

Also, as it was the first time this proposal was suggested, we understand that some companies were reluctant with this approach. Nevertheless, we still think this would be the most efficient way to specify SAN RF requirements in FR1-NTN, equivalent to those specified in FR2-NTN. We are elaborating further why in the following sections, checking how NTN KU-bands requirements should be specified in both FR1-NTN, keeping equivalence with those specified in FR2-NTN for the corresponding bands.

### Tx requirements

In TS 38.108, most of the SAN type 1-O Tx requirements are just referring to the conducted SAN type 1-H requirements, this is for example the case for the OTA total power dynamic range requirement:

This means that for the NTN Ku-bands in FR1-NTN, those Tx requirements would also refer to the SAN type 1-H requirements, which is a type that RAN4 agreed to not specify for the NTN Ku-bands ([5]):

This might be solved by updating the current wording but still, it would remain weird, for the NTN Ku-bands in FR1-NTN to refer to conducted requirements which are not relevant in the NTN Ku-bands context.

Observation1: It would be weird for SAN type 1-O in the NTN Ku-bands to refer to conducted requirements: the 1-C requirements are not relevant in the context of the NTN Ku-bands as RAN4 agreed to not specify SAN type 1-C, not SAN type 1-H for those bands.

Moreover, in last RAN4#114bis meeting, RAN4 agreed ([2]) to specify new OTA total power dynamic range requirements for the NTN Ku-bands SAN:

If those requirements would have to be specified for the NTN Ku-bands, it would then be straight forward to specify them in a new clause (e.g. clause “9.4.3.4 Minimum requirement for SAN type 1-XXX”) for the new SAN type 1-XXX, instead of introduction some exception based the NTN band number then.

Observation2: Introducing a new SAN type 1-XXX will ease the specification of requirements which should be equivalent FR1-NTN and FR2-NTN.

### Rx requirements

In last RAN4#114bis, it was proposed ([6]) to specify FR1-NTN REFSENS requirements for the NTN Ku-bands based on EISREFSENS_10M which would a manufacturer declared value characterising the considered SAN sensitivity for 10 MHz channel BW. This would be a very similar approach to FR2-NTN REFSENS requirements which is specified based on EISREFSENS_50M which is declared for 50MHz channel BW.

If RAN4 agreed with this approach, REFSENS requirements for SAN type 1-O would have 2 different specifications:

- The “legacy” SAN type 1-O REFSENS will be specified based on a fixed value, adding an offset (ΔOTAREFSENS).

- The NTN Ku-bands SAN type 1-O REFSENS will be specified based on EISREFSENS_10M, adding another offset (ΔFR2_REFSENS).

The REFSENS specification for SAN type 1-O would become ambiguous. On the other hand, by introducing a new SAN type 1-XXX,  REFSENS specification would be simplified, introducing a new clause “10.3.4 Minimum requirement for SAN type 1-XXX” , highly increasing the specification readability as shown in 5-Annex A of this document.

Similar observation and conclusion could be done for the other Rx requirements: OTA ACS, OTA out-of-band blocking and OTA in-channel selectivity.

Observation3: Introducing a new SAN type 1-XXX will clarify the specification of NTN Ku-bands Rx requirements, not mixing different requirement specifications inside the same SAN type 1-O.

### Conclusion

In the previous sections, we have shown the advantages of introducing a new SAN type 1-XXX for specifying the NTN Ku-bands requirements in FR1-NTN.

Introducing this new type is not only motivated by increasing the specification readability and its simplification, but it would also be justified by the taken approach when specifying NTN Ku-bands requirements which shall be equivalent in FR1-NTN and FR2-NTN.

Our draft CR ([3]) submitted in last RAN4#bis meeting captured what type of changes should be done to introduce this new SAN type for the NTN Ku-bands. As it could be observed, those changes are relatively limited and would ensure the consistency of RF requirements for the NTN Ku-bands.

Moreover, in the future, it would be straight forward to reuse this new SAN type to introduce any NTN band that should also be specified in both FR1-NTN and FR2-NTN.

We would then make the following proposal.

Proposal1: Introduce a new SAN type (type 1-XXX) in FR1-NTN based on SAN type 2-O RF requirements for the NTN Ku-bands in FR1-NTN.

# Conclusion

In this contribution, we further elaborated on our proposal from last meeting to introduce a new SAN type, helping specifying SAN RF requirements in both FR1-NTN and FR2-NTN in a consistent way. We made the following observations and proposal:

Observation1: It would be weird for SAN type 1-O in the NTN Ku-bands to refer to conducted requirements: the 1-C requirements are not are not relevant in the context of the NTN Ku-bands as RAN4 agreed to not specify SAN type 1-C, not SAN type 1-H for those bands.

Observation2: Introducing a new SAN type 1-XXX will ease the specification of requirements which should be equivalent FR1-NTN and FR2-NTN.

Observation3: Introducing a new SAN type 1-XXX will clarify the specification of NTN Ku-bands Rx requirements, not mixing different requirement specifications inside the same SAN type 1-O.

Proposal1: Introduce a new SAN type (type 1-XXX) in FR1-NTN based on SAN type 2-O RF requirements for the NTN Ku-bands in FR1-NTN.

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2504720, WF for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2503945, Draft CR to TS 38.108 - Introduction of a new BS type in FR1-NTN for the NTN Ku-bands.

- R4-2503943, NTN Ku bands SAN RF, Ericsson

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2503742, Discussion on SAN RF requirements for NR NTN Ku band, ZTE Corporation, Sanechips

# Annex A: Example of the REFSENS specification impacts when introducing a new SAN type 1-XXX

## 10.3	OTA reference sensitivity level

### 10.3.1		General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received at the RIB at which a reference performance requirement shall be met for a specified reference measurement channel.

The OTA REFSENS requirement shall apply to each supported polarization, under the assumption of polarization match.

### 10.3.2		Minimum requirement for SAN type 1-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-1: SAN GEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 | -99.3 - ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-NTN-A1-2 | -99.4 - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-NTN-A1-3 | -96.5 - ΔOTAREFSENS |
| 20 | 15 | G-FR1-NTN-A1-4 | -92.9 - ΔOTAREFSENS |
| 20 | 30 | G-FR1-NTN-A1-5 | -93.2 - ΔOTAREFSENS |
| 20 | 60 | G-FR1-NTN-A1-6 | -93.3 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |

Table 10.3.2-2: SAN LEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 | -102.4 - ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-NTN-A1-2 | -102.5 - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-NTN-A1-3 | -99.6 - ΔOTAREFSENS |
| 20 | 15 | G-FR1-NTN-A1-4 | -96.0 - ΔOTAREFSENS |
| 20 | 30 | G-FR1-NTN-A1-5 | -96.3 - ΔOTAREFSENS |
| 20 | 60 | G-FR1-NTN-A1-6 | -96.4 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |

### 10.3.3	Minimum requirement for SAN type 2-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz SAN channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a reference measurement channel with 50 MHz SAN channel bandwidth it does not imply that SAN has to support 50 MHz SAN channel bandwidth.

For GEO class SAN, EISREFSENS_50M is an integer value in the range [-140] to [-149] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_50M is an integer value in the range [-120] to [-129] dBm. The specific value is declared by the vendor.

Table 10.3.3-1: FR2-NTN OTA reference sensitivity requirement

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-NTN-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
| 50 | 120 | G-FR2-NTN-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-NTN-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

### 10.3.4	Minimum requirement for SAN type 1-XXX

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_10M, which is based on a reference measurement channel with 10 MHz SAN channel bandwidth. EISREFSENS_10M itself is not a requirement and although it is based on a reference measurement channel with 10 MHz SAN channel bandwidth it does not imply that SAN has to support 10 MHz SAN channel bandwidth.

For GEO class SAN, EISREFSENS_10M is an integer value in the range [YY1] to [ZZ1] dBm. The specific value is declared by the vendor.

For LEO class SAN, EISREFSENS_10M is an integer value in the range [YY2] to [ZZ2] dBm. The specific value is declared by the vendor.

Table 10.3.3-1: FR1-NTN OTA reference sensitivity requirement

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10 | 30 | TBD | [EISREFSENS_10M + ΔFR1_REFSENS] |
| 10 | 15 | TBD | [EISREFSENS_10M + ΔFR1_REFSENS] |
| 15, 25, 35, 50 | 15 | TBD | TBD |
| 20, 25, 35, 50, 70, 100 | 30 | TBD | TBD |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_10M shall be within the range specified above. |  |  |  |