3GPP TSG-RAN WG4 Meeting #115	R4-2506165

St Julian’s, Malta, 19th – 25th May 2025

Title:	UE architecture switching process

Source:	Qualcomm Incorporated

Agenda item:	7.5.3

Release:	Rel-19

WI: 	FS_NR_FR1_DL_Frag_Carrier

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

Ran4 approved a WF [7] on DL Fragmented carriers study SI [1]. In this paper we discuss some agreements in WF, UE CA configuration and architecture switching and related capabilities. We also discuss capability handling.

# 2. 	Discussion

## 2.1	UE architecture switching triggering

In the WF, the architecture switching is said “FFS on whether it is decided by UE itself or decided by network”. In our view, this is more complicated than it is decide either by UE or by network. First, considering this is decided by network:

For example for one band with NC DL CA deployment, for example CA_n3(2A), if one specific UE should support this configuration with one rx or two rx’s should not concern network. Network involvement in UE architecture switching may make sense if network has desire to configure the UE to its envelope capability when this UE needs to support this non-contiguous CA configuration with one rx chain, i.e. UE needs to receiver two CCs on same band with same RX chain.

Proposal 1: Define UE envelope CA configuration as the highest order CA configuration where UE declares that it supports the non-contiguous CA configuration with one receiver chain

Observation: By configuring UE to and from this envelope capability, network indirectly decides UE architecture switching.

Network can do the decision based on regular channel quality reports since it has information on the measurements from the CCs to be configured before it is configuring them. This is normal process for adding CC, network needs to know if the CC is available for this UE. After this non-contiguous intra-band CA configuration has been contigured and activated, and if the channel quality becomes bad in one of the CCs, then network can decide to relace that CC or an other CC and again indirectly trigger an architecture switch to UE receiving the non-contiguous intra-band CA configuration with two receiver chains if network releases and other CC from an other band.

For any other case, network should not be burdened by UE architecture switching.

Proposal 2: Only case when network is involved in architecture switching is when it configured or de-configured UE with envelope CA configuration.

## 2.2	CA configuration switching

This is discussed also in the previous section. CA configuration switching is as agreed in WF [7] is decided by network and it is proposed not to add ne mechanisms that the current configuration based mechanism to the 3GPP framework. UE should not make independent decision on configuring and releasing CC in a CA configuration even based on indirect indications such as measurement reports from aggressor levels.

Proposal 3: Current procedures are sufficient for CA configuration switching and no new mechanisms are added

## 2.3	Triggering condition

Regarding how architecture switching is triggered, based on discussion and proposals in previous sections, only external triggering condition is configuration of envelope CA configuration.

Proposal 4: Only external triggering condition is configuration and de-configuration of envelope CA configuration

Before network makes the decision to configure the envelope CA configuration, it has access to neighbour cell reports and it can determine if the CC is viable for this UE.

Internal trigger conditions should be left for the UE. UE can determine based on its own CQI/RSRP and such  measurement information if the configured CCs on same band can be supported by single RX chain or not.

Proposal 5: If UE is configured for non-contiguous intra-band DL CA for a configuration that lower order than its envelope CA configuration, it is up to the UE to choose to use single RX and two RX chains

## 2.4	Capabilities

As discussed, there would need to be an indication in CA configuration when UE supports a non-contiguous DL CA with single receiver. This indication should be declared only when UE cannot support the CA configuration with two receiver chains.

Proposal 6: Indicate the need to support non-contiguous DL CA configuration with single receiver chain only in the highest order envelope configuration when UE is not able to support that configuration with two receiver chain for non-contiguous intra-band part

# Conclusion

We discussed, observed and proposed:

Observation: By configuring UE to and from this envelope capability, network indirectly decides UE architecture switching

Proposal 1: Define UE envelope CA configuration as the highest order CA configuration where UE declares that it supports the non-contiguous CA configuration with one receiver chain

Proposal 2: Only case when network is involved in architecture switching is when it configured or de-configured UE with envelope CA configuration Proposal 3: Current procedures are sufficient for CA configuration switching and no new mechanisms are added

Proposal 4: Only external triggering condition is configuration and de-configuration of envelope CA configuration

Proposal 5: If UE is configured for non-contiguous intra-band DL CA for a configuration that lower order than its envelope CA configuration, it is up to the UE to choose to use single RX and two RX chains

Proposal 6: Indicate the need to support non-contiguous DL CA configuration with single receiver chain only in the highest order envelope configuration when UE is not able to support that configuration with two receiver chain for non-contiguous intra-band part

# Reference

[1]	RP-243002, “Revised SID: Study on NR FR1 DL fragmented carriers”, MediaTek Inc., 3GPP TSG RAN Meeting #106, Madrid, Spain, Dec 09-12, 2024

[2]	R4-2420519, “WF on NR FR1 DL fragmented carriers study”, MediaTek, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[3]	R4-2502873, “WF on fragmented DL”, MediaTek, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[4]	R4-2502925, “TP for TR38.755”,  MediaTek Inc., Nokia, 3GPP TSG-RAN4 Meeting #114Bis, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2502949, “Impacts on UE RF requirements and DL performance impacts”, MediaTek Inc. 3GPP TSG-RAN4 Meeting #114Bis, Athens, Greece,  16th – 21st February, 2025

[6]	R4-2503557, “UE performance in one RX mode”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[7]	R4-2505109, “WF on fragemented DL”, MediaTek, Apple, Qualcomm, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[8]	R4-2503556, “Procedures for fragmented carriers”, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[9]	R4-2503558, “TP for TR38.755: Company C evaluation results”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

# Appendix: WF [7] agreements

#### Issue 2-2: Discussion on semi-statically switch hardware resources procedure and fallback considerations

- Further discussions on

  - CA fallback with/without UE architecture switching

    - Consider the fall back

      - from high order CA to lower order CA

      - from CA to single carrier mode

    - Decided by network

  - UE architecture switching:

    - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA

    - FFS on whether it is decided by UE itself or decided by network

      - FFS on whether the additional indication from UE side is needed

#### Issue 2-3: Triggering condition to enable ”One Rx RF chain”

RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode:

- Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

- Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

- Note: detailed conditions to enable/disable can be further discussed and decided separately