3GPP TSG-RAN WG4 Meeting #115	R4-2506166

# St Julian’s, Malta, 19th – 25th May 2025

Title:	Switching time diagram

Source:	Qualcomm Incorporated

Agenda item:	7.7.3

Release:	Rel-19

WI: 	NR_LBCA_Sw-Core

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

For WI LB DL CA via switching, WF [9] was approved in RAN4#114bis. This paper discusses some aspect of the agreements in the WF and brings some RAN4 views to RAN1 agreements

# 2. 	Discussion

## 2.1	Switching diagram aspects

In RAN4#114bis many proposals for switching time diagrams were submitted. The purpose of the ON-OFF time masks in clause 6.3 is to indicate the position of the transient period with respect to the ON and OFF power requirements. The diagram can reflect other requirements too but it is important to clearly agree what requirement the picture reflects. Here we discuss what the picture can contain.

Switching gap location with respect to slot boundary - RAN1 agreed that the switching gap location is on the slot boundary [A].

Switching gap location from SDL to FDD carrier – RAN1 agreed that the switching gap is in the SDL carrier [D]. Placement for the FDD to SDL is still in discussion with all possible options. While drafting the figure, it should be noted that the switching gap and switching period are different things [C] and care should be taken which term is used.

Timing advance – The diagram can include the handling of the timing advance [B], [C]. So far RAN4 has no example of such case in TS even some nice ones were submitted in the Rel-18 TX Switching discussion. It should be noted that RAN1 uses term switching gap that contains the switching period and sufficient time to accommodate the TA. [C]

What to write as requirement for the case when TA is taken into account should be discussed. The TA advances the UL i.e. it is relevant for the FDD to SDL switch direction since if the TA is larger than estimated, the UE may end up in to the situation that it needs to transmit before DL has stopped. This would be considered as

Further thinking is needed how the mask is defined for receive side, like Issue 3 of WF [9] states. If the requirement is for the receiver for the switching gap is on symbol level or slot level is being discussed in RRM domain. Symbol level requirement will lead to more optimised solution. What to write as a requirement other than TX ON and OFF powers and transient periods in RF spec is unclear.

Observation 1: How to write the RX requirement for the timing diagram is unclear and should be left to RRM room discussion

It should be noted that the agreement [C] says the switching gap can be different to different direction of the switching but not that switching period is different.

## 2.2	Switching frequency

RAN1 made some agreement on the switching periodicity [E-1/2/3]. The unit of the switching pattern is 1 slot [E-1]  and from there we can extend that there is at most one switch per slot, assuming both bands are deploying same SCS. Agreement [E-3] says that the periodicity of 40 msec is atleast supported and [E-2] implies there will be an agreement on fastest periodicity.

Observation 2: RAN1 is discussing periodicity limits for the number of X switches per Y slots and X<Y.

There may not be need to discuss this aspect anymore in RAN4.

# Conclusion

We discussed the switching diagram aspects and what should be included and what not. We also discussed the limits how many switches should UE be prepared to do in a slot. We made two observations:

Observation 1: How to write the RX requirement for the timing diagram is unclear and should be left to RRM room discussion

Observation 2: RAN1 is discussing periodicity limits for the number of X switches per Y slots and X<Y.

# Reference

[1]	RP-243317, “New WID on low band carrier aggregation via switching”, Apple, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[3]	R4-2503016, “WF on low-low band switching”, Apple, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[4]	R4-2502877, “LS on switching periods for low-low band switching”, AT&T, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[5]	R4-2500492, “Initial aspects of RX Switching”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[6]	R4-2501572, “Time masks for LB-LB CA switching”, Ericsson, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[7]	R4-2500405, “NR_LBCA_Sw Considerations”, Nokia, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, February 17-21, 2025

[8]	R1-2502860, “Low band carrier aggregation via switching”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[9]	R4-2505223, “WF on UE RF requirements for LB CA via switching”, Apple, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

# RAN1#120bis agreements

[A] Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier.

[B] Agreement

For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell.

[C] Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW,

- Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch.

- Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed)

  - For the UL TA,

    - Switching gap for SDL to FDD switch ends at the end of slot on SDL

      - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA

    - FFS: whether it has specification impact

[D] Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap,

- For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier.

- For switching from FDD carrier to SDL carrier, downselect one of the following solutions

  - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier

  - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier

  - Alt 3: Switching gap is always assumed at the SDL carrier

[E-1] Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS).

[E-2] Agreement

- For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design

  - NW configures a periodicity, denoted as  slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2

  - The pattern starts from the beginning of SFN 0 of PCell

- Restriction of maximum X switch(es) within Y slot(s),

  - FFS: detailed value and relation of X and Y.

- FFS: other restriction

[E-3] Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms