3GPP TSG-RAN WG4 Meeting#115		R4-2506231

Malta, MT, May 19th -23rd, 2025

Agenda item:	7.26.3.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on RF requirements for CW

Document for:	Discussion

Introduction

In RAN4#114bis meeting, a WF [1] was agreed. This contribution continues to discuss remaining issues as follows.

### Issue 3-1: CW Output power - Agreement: - CW output power <=33dBm, and can be declared by manufacture. ### Issue 3-2-1: ON/OFF power - Agreement: - Define CW OFF power - Value TBD ### Issue 3-4: Phase noise - Agreement: - FFS on whether to define phase noise requirements or leave to implementation in Rel-19 WI. - Encourage companies provide phase noise profile ### Issue 3-5: Unwanted emission - Agreement: - FFS on unwanted emission boundary and corresponding values: - Option 1: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth - Option 2: FFS whether or not to define channel bandwidth for CW - Option 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4. Table 2.2-4: CW node spectrum emission mask Δf (kHz); Emission limit (dBm); Measurement bandwidth. 0-90; N/A. 90-270; -15; 30 kHz. 270-450; -20; 30 kHz. 450- fmax; -27; 30 kHz. - Other options are not precluded. ### Issue 3-6: Spurious emissions - Agreement: - FFS on spurious emissions boundary and corresponding values: - Option 1: The spurious emission of CW starts from the boundary of D2R channel bandwidth. - Option 2: The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as ΔF = 10 MHz - Other options are not precluded. ### Issue 3-8: CW others - Agreement: - CW transmission and A-IoT BS downlink data transmission are non-concurrent. - FFS whether or not introduce the synchronization requirement of 3 us between CW node and A-IoT BS

Discussion

- OFF power

According to the last meeting, OFF power needs to be defined. It is understood that CW and Base Station come from the same manufacturer. Therefore, for value of off power, we can reuse NR BS requirements, the requirements for transmitter OFF power spectral density shall be less than -85dBm/MHz per antenna connector.

Proposal 1: OFF power of CW can be -85dBm/ MHz.

- Phase noise

In current spec, there are no phase noise requirements. In addition, it is understood that CW and Base Station come from the same manufacturer. From the point of view of the manufacturer, which needs to improve the phase noise if the phase noise impact the SNR of D2R so as to affect the sensitivity of the BS receiver. Thus, in our understanding, it is fine to leave the phase noise to implementation in R19 WI.

Proposal 2: Leave the phase noise to implementation in R19 WI.

- Unwanted emission

Unwanted emissions consist of out-of-band emissions and spurious emissions. The out-of-band emissions limit is specified in terms of a spectrum emission mask (SEM) and an adjacent channel leakage power ratio (ACLR). We can assume the channel bandwidth of CW is 15 kHz due to CW is a single tone. We can use NB-IoT requirements as a reference, SEM and ACLR

Proposal 3: Channel bandwidth of CW is needed to define (i.e., 15 kHz)

Proposal 4: Unwanted emission can reuse NB-IoT requirements.

- Spurious emissions

It is well known that, ACLR requirement is aimed at the interference of the system itself, that is, if there is an adjacent carrier in the system, you can’t interfere with it. The main purpose of spurious emissions is to avoid interfere to other system. We can think that the frequency range of A-IoT system is the maximum of D2R channel bandwidth. In our understanding, the boundary of spurious emissions can be the maximum of D2R channel bandwidth. The spurious emissions can reuse NR requirements in Table 1.

Table1: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26GHz | -30dBm | 1MHz | 2 |
| NOTE 1: Applies for Band 22, Band 42, Band 43 and Band 48. NOTE 2: Applies for Band 46 and Band 47. |  |  |  |

Proposal 5: The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as the maximum of D2R channel bandwidth.

Conclusions

In this paper, we provide our views on RF requirements for CW, we have made the following proposals.

Proposal 1: OFF power of CW can be -85dBm/ MHz.

Proposal 2: Leave the phase noise to implementation in R19 WI.

Proposal 3: Channel bandwidth of CW is needed to define (i.e., 15 kHz)

Proposal 4: Unwanted emission can reuse NB-IoT requirements.

Proposal 5: The boundary of spurious emissions can be the maximum of D2R channel bandwidth.

Reference

- R4-2505097, WF on requirements for A-IoT BS and CW, Huawei, TSG RAN-4 meeting #114bis