# 3GPP TSG-RAN WG4 Meeting #115                                                                R4-2506234

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | - | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS36.102 Introduction of New band for 5G Broadcast Utilizing Geosynchronous Satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_band_5G_bcast_GSO-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of New band for 5G Broadcast Utilizing Geosynchronous Satellite requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new chapter 7.6.C to support New band for 5G Broadcast Utilizing Geosynchronous Satellite Blocking characteristics. - Add a new chapter 7.7.C to support New band for 5G Broadcast Utilizing Geosynchronous Satellite Spurious response characteristics. - Add a new chapter 7.8.C to support New band for 5G Broadcast Utilizing Geosynchronous Satellite Intermodulation characteristics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New band for 5G Broadcast Utilizing Geosynchronous Satellite requirements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6, 7.7, 7.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

7.6	Blocking characteristics

This clause is reserved.

Editor’s note: the additional blocking requirements for band 253 will be introduced following further feedback from ETSI and additional studies.

(unchanged part skipped)

7.6C	Blocking characteristics for LTE based 5G broadcast over GSO satellite

7.6C.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

7.6C.2	In-band blocking requirements for LTE based 5G broadcast over GSO satellite

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.1.3 with parameters specified in Tables 7.6C.2-1 and 7.6C.2-2.

Table 7.6C.2-1: In band blocking parameters

| Rx parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| BWInterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5+0.0025 |
| FIoffset, case 2 | MHz | 12.5+0.0125 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.1.3. |  |  |

Table 7.6C.2-2: In-band blocking

| Operating band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 246 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

7.6C.3	Out-of-band blocking requirements for LTE based 5G broadcast over GSO satellite

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5C and subclause 7.6C.2 shall be applied.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.1.3 with parameters specified in Tables 7.6C.3-1 and 7.6C.3-2.

Table 7.6C.3-1: Out-of-band blocking parameters

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| NOTE: Reference measurement channel is specified in Annex A.1.3. |  |  |

Table 7.6C.3-2: Out of-band blocking

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 246 | Finterferer (CW) | MHz | -95 < f – FDL_low < -15 or 15 < f – FDL_high < 85 | -95 < f – FDL_low ≤ -120 or 85 ≤ f – FDL_high < 110 | 1 ≤ f ≤ FDL_low – 120 or FDL_high + 110 ≤ f ≤ 12750 |

For Table 7.6C.3-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration. For these exceptions the requirements of subclause 7.7C spurious response are applicable.

7.6C.4	Narrow band blocking for LTE based 5G broadcast over GSO satellite

Narrow-band blocking requirements are not applicable to LTE based 5G broadcast over GSO satellite.

# << Next change >>

7.7	Spurious response

This clause is reserved.

(unchanged part skipped)

7.7C	Spurious response for LTE based 5G broadcast over GSO satellite

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in subclause 7.6C.2 is not met.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.1.3 with parameters specified in Tables 7.7C-1 and 7.7C-2.

Table 7.7C-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| NOTE: Reference measurement channel is specified in Annex A.1.3. |  |  |

Table 7.7C-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

# << Next change >>

7.8	Intermodulation characteristics

Intermodulation response rejection is a measure of the capability of the receiver to receiver a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

(unchanged part skipped)

7.8C	Intermodulation characteristics for LTE based 5G broadcast over GSO satellite

The definition in clause 7.8 shall apply. The wide band intermodulation requirement is defined following the same principles using modulated E-UTRA carrier and CW signal as interferer.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.1.3 with parameters specified in Table 7.8C.1 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8C-1: Wide band intermodulation

| Rx Parameter | Units | PMCH bandwidth |
| --- | --- | --- |
|  |  | 10 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 6 |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| FInterferer 1 (Offset) | MHz | -BW/2 – 7.5 / +BW/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 1: Reference measurement channel is specified in Annex A.1.3. |  |  |

# << End of change >>