3GPP TSG-RAN WG4 Meeting #115	R4-2506252

St Julian’s, Malta, 19th – 23rd May 2025

# Source: 	ZTE Corporation

# Title: 	Discussion on power domain enhancements for single carrier

# Agenda item:	7.1.2.2.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on power domain enhancements for single carrier.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

  - 2.1 Approaches to enable MPR reduction

- Issue 1-1-1: RB based or CBW based extension - WF - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5 - The extended RBs are aligned with common RB grid - Issue 1-1-2: Fixed or flexible extension - Agreement in main session: - Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature - In addition, consider a limited number of additional fixed values with capability - Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - FFS in next meeting - Issue 1-1-4: Guard band for extended CBW - Options - Option 1: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band. - Option 2: NRB based extension and associated guard band: - maximum bandwidth extension: The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext. - flexible NRB-based extension: Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext - Option 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW. The GBs of extended UE CBW reuse the size of original GB of UE CBW. - Option 4: The GBs of the extended CBW shall be the remaining portion obtained by subtracting the RB numbers from the specified channel bandwidth when the extended CBW is selected from Table 5.3.2-1 in 3GPP TS 38.101-1. - Others - WF - FFS in next meeting

Based on WF in the last meeting [2], we have reached a consensus on extension ratios: for UEs supporting the MPR reduction feature, a default fixed value will be defined for both symmetrical and asymmetrical cases. Additionally, a limited number of alternative fixed values can be considered. Regarding extension granularity with RB based or CBW based, we can take the current signallings about bandwidth indicating as an example. There are cell-specific carrier bandwidth within SCS-SpecificCarrier broadcast in SIB1 and UE specific channel bandwidth indicated via ServingCellConfig. The two signallings are finally indexed to IE SCS-SpecificCarrier, which is shown in the following. The bandwidths indicated by the network are all in PRB granularity. For MPR reduction feature, it’s better to adopt the same approach to indicate the extended CBW.

Proposal 1: For MPR reduction feature, the extension can be RB based.

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=      SEQUENCE {

offsetToCarrier              INTEGER (0..2199),

subcarrierSpacing            SubcarrierSpacing,

carrierBandwidth             INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation  INTEGER (0..4095)                                       OPTIONAL            -- Need S

]]

}

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

To ensure RB alignment between the UE channel bandwidth and the extended CBW, we propose that RB based extension starts from the edge of the maximum transmission bandwidth configuration, which overlaps with the guard band of the UE channel bandwidth (as illustrated in Figure 2.1-1). In terms of the guard band for extended CBW, we propose not to define it. If the guard band of the extended CBW is defined, the extension will be RB based & CBW based, where the applying range of IBE and OOBE requirements is depicted in Figure 2.1-2. In this approach, IBE requirement will end at the last extended RB and SEM requirement will start from the edge of the extended CBW. There is no requirement applied in the guard band of the extended CBW. Since the extended RB is a virtual bandwidth, the part or all of extended bandwidth may be allocated to another UE. To ensure interference, it is important to apply emission requirement in the guard band of the extended CBW.

Proposal 2: RB based extension can start from the edge of the maximum transmission bandwidth configuration, which overlaps with the guard band of the UE channel bandwidth.

Proposal 3: Not to define the guard band for extended CBW.

Figure 2.1-1: RB based extension

Figure 2.1-2: Applicable emission requirement when guard band of the extended CBW is defined

For the new inner RB allocation in UE CBW, we can use a common approach to define it based on the extended UE CBW, which is the original NRB adding the extended RBs at each side. From the following picture, we can observe that new inner RB allocation (yellow within the red line) in UE CBW is the intersection of inner RB allocation in the extended UE CBW and UE RB allocation, in which UE RB allocation corresponds to the configured UE CBW.

Figure 2.1-3: New inner RB allocation in UE CBW based on the extended UE CBW

For the extended UE CBW, NRB,extended is NRB plus extended RBs at each side, and its inner RB allocation can be expressed by the following equation:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB,extended/2)

where ceil(x) is the smallest integer greater than or equal to x, NRB,extended is the maximum transmission bandwidth configuration of the extended channel bandwidth, RBStart,Low, RBStart, RBStart,High is relative to the extended UE CBW.

For the new inner RB allocation in UE CBW, RBStart should meet:

- RBshift  ≤  RBStart  ≤  NRB + RBshift

- where RBshift is the RB offset between the first RB of UE CBW and the first RB of the extended UE CBW.

- In addition, the new inner RB allocation should be under the hypotenuse of the right triangle of UE RB allocation, which can be expressed by the following equation:

- RBStart − RBshift + LCRB ≤ NRB

- where NRB is the maximum transmission bandwidth configuration of the configured UE channel bandwidth.

- The above can be deduced to

- RBStart ≤ NRB + RBshift − LCRB

- Based on the above analysis, the new inner RB allocation in UE CBW can be defined based on the extended UE CBW as follows:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

RBshift  ≤  RBStart  ≤  NRB + RBshift, and

- RBStart ≤ NRB + RBshift − LCRB, and

LCRB  ≤  ceil(NRB,extended/2)

where ceil(x) is the smallest integer greater than or equal to x, and NRB,extended is NRB plus extended RBs at each side.

Figure 2.1-4: New inner RB allocation in UE CBW for fixed extension at low side

For the asymmetrically fixed value 0.5*NRB, if the extension is at the low side as illustrated in Figure 2.1-4, the new inner RB allocation can be derived as follows:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = 3/2*NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

1/2*NRB ≤  RBStart  ≤  3/2*NRB, and

LCRB  ≤  ceil(3/4*NRB)

Figure 2.1-5: New inner RB allocation in UE CBW for fixed extension at high side

For the asymmetrically fixed value 0.5*NRB, if the extension is at the high side as shown in Figure 2.1-5, the new inner RB allocation can be derived as follows:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = 3/2*NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

0  ≤  RBStart  ≤  NRB , and

- RBStart ≤ NRB − LCRB, and

LCRB  ≤  ceil(3/4*NRB)

Proposal 4:

- For the asymmetrically fixed extension 0.5*NRB at the low side, the new inner RB allocation can be derived based on the extended CBW as follows:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = 3/2*NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

1/2*NRB ≤  RBStart  ≤  3/2*NRB, and

LCRB  ≤  ceil(3/4*NRB)

- For the asymmetrically fixed extension 0.5*NRB at the high side, the new inner RB allocation can be derived based on the extended CBW as follows:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = 3/2*NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

0  ≤  RBStart  ≤  NRB , and

- RBStart ≤ NRB − LCRB, and

LCRB  ≤  ceil(3/4*NRB)

- Proposal 5: If a limited number of additional fixed values are introduced, the new inner RB allocation in UE CBW can be defined in a common approach based on the extended UE CBW as follows:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

RBshift  ≤  RBStart  ≤  NRB + RBshift, and

- RBStart ≤ NRB + RBshift − LCRB, and

LCRB  ≤  ceil(NRB,extended/2)

where ceil(x) is the smallest integer greater than or equal to x, NRB,extended is NRB plus extended RBs at each side, and RBshift is the RB offset between the first RB of UE CBW and the first RB of the extended CBW.

  - 2.2 Applicable requirements

- Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW Issue1 Issue 2; Option1. Allocate the extended RBs from the GB of the original UE; Option2. Allocate the extended RBs from the edge of the UE CBW. Option1. Individual evaluation of the extended RBs & Horizontal line; Case1; Case2. Option2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line; Case3; Case4. Note: It is assumed that extended RBs are RB aligned with the UE and BS RBs for the listed cases.. - WF: - Case 1 and case 3 could be further down selected - GB of UE CBW may or may not be scheduled to other UEs - Extended IBE can be applied in the GB of UE CBW - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW - Issue 1-2-3: Derivation of the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - Option 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs. - Option 3: Others - WF - Discuss together with issue 1-2-2, and down select the options in next meeting. - Issue 1-2-4: Relaxation of the extended IBE - Proposals - Proposal 1: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. (Xiaomi) - Proposal 2: (vivo) - If the extended RBs are allocated from the GB of the original UE (case1/3), then the IBE in the GB should be relaxed to ensure the MPR of outer allocation reduces to the inner level: - Way 1: Skip the IBE evaluation for the extended RBs allocated in the GB. - Way2: Relax EVM in the IBE formula to that for BPSK for extended RBs. IBE_general = max(PRB - 30 dB, ) - WF - For case 1 in issue 1-2-2, use BPSK EVM in the IBE formula for all extended RBs - For case 3 in issue 1-2-2, FFS the relaxation

In terms of how to derive the extended IBE, case 1 and case 3 could be further down selected. For case 1, relaxation of the extended IBE is needed as the extended IBE requirement between the edge of the configured UE CBW and the edge of the extended CBW will be more stringent than the original SEM requirement for QPSK or 16QAM when UE CBW is full RB allocation. The way forward in last meeting is using BPSK EVM in the IBE formula for all extended RBs to relax IBE in extended CBW.

The aim of R19 power domain enhancement is to convert outer MPR to inner MPR through relaxing applicable ACLR/SEM/SE requirement because ACLR/SEM is mainly limiting factor to relax MPR for outer RB allocation. Since IBE is not the limiting factor to relax MPR of outer RB allocation, there is no need to further relax IBE requirement in the extended CBW. Comparing with case 1, case 3 is more preferred.

- Proposal 6: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB with NRB’= NRB + extended RBs as below functions

max−25−10⋅log10(NRB'),
20⋅log10EVM−3−5⋅(ΔRB−1)/LCRB,
−57dBm+10log10SCS/15kHz−PRB

  - 2.3 Signalling

- Issue 1-3-1: NW signaling - WF - FFS in next meeting - Issue 1-3-2: UE capability - Agreement in main session: - An optional UE capability to indicate whether it supports the CBW extension based MPR reduction feature - To discuss the following aspects - Whether to have separate UE capabilities for scenario 1 and scenario 2 - Whether to indicate multiple values of fixed extension capability

For MPR reduction signalling, network needs to ensure the system performance without degradation on adjacent system and confirm whether the current scenario can apply MPR reduction scheme. Hence, it is necessary for network to enable the MPR reduction feature. Since multiple values of fixed extension may be introduced, the network also needs to indicate maximum allowed extension based on the actual scheduling environment.

Proposal 7: Network needs to enable the MPR reduction feature and indicate maximum allowed extension.

For UE capability, there is no need to have separate UE capabilities for scenario 1 and scenario 2 if network can enable the feature. It’s better to indicate multiple values of fixed extension capability to allow more extension opportunities. UE needs to indicate the extension ratio at each side, and the extension ratio at each side can be different.

Proposal 8: No need to have separate UE capabilities for scenario 1 and scenario 2.

Proposal 9: To indicate multiple values of fixed extension capability and the extension ratio at each side needs to be indicated and can be different.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: For MPR reduction feature, the extension can be RB based.

Proposal 2: RB based extension can start from the edge of the maximum transmission bandwidth configuration, which overlaps with the guard band of the UE channel bandwidth.

Proposal 3: Not to define the guard band for extended CBW.

Proposal 4:

- For the asymmetrically fixed extension 0.5*NRB at the low side, the new inner RB allocation can be derived based on the extended CBW as follows:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = 3/2*NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

1/2*NRB ≤  RBStart  ≤  3/2*NRB, and

LCRB  ≤  ceil(3/4*NRB)

- For the asymmetrically fixed extension 0.5*NRB at the high side, the new inner RB allocation can be derived based on the extended CBW as follows:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = 3/2*NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

0  ≤  RBStart  ≤  NRB , and

- RBStart ≤ NRB − LCRB, and

LCRB  ≤  ceil(3/4*NRB)

- Proposal 5: If a limited number of additional fixed values are introduced, the new inner RB allocation in UE CBW can be defined in a common approach based on the extended UE CBW as follows:

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB,extended – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

RBshift  ≤  RBStart  ≤  NRB + RBshift, and

- RBStart ≤ NRB + RBshift − LCRB, and

LCRB  ≤  ceil(NRB,extended/2)

where ceil(x) is the smallest integer greater than or equal to x, NRB,extended is NRB plus extended RBs at each side, and RBshift is the RB offset between the first RB of UE CBW and the first RB of the extended CBW.

- Proposal 6: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB with NRB’= NRB + extended RBs as below functions

max−25−10⋅log10(NRB'),
20⋅log10EVM−3−5⋅(ΔRB−1)/LCRB,
−57dBm+10log10SCS/15kHz−PRB

Proposal 7: Network needs to enable the MPR reduction feature and indicate maximum allowed extension.

Proposal 8: No need to have separate UE capabilities for scenario 1 and scenario 2.

Proposal 9: To indicate multiple values of fixed extension capability and the extension ratio at each side needs to be indicated and can be different.

# 4 Reference

- RP-230828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei

- R4-2505099, WF on power domain enhancements, Huawei, HiSilicon

- R4-2503837, Discussion on UE RF power enhancement for NR single carrier, vivo

- R4-2501832, On power domain enhancements for single carrier, Huawei, HiSilicon