3GPP TSG-RAN WG4 Meeting # 115	R4-2506297

Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of UE-initiated/event-driven beam management

Agenda item:	7.24.4.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 MIMO operation [1], following objective was captured:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we provide our views on RRM core requirements for UE-initiated/event-driven beam reproting

Discussion

Define measurement period in Event triggered measurement reporting delay

In the previous meeting, measurement period for reporting delay has been discussed, and following has been agreed:

#### Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - For T_RS (TSSB or TCSI-RS) in L1_measurement periods - T_RS (TSSB or TCSI-RS): - the value of current beam and new beams are the same. - For M in L1_measurement periods: - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam - If timeRestrictionForChannelMeasurement is not configured, - Further discuss on: - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used - Option 2: No requirements even the NW can configure it. - Option 3: FFS - For P (sharing factor) in L1_measurement periods: - For SSB based or CSI-RS based, - The definition is the same as legacy - The value can be the same or different for the current beam and new beam

In last meeting, RAN4 has discussed the measurement period when timeRestrictionForChannelMeasurement is configured, and achieved agreement based on the most recent result from both new beam and current beam. It means that requirements defines which sample to be measured in the measurement period if a single sample is measured.

When timeRestrictionForChannelMeasurement is not configured, a UE is required to measure M=3 samples. In general, the UE have freedom to measure the RS within a measurement period. The UE is required to meet accuracy requirement in the measurement period. In that sense, Option 1 aligns with the principle of legacy requirement. If Option 1 is applied, the UE selects which samples to be measured based on UE implementation withour any restriction, which seems natural. The same approach used for M=1 is not appropriate, since the UE is required to perform multiple measurement across multiple RS occasions, including every case is too complex to be captured in the requirement.

Proposal 1: If timeRestrictionForChannelMeasurement is not configured,

- Reuse the same values of legacy, measurement period is based on 3 sample without restriction

Reporting delay requirements

#### Issue 1-2: Components of Event triggered measurement reporting delay? Agreement: - The event triggered measurement reporting delay includes the Tfirst UL channel - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH Further discuss the two options and select one option in the next meeting: - The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK) - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

Remaining is how to determine the measurement period, if TL1_measurement period is different for current beam and new beam. The reason of differece comes form different P in TL1_measurement period. In the meausurement period, event evaluation is performed based on new beam or current beam. It is important to include both new beam and current beam, since the result of L1-RSRP is compared during the measurement period. In our view, Option1 ensures that containing both new beam and current beam in any cases. Thus, we prefer Option1.

Proposal 2: For TL1_measurement period, Option1 (maximum of the L1 measurement periods) is preferred

Time window based or cover both no time window based and with time window based

#### Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19? <Way forward > - Option 1: (Qualcomm, Xiaomi, Apple, LGE) - Do not define requirements for eventInstanceCount-r19 > 1 - Option 1a: (Apple): To extend the requirements for case when eventInstanceCount-r19 > 1 is configured, add note longer delay is expected if eventInstanceCount-r19 > 1 is configured. - Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19 - Option 3: (Nokia, Ericsson) - Wait for further RAN1 conclusion - For how to specify the requirements in option 2 above: - Option 2a: - Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where, - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - C: event instances threshold is configured by NW. - Ttime_window: time window is configured by NW. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. - Other options are not precluded

According to RAN1 agreement, Event 2 can be no-time-window-based, or time-window-based with configurable M. For measurement before event triggering, it is reasonable to count multiple instance to avoid ping-pong effect. On the other hand, during measurement period for evaluation, where the event is already triggered, the reason is not clear to fulfil multiple counts. What UE needs to do is to evaluate the current and new beam, and confirm, which requires a no-time-window-based measurement period. So we think specifying only no-time-window based is enough.

Proposal 3: Specify only no-time window based requirment

Whether to specify new unified TCI switching delay requirements?

<Way forward > - For MAC-CE based DL/UL TCI state switch delay - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson) - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management - Option 2: (ZTE) - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc - Option 3: (MediaTek) - FFS on whether to remove Tfirst-SSB+TSSB-proc - For DCI based DL/UL TCI state switch delay - Option 1: - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management - Option 2: - Whether to further reduce the delay, wait for further RAN1 progress Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded

In RAN1#119 meeting, reducing beam application latency after a UE-initiated/event-driven beam reporting was discussed. The issue is to whether to reduce activated TCI state switching delay by removing additional SSB reception (Tfirst-SSB) for reducing the unified TCI state beam application time. In RAN1#120bis meeting, RAN1 try to draw conclusion, but there was no concensus. Although the agreement was not made, the basic assumption is that the UE stores the QCL properties of the SSB associated with the refrecne signal reported in the UE initiated beam report.

In our understanding, RAN1’s assumption can be interpreted as ‘target TCI state is known’. If target TCI stae is known, MAC-CE based TCI state switching delay requriement for downlink/uplink unified TCI state in 38.133 – 8.15.3, and 8.16.3 is capture as below:

8.15.3 MAC-CE based downlink TCI state switch delay … If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ where THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]; - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state - TSSB-proc = 2 ms; - TOk = 1 if target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise. … 8.16.3 MAC-CE based uplink TCI state switch delay … For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell, - If target TCI state is known, - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2 ms) / NR slot length. … Where, - THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. - NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise. …

As highlighted in the downlink TCI state switch delay, whether applying Tfirst-SSB + TSSB-proc is determined by TOk, and TOk is affected by target TCI state and the activated TCI state list for PDSCH/PDCCH. Uplink TCI state switch delay is also similar to downlink. The first Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2 ms is afftected by NM, which is based on target PL-RS.

We are not sure RAN1 acknowledges such conditions. If current TCI state delay is reused, there can be a case when SSB or PL-RS needs to be checked, which is not expected from RAN1 perspective. Thus, if RAN1 agrees to reduce MAC-CE based MAC-CE based TCI state switching delay, specification impact is expected.

On the other hand, for DCI based TDI state switching delay, the existing requirements for TCI state switch delay can be reused

Proposal 4: For unified TCI switching delay requirements, RRM impact is expected when RAN1 agrees to introduce reduced TCI state switch delay

Conclusion

In this contribution, we provide views on RRM requireiments for UE-initiated/event-driven beam reproting, and we propose

Proposal 1: If timeRestrictionForChannelMeasurement is not configured,

- Reuse the same values of legacy, measurement period is based on 3 sample without restriction

Proposal 2: For TL1_measurement period, Option1 (maximum of the L1 measurement periods) is preferred

Proposal 3: Specify only no-time window based requirment

Proposal 4: For unified TCI switching delay requirements, RRM impact is expected when RAN1 agrees to introduce reduced TCI state switch delay

Reference

[1] RP-242394, “WID revision: NR MIMO Phase 5,” Samsung