3GPP TSG-RAN WG4 Meeting #115			R4-2506329

St Julian’s, Malta, 19-23 May 2025

Agenda item:	5.8.1

Title:	Discussion on UE RF A-MPR for B254

Source:	Nordic Semiconductor ASA

Document for:	Discussion

1. Introduction

TS 36.102 is uncomplete. A-MPRs are set to TBD.

6.2B.3 UE additional maximum output power reduction for category NB1 and NB2 UE Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise. For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1. Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE Network Signalling value; Requirements (subclause); E-UTRA Band; A-MPR (dB). NS_01; 6.5B.4.2; Table 5.2-1; N/A. NS_02N; 6.5B.4.4.2; 255; N/A. NS_03N; 6.5B.4.4.4; 254; N/A. NS_04N; 6.5B.4.4.5; 254; TBD. NS_05N; 6.5B.4.4.6; 254; TBD. NS_24; 6.5B.4.4.3; 256; PC3; PC5. ≤ 3.5; ≤ 0.5.

2. Discussion

For the stricter NS_05N requirement, to accommodate the NB-IoT UE implementations, based on our measurements, and for the edge 15kHz tone of b254, the A-MPR required is 20dB and for NS_04N 3dB less, i.e. 17dB

Proposal-1: A-MPR for NS_05N is 20dB and for NS_04N is 17dB.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 | ≤ 17 |  |
| NS_05N | 6.5B.4.4.6 | 254 | ≤ 20 |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |

Alternative is to follow the concept of ETSI called nominated bandwidth. The ETSI requirement [2] for unwanted emissions within the band 1610 MHz to 1628.5 MHz applies for the nominated bandwidth which is defined in [2] to be specified by the applicant. Therefore, RAN4 should not make the generalization of defining “nominated bandwidth” equals “channel bandwidth” If a UE declares channel BW to be X, such UE shall not access channels which are X/2 at the edge of band, i.e. 1616-1626.5 MHz. If a UE declares nominated bandwidth of 400kHz, the standard NB-IoT SEM from Table 6.5B.3.2-1 from TS36.102 can apply. Because at 100kHz Emission limit is -5dBm which matches NS_05N. Therefore, we propose to keep NS_04N and NS_05N requirement applicable only to UEs that declare nominated channel bandwidth to be the same as channel bandwidth.

Proposal 2: Introduce the alternative way to fulfil the ETSI requirements by introducing reference to nominated channel bandwidth. Update the TS36.102 as the following:

6.5A.3.3.2	Requirements for network signalling value "NS_04N"

When "NS_04N" is indicated in the cell and UE declares nominated channel bandwidth [2] for the band to be the same as channel bandwidth, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.2-1 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5A.3.3.2-1: Additional requirements for "NS_04N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -2 | 30kHz |
| ± 160-2300 | -2 to -26 |  |
| ± 2300-18500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

6.5A.3.3.3	Requirements for network signalling value “NS_05N"

When "NS_05N" is indicated in the cell and UE declares nominated channel bandwidth [2] for the band to be the same as channel bandwidth, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-1 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5A.3.3.3-1: Additional requirements for "NS_05N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -5 | 30kHz |
| ± 160-225 | -5 to -8.5 |  |
| ± 225-650 | -8.5 to -15 |  |
| ± 650-1365 | -15 |  |
| ± 1365-1800 | -23 to -26 |  |
| ± 1800-16500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

3. Conclusion

Proposal-1: A-MPR for NS_05N is 20dB and for NS_04N is 17dB.

Proposal-2: Introduce an alternative way to fulfil the ETSI requirements, by introducing reference to nominated channel bandwidth in ETSI harmonized standard.

4. References

[1] Radio regulations, ITU, 2024

[2] ETSI EN 301 441