# 3GPP TSG-RAN WG4 Meeting #115	R4-2506337

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.25.4.2

Source:	China Telecom

Title:	Discussion on RRM impacts for SBFD operation for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, RRM impacts for SBFD operation for evolution of NR duplex operation was discussed and the WF was approved [1].

In this paper, we present our views on RRM impacts for SBFD operation for evolution of NR duplex operation issues captured in the WF [1] and summary [2].

- Discussion

RRM impacts of SBFD operation

Status in the WF [1]:

- Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - Agreements - For L1-RSRP and L1-SINR - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network - For L1-RSRP and L1-SINR measurement period, further discuss the following options. - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. - Option 2: other - For RLM/BFD/CBD, further discuss the following options - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type - Existing measurement period apply - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

For CSI-RS based L1-RSRP and L1-SINR measurement with different symbol types, in our views, the R16 NR-U approach can be reused, the measurement period is extended by excluding the resource occasions on the invalid symbol type.

Proposal 1: For CSI-RS based L1-RSRP and L1-SINR measurement with different symbol types, the measurement period is extended by excluding the resource occasions on the invalid symbol type.

Status in the summary [2]:

- Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Proposals for requirements for DUD case - Proposal 1 (CATT, Nokia, E///, ZTE): - RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells. - UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell. (CATT, ZTE) - RAN4 can send LS to RAN2 to ask for supporting the configuration (E///) - RE-use the conclusion of L1-RSRP/SINR CSI-RS resource in different symbol types (ZTE) - Proposal 2 (Samsung, CMCC, HW): - For DUD case, use the same principle as agreed in last meeting for DU case - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS. (HW) - Proposal 3 (MTK): - RAN4 shall deprioritize CSI-RS based L3 measurements. - Proposals for collision between CSI-RS resource and dynamic UL transmission - Proposal 1 (CATT): - Measurement is prioritized over UL transmission - Proposal 2 (ZTE): - For serving cell CSI-RS based L3 measurement, apply same principle as that for CSI-RS based L1 measurement. - For neighbour cell CSI-RS based L3 measurement, follow existing scheduling restriction to handle the collision. - Recommended WF - Discuss the following options on principles for defining requirements for DUD case - Option 1: define requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells - Option 2: same as DU case, UE measurement requirements would be same as in legacy under certain conditions - Discuss the following options for collision between CSI-RS resource and dynamic UL transmission - Option 1: existing scheduling restriction apply, i.e. measurement is prioritized - Option 2: Serving cell measurement is de-prioritized and neighbour cell measurement is prioritized over dynamic UL transmission.

For CSI-RS based L3 measurement for DUD case, we think there is no big difference for DUD case and DU case, same principle as DU case can be used for DUD case, UE measurement requirements would be same as in legacy under certain conditions.

Proposal 2: For CSI-RS based L3 measurement for DUD case, same principle as DU case can be used for DUD case.

Status in the summary [2]:

- Issue 2-1-3: Requirements for scheduling restriction - Proposals related to UL/DL transition - Proposal 1 (CATT, Nokia, E///, CMCC, HW, CTC, MTK): - Do not define new scheduling restriction requirements for DL/UL transition. - Proposals related to inter-frequency SSB measurement without gap - Proposal 1 (vivo, MTK): - For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed - the scheduling restriction in section 9.3.9.4 can be used as a reference (vivo) - UE is not required to receive DL in DL subbands on SBFD symbols (MTK) - Proposal 2 (CATT, Nokia, Samsung, HW): - RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point. - Recommended WF for UL/DL transition - It is suggested to agree on P1. - Recommended WF for inter-frequency SSB measurement outside MG - FFS whether additional scheduling restriction applies for inter-frequency SSB measurement outside MG on SBFD symbols, when the SSB is fully within DL active BWP but not completely contained in DL sub-band. - Option 1: yes, UE is not required to receive DL in DL subbands on SBFD symbols - Option 2: no, UE is required to receive DL in DL subbands on SBFD symbols

For scheduling restriction requirements for UL/DL transition, we support not to define new scheduling restriction requirements for DL/UL transition.

Proposal 3: RAN4 scheduling restriction requirement is not needed for DL/UL transition.

- Conclusion

The contribution presents our views on RRM impacts for SBFD operation for evolution of NR duplex operation, with the following proposals:

Proposal 1: For CSI-RS based L1-RSRP and L1-SINR measurement with different symbol types, the measurement period is extended by excluding the resource occasions on the invalid symbol type.

Proposal 2: For CSI-RS based L3 measurement for DUD case, same principle as DU case can be used for DUD case.

Proposal 3: RAN4 scheduling restriction requirement is not needed for DL/UL transition.

- References

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.

- R4-2503623, Topic summary for [114bis][213] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.