3GPP TSG-RAN WG4 Meeting #115		R4-2506422

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.27.3.1

Source: 	CMCC

Title: 			  Discussion on RRM requirement of OD-SSB for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 mainly discussed fast mode and Case 2 measurement procedures and requirement definitions about OD-SSB in NES enhancement WI. The WF has been approved in [1]. In this paper, we further discuss OD-SSB related open issues.

- OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 1: Lower bound for measurement in FMW

Agreement

Introduce UE capability on whether a lower bound is needed, i.e.,

  - The lower bound is not needed (default value), or,

  - A lower bound is needed and based on OD-SSB periodicity of [10] ms

    - Further confirm whether [10] is feasible, and only one value

- In last meeting, considering 5ms L3 measurement sampling periodicity is not the default UE capability in earlier release, we compromised to introduce the UE capability about whether a lower bound is needed.

- Typically, in RRC_IDLE and RRC_Inactive Mode, processing 20ms SSB periodicity is the default UE capability. Then, in RRC_Connected Mode, higher capability is expected, 10ms is reasonable as the value for lower bound.

Proposal 1: For the UE that requires a lower bound for measurement in FMW, define 10ms as the lower bound value.

Sub-issue 3: OD-SSB in FMW collided with MG

Wayforward

- FFS UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

To avoid the impact on MG based measurement, UE shall be allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

Currently, the length of FMW is Tuncertain + min(Treport , Tidentify), and Tidentify equals the minimum RAN4 OD-SSB measurement period requirement as follow.

- A detected cell: OD-SSB based measurement period

- A newly detectable cell: OD-SSB based cell identification period

We think the above OD-SSB skipping behavior has already been captured in the definition of Tidentify, in which ‘Kp’ scaled the measurement period with the consideration of measurement occasion collides with MG.

Proposal 2: UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion, and this has been considered in the ‘Kp’ factor of Tidentify when define the FMW length.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

- Background

  - The deactivated SCell measurement requirement in FR1 is as follow.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

  - The OD-SSB based deactivated SCell measurement procedure is shown as follow.

Sub-issue 1: The start point of T2

Wayforward

The start point of T2 is

- Option 1:

  - The time point of OD-SSB deactivation

- Option 2:

  - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify.

- Option 2a:

  - The time point of OD-SSB activation

One of the initial purpose for introducing the T2 was to prevent frequent fast measurement of UE. Therefore, the most straight forward is to control the time gap between two FMWs. We propose to align the starting point of T2 with the starting point of  FWM, which is the time point of OD-SSB activation indication. After T2, UE can start another FMW.

Meanwhile, by such design, network knows very clearly about when the OD-SSB re-activation can be fast measured by UE again, furthermore, helping network to schedule the OD-SSB transmission.

Proposal 3: Define the starting point of T2 as the starting time point of FMW, which is the time point of OD-SSB activation indication.

Sub-issue 2: The minimum interval(T2) between multiple FMWs

Wayforward

- Option 1: 5* measCycleSCell

- Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity)

- Option 1b: 5*max (measCycleSCell, DRX cycles)

- Option 2: Minimum measurement period defined in existing spec.

- Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1

Then, the known condition period can be reused as the T2 length. Such design can guarantee the known state of deactivated SCell if UE measure it with FMW, and guarantee no additional power consumption from fast measurement.

Proposal 4: Define the length of T2 as the known condition period. 5*max (measCycleSCell, DRX cycles) for FR1 and 4s for FR2.

Sub-issue 3: UE’s behaviour when OD-SSB further activation/reconfiguration within T2

Wayforward

- Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

- Option 2: The UE is assumed to perform fast mode measurement without cell identification period.

- Option 3: No requirement for UE within T2.

For move forward, we can compromise to Option 1, to make the T2 design serve for UE power saving demand.

Proposal 5: If OD-SSB further activation/reconfiguration within T2, UE is not required to perform ‘fast measurement’ but follows legacy deactivated SCell measurement requirement.

- OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-2-3: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: Define the requirement based on OD-SSB periodicity.

- Option 2: Not to define requirement.

The definition of Alt Time-C2 is:

Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

Firstly, we think it is rather complicated to define the requirement based on the uneven distributed union of AO-SSB and OD-SSB. Then, considering the OD-SSB periodicity is not larger than the AO-SSB periodicity, we think the requirement shall be defined based on OD-SSB periodicity to ensure the network power saving gain. The requirement we mentioned here including the measurement requirement in FMW and the SCell activation delay requirement.

Proposal 6: For Alt Time-C2 case, the measurement requirement in FMW and the SCell activation delay requirement shall be defined based on OD-SSB periodicity.

Issue 1-2-4: Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

Agreement

RAN4 to first discuss the RAN4 impact based on RAN2 LS R2-2503015.

- Option 1: RAN4 to define requirement for different frequencies

- Option 2: RAN4 not to define requirements for different frequencies

- The final LS from RAN2 is R2-2503019, we copy the contents below for information.

In RAN2#129b, RAN2 discussed how to configure L3 measurement when always-on SSB (A-SSB) and on-demand SSB (OD-SSB) have different center frequency, and made the following Working Assumption: Working assumption: - When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. Note: The WA was made from RRC signaling perspective. RAN2 relies on RAN4/1 to decide whether to support the scenario when A-SSB and OD-SSB have different center frequencies or not.

- Firstly, we think the scenario that AO-SSB and OD-SSB have different center frequency has been identified and confirmed by RAN1. RAN2 also introduce a new servingCellMO to indicate the MO of OD-SSB. It is able for RAN4 to define the requirement for this scenario 3.

- For the deactivated SCell measurement, we think the agreement for Case 2 can be reused here, with one additional clarification that if OD-SSB is triggered, and the new servingCellMO is configured, then the serving cell measurement shall be based on OD-SSB.

Proposal 7: For Scenario 3 that OD-SSB and AO-SSB are within different frequencies, the Case 2 deactivated SCell measurement procedure can be reused, and add one more note that the serving cell measurement shall be based on OD-SSB if OD-SSB is activated and the new servingCellMO is configured.

- As for the neighbour cell measurement on the carrier frequency of AO-SSB, at least in FMW, we think the neighbour cell measurement can be omitted. After the FMW, when UE perform serving cell measurement on OD-SSB carrier in slow mode, the neighbour cell measurement on the carrier frequency of AO-SSB can be treated as inter-frequency measurement without MG.

Proposal 8: For Scenario 3 that OD-SSB and AO-SSB are within different frequencies, when UE measure the OD-SSB in FMW, the neighbour cell measurement on AO-SSB carrier shall be skipped; when UE measure the OD-SSB in slow mode, the neighbour cell measurement on AO-SSB carrier can be treated as inter-frequency measurement without MG.

- For SCell activation delay, our analysis is under Issue 1-3-6.

- OD-SSB based SCell activation

Issue 1-3-2: The known cell condition – period of known condition

- Wayforward

Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

About the period of the known/unknown condition, in legacy FR1, the known case requires UE to sent a valid measurement report during a period before the reception of the SCell activation command, the period equal to max(5*measCycleSCell,  5*DRX cycles), which related to the legacy L3 measurement requirement.

- In other words, as long as UE reported the measurement report during max(5*measCycleSCell,  5*DRX cycles) before the SCell activation command, UE could maintain the known states of this deactivated SCell. From this point of view, we think this known/unknown condition can be reused even for OD-SSB case.

Proposal 9: Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

Issue 1-3-6: SCell activation detail requirement

- Based on the agreement from RAN1, the relationship between AO-SSB and OD-SSB is like:

- When AO-SSB is NCD-SSB:

- The frequency location of AO-SSB and OD-SSB is same

- The spatial relation of AO-SSB and OD-SSB is same

  - the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported

- The transmit power of AO-SSB and OD-SSB is same

- When AO-SSB is CD-SSB:

- The frequency location of AO-SSB and OD-SSB is different

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

- First of all, we think the known/unknown condition can be defined based on either AO-SSB or OD-SSB.

- Secondly, the SSB measured for known cell condition and the SSB used in SCell activation procedure can be same or different, and the SCell activation delay requirements shall be defined case by case.

- The current known SCell activation delay methodology can be reused for the following cases:

  - When the SSB measured for known cell condition and the SSB used in SCell activation procedure are the same

  - When the SSB measured for known cell condition is AO-SSB not on sync-raster, and the SSB used in SCell activation procedure is OD-SSB

  - When the SSB measured for known cell condition is OD-SSB, and the SSB used in SCell activation procedure is AO-SSB not on sync-raster

- Further study whether the AGC adjustment time should be included in the known SCell activation delay requirement.

  - When the SSB measured for known cell condition is always-on SSB on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, due to the frequency location is different

Proposal 10: The SSB measured for known cell condition and the SSB used in SCell activation procedure can be same or different.

Proposal 11: For SCell activation detail requirement:

- The current known SCell activation delay methodology can be reused for the following cases:

  - When the SSB measured for known cell condition and the SSB used in SCell activation procedure are the same

  - When the SSB measured for known cell condition is AO-SSB not on sync-raster, and the SSB used in SCell activation procedure is OD-SSB

  - When the SSB measured for known cell condition is OD-SSB, and the SSB used in SCell activation procedure is AO-SSB not on sync-raster

- Further study whether the AGC adjustment time should be included in the known SCell activation delay requirement.

  - When the SSB measured for known cell condition is always-on SSB on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, due to the frequency location is different

- OD-SSB general issues

Issue 1-4-1: Additional Processing time for OD-SSB reception – scenario

Issue 1-4-2: Additional Processing time for OD-SSB reception - value

- Background

In RAN4 #113 meeting, RAN4 achieved agreements as follow.

Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

- Information

RAN1 #120-bis Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. - If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ

Agreement

UE applies the additional processing timing only in deactivated SCell measurement.

- Note: The additional UE processing timing is not applied to SCell activation procedure.

The additional processing timing is T_processing, the value of T_processing:

Introduce different values with UE capability: [2 or 3]ms and 5ms.

In last meeting, companies discussed the UE capability about the additional processing time. If the new UE capability will be introduced, then 2ms shall be the starting point.

Meanwhile, we don’t think it is very important to introduce a new UE capability just for the OD-SSB time sequence. If one union value can be used, we think 3ms can be starting point, considering that 5ms from XR including DCI processing time and MG adjustment time.

Proposal 12: For UE additional processing time in deactivated SCell measurement, either introduce 2ms and 5ms with UE capability, or use one union 3ms for all UE.

Issue 1-4-3: Additional Processing time for OD-SSB deactivation

Wayforward

- FFS RAN4 to consider defining the same timeline for activating and deactivating OD-SSB

RAN1 sent a LS R1-2503108 about OD-SSB deactivation processing time, we copy it below for information:

RAN1 has made the following agreement for MAC-CE based deactivation of on-demand SSB (OD-SSB) transmission on SCell. Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification. - (Working assumption) T=T_min - RAN4 to confirm the above two working assumptions - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication RAN1 respectfully asks RAN4 if the two working assumptions can be confirmed to agreement.

For the case that time instance B’ doesn’t fall within an OD-SSB burst, time instance B = reception timing of OD-SSB deactivation +T_min, which T_min =  m+3Nslotsubframe,μ+1.

From RAN4 perspective, we think the working assumptions from RAN1 can be confirmed. No additional processing time is needed since UE is not required to process the upcoming measurement RS.

Proposal 13: For the case that time instance B’ doesn’t fall within an OD-SSB burst, confirm the working assumption of RAN1 that the time instance B = reception timing of OD-SSB deactivation +T_min, and the definition of T_min.

Issue 1-4-9: Interruption

Agreement

- New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation.

- New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command.

- The interruption can be applied in both Case 1 and Case 2.

- The detail condition, duration and position of the interruption is FFS.

- The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation.

- No new interruption requirement is defined.

- The interruption requirement follows the same principle of legacy SCell activation.

- For scenario 2, for case 1 and case 2 with AO-SSB not on the sync raster, one condition of applying such interruption is that the measCycleSCell equal or longer than 640ms.

Proposal 14: For Case 1 Scenario 2 and Case 2 Scenario 2 with AO-SSB not on the sync raster, the side condition of interruption shall include the measCycleSCell equal or longer than 640ms.

L1 and L3 measurement for Scenario 3B

- Agreement from RAN1#120bis

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- In Scenario#3B, for L1 measurement, after UE receiving the MAC-CE for updating the OD-SSB transmission parameter, the measurement shall be based on the legacy framework and updated OD-SSB.

- For L3 measurement, the measurement shall be based on the SMTC and DRX configuration. Therefore, RAN4 should wait for RAN2’s discussion about whether SMTC will be updated after receiving the MAC-CE.

Proposal 15: In Scenario#3B, after UE receiving the MAC-CE for updating the OD-SSB transmission parameter, the L1 measurement shall be based on the legacy framework and updated OD-SSB, the discussion of L3 measurement behavior can wait for RAN2’s input about SMTC design.

Issue 2-1-1: OD-SSB based multiple SCells activation

- Wayforward

- Option 1:

  - RAN4 to define OD-SSB based multiple SCells activation delay requirements.

    - The limitation is needed

    - It doesn’t impact the WI’s completion.

- Option 2:

  - Not define multiple OD-SSB carriers requirement in this release.

- To enlarge the application scenario of OD-SSB feature, we support to define OD-SSB based multiple SCells activation delay requirements. However, considering only two meetings left, we support to add some limitations on the applicable scenario to reduce the specification workload.

- For deactivated SCell measurement in multiple SCells scenario, OD-SSB fast measurement window among multi-OD-SSB carriers shall be non-overlapped with each other. Then, all of the measurement design for Case 1 and Case 2 can be reused.

- For multiple SCell activation, all of the to-be-activated SCells shall trigger OD-SSB transmission, then the legacy SCell activation delay requirement can be reused, just replace SMTC with OD-SSB periodicity.

Proposal 16: Define OD-SSB based multiple SCells activation delay requirements with some limitation to applicable scenario, including:

- OD-SSB fast measurement window among multi-OD-SSB carriers shall be non-overlapped with each other

- all of the to-be-activated SCells shall trigger OD-SSB transmission

- Conclusion

Based on the discussion above, the following proposals and observations are concluded.

Proposal 1: For the UE that requires a lower bound for measurement in FMW, define 10ms as the lower bound value.

Proposal 2: UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion, and this has been considered in the ‘Kp’ factor of Tidentify when define the FMW length.

Proposal 3: Define the starting point of T2 as the starting time point of FMW, which is the time point of OD-SSB activation indication.

Proposal 4: Define the length of T2 as the known condition period. 5*max (measCycleSCell, DRX cycles) for FR1 and 4s for FR2.

Proposal 5: If OD-SSB further activation/reconfiguration within T2, UE is not required to perform ‘fast measurement’ but follows legacy deactivated SCell measurement requirement.

Proposal 6: For Alt Time-C2 case, the measurement requirement in FMW and the SCell activation delay requirement shall be defined based on OD-SSB periodicity.

Proposal 7: For Scenario 3 that OD-SSB and AO-SSB are within different frequencies, the Case 2 deactivated SCell measurement procedure can be reused, and add one more note that the measurement shall be based on OD-SSB if OD-SSB is activated and the new servingCellMO is configured.

Proposal 8: For Scenario 3 that OD-SSB and AO-SSB are within different frequencies, when UE measure the OD-SSB in FMW, the neighbour cell measurement on AO-SSB carrier shall be skipped; when UE measure the OD-SSB in slow mode, the neighbour cell measurement on AO-SSB carrier can be treated as inter-frequency measurement without MG.

Proposal 9: Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

Proposal 10: The SSB measured for known cell condition and the SSB used in SCell activation procedure can be same or different.

Proposal 11: For SCell activation detail requirement:

- The current known SCell activation delay methodology can be reused for the following cases:

  - When the SSB measured for known cell condition and the SSB used in SCell activation procedure are the same

  - When the SSB measured for known cell condition is AO-SSB not on sync-raster, and the SSB used in SCell activation procedure is OD-SSB

  - When the SSB measured for known cell condition is OD-SSB, and the SSB used in SCell activation procedure is AO-SSB not on sync-raster

- Further study whether the AGC adjustment time should be included in the known SCell activation delay requirement.

  - When the SSB measured for known cell condition is always-on SSB on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, due to the frequency location is different

Proposal 12: For UE additional processing time in deactivated SCell measurement, either introduce 2ms and 5ms with UE capability, or use one union 3ms for all UE.

Proposal 13: For the case that time instance B’ doesn’t fall within an OD-SSB burst, confirm the working assumption of RAN1 that the time instance B = reception timing of OD-SSB deactivation +T_min, and the definition of T_min.

Proposal 14: For Case 1 Scenario 2 and Case 2 Scenario 2 with AO-SSB not on the sync raster, the side condition of interruption shall include the measCycleSCell equal or longer than 640ms.

Proposal 15: In Scenario#3B, after UE receiving the MAC-CE for updating the OD-SSB transmission parameter, the L1 measurement shall be based on the legacy framework and updated OD-SSB, the discussion of L3 measurement behavior can wait for RAN2’s input about SMTC design.

Proposal 16: Define OD-SSB based multiple SCells activation delay requirements with some limitation to applicable scenario, including:

- OD-SSB fast measurement window among multi-OD-SSB carriers shall be non-overlapped with each other

- all of the to-be-activated SCells shall trigger OD-SSB transmission

- Reference

- R4-2504906, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson