3GPP TSG RAN WG4 Meeting#115                                    R4- 2506472

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.22.3

Source:	vivo

Title:	Discussion on testing issues for CSI compression two-sided models

Document for:	Discussion

- Introduction

In R18 SI, we have studied on general aspects, specific issues related to use cases, and interoperability and testability aspects. The agreements have been collected in TR [1]. In Rel-19, the study on interoperability and testability for CSI compression continues in the WI [2].

In last meeting, we have the following agreements for CSI compression.

Topic 1: CSI compression: Simulation analysis for options 3 and 4 - Datasets Endorsed preliminary observations - Datasets consist of separate training data and separate test data. - All companies have produced datasets based on the same simulation assumptions, however differences can be observed between performance with different companies datasets. - The “mixed dataset” provides much more consistent results than training or testing using different individual company datasets - The exercise demonstrates that in order to agree on test decoder(s) and or reference encoder(s), some effort is needed to align dataset(s). - Option 3 track 1 Endorsed preliminary observations - The investigation on option 3 is based on using the agreed structure for the decoder and encoder - It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional - The performance of the encoder is very dependent on the alignment of the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testing dataset - The variation in the performance depending on the training datasets and test datasets can be as much as [20%] in SGCS. Further details are available in the results spreadsheet. - Preliminary observation; further results may be added/refined and the observation updated (for example, using the mixed dataset as in track 2) - For option 3, track 1 to be useful, careful attention would be needed to the alignment of datasets - Up to now, the same encoder structure is considered as used when creating the reference decoder; the impact of different encoder structure is not yet considered Further simulation activity for option 3 track 1 No further simulation activity is needed for option 3 track 1. - Option 3 track 2 Endorsed preliminary observations - There is reasonable SGCS convergence when companies train an encoder-decoder pair using the mixed dataset and test using the mixed testset - Around 1% variation in SGCS; see results spreadsheet for more details - Preliminary observation; further results may be added - It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional - When training “own” encoder with the frozen decoder and testing, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results from results presented so far - Around [+- 2%] variation in SGCS from the average; see results spreadsheet for more details - Preliminary observation; further results may be added - Up to now, the same encoder structure is considered as used when creating the reference decoder by most companies; the impact of different encoder structure is not yet fully considered Further simulation activity for option 3 track 2 - Definitions - Training Dataset 1 is the dataset used for training of the frozen decoder. - Training Dataset 2 is the dataset used for training of the new encoder based on the frozen decoder. - For the existing results above, Training dataset 1 = training dataset 2 = mixed training dataset. Test dataset = mixed test dataset. - Additional results 1 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test Dataset = Mixed test dataset. - Additional results 2 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test dataset = company specific test dataset. - Additional results 3 for RAN4#115: Further results using a lower complexity CNN or transformer encoder - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = mixed. Test Dataset = Mixed test dataset. - Encoder structure: As suggested in [R4-2505156] or company specific selection. Structure and complexity should be reported. - Option 4 Endorsed preliminary observations - For option 4b: It is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder when the mixed dataset is used - For option 4a: For the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b) Further simulation activity for option 4 - All interested companies are requested to upload their “own” decoders to the server as soon as possible. - “Own” decoder means a decoder trained using the Samsung Frozen encoder and the mixed training dataset. - Definitions - Dataset 1 = Dataset used for training the frozen encoder - Dataset 2 = Dataset used for training the own decoder (using the frozen encoder) - Dataset 3 = Dataset used for training the own encoder (using the own decoder) - First results priority results for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your company’s “own” decoder (this is a test for interoperability between encoders and test decoders) - Second priority results for RAN4#115: - Dataset 1 = Dataset 2 = Mixed training dataset - Dataset 3 = Company own dataset - Test Dataset: Mixed Dataset, also consider own test dataset if interested. - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - Results for consideration of lower complexity encoder for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Encoder: Lower complexity “own” encoder, examples are provided in [R4-2505156] - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - For a future meeting, a further consideration, for option 4b is to consider the frozen encoder with a company own dataset; i.e. Dataset 1 = Mixed training dataset, Dataset 2 = Dataset 3 = Company specific dataset. - Reduction of simulation results volume In order to simplify simulations, the following was agreed; - No more results are expected Option 3 track 1 - No more results are expected Unquantized model results - For option 3 track 2, use the Samsung decoder as the frozen decoder. Lower priority for Nokia decoder. - For option 4, use the Samsung encoder as the frozen encoder. Lower priority for Nokia encoder. Topic 2: CSI compression: Non-simulation issues - What needs to be captured or specified for Option 3 Agreements - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) - What needs to be captured or specified for Option 4 Agreement: - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. - Relation between RAN1 option 3 and RAN1 inter-vendor collaboration direction C Agreement: - FFS on the relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions.

In this contribution, we further provide our views on testing issues for CSI compression two-sided models.

- Discussion

  - Simulation results for Option 3 track 2

In this section, the simulation results of Option 3 track 2 will be provided. In last meeting, Samsung models are selected for further track 2 evaluations. The next steps of Option 3 are clear, which are shown as the following agreement.

Further simulation activity for option 3 track 2 - Definitions - Training Dataset 1 is the dataset used for training of the frozen decoder. - Training Dataset 2 is the dataset used for training of the new encoder based on the frozen decoder. - For the existing results above, Training dataset 1 = training dataset 2 = mixed training dataset. Test dataset = mixed test dataset. - Additional results 1 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test Dataset = Mixed test dataset. - Additional results 2 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test dataset = company specific test dataset. - Additional results 3 for RAN4#115: Further results using a lower complexity CNN or transformer encoder - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = mixed. Test Dataset = Mixed test dataset. - Encoder structure: As suggested in [R4-2505156] or company specific selection. Structure and complexity should be reported.

According to the agreement, we provide the additional results 1&2 of Option 3 for track 2, using Samsung models with quantization. From this results, it seem that the mismatch of training dataset 1 and training dataset 2 has not large performance impact.

Table 2.1-1. SGCS results of Option 3 for track 2 using Samsung models, with quantization.

| Option 3 for track 2 using Samsung models, with quantization |  |  |  |  |
| --- | --- | --- | --- | --- |
| Cases | Encoder | Decoder | Test dataset | SGCS results |
| Baseline | Own | Own | mixed dataset | 0.696 |
| Option 3 results in RAN4#114bis | Trained based on Samsung’s decoder, using mixed dataset | Samsung’s decoder | mixed dataset | 0.689 |
| Additional results 1 for RAN4#115 | Trained based on Samsung’s decoder, using vivo dataset | Samsung’s decoder | mixed dataset | 0.691 |
| Additional results 2 for RAN4#115 | Trained based on Samsung’s decoder, using vivo dataset | Samsung’s decoder | vivo dataset | 0.731 |

Observation 1: The additional results of Option 3 track 2 using of Samsung models with quantization, are shown in Table 2.1-1.

  - Simulation results for Option 4 track 2

In this section, the simulation results of Option 4 track 2 will be provided. The next steps of Option 3 are clear, which are shown as the following agreement.

Further simulation activity for option 4 - All interested companies are requested to upload their “own” decoders to the server as soon as possible. - “Own” decoder means a decoder trained using the Samsung Frozen encoder and the mixed training dataset. - Definitions - Dataset 1 = Dataset used for training the frozen encoder - Dataset 2 = Dataset used for training the own decoder (using the frozen encoder) - Dataset 3 = Dataset used for training the own encoder (using the own decoder) - First results priority results for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your company’s “own” decoder (this is a test for interoperability between encoders and test decoders) - Second priority results for RAN4#115: - Dataset 1 = Dataset 2 = Mixed training dataset - Dataset 3 = Company own dataset - Test Dataset: Mixed Dataset, also consider own test dataset if interested. - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - Results for consideration of lower complexity encoder for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Encoder: Lower complexity “own” encoder, examples are provided in [R4-2505156] - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - For a future meeting, a further consideration, for option 4b is to consider the frozen encoder with a company own dataset; i.e. Dataset 1 = Mixed training dataset, Dataset 2 = Dataset 3 = Company specific dataset.

According to the agreement, we provide the first priority results and second priority results of Option 4 for track 2, using Samsung models with quantization. First priority results are shown in Table 2.2-1. Second priority results are shown in Table 2.2-2 and Table 2.2-3. In these tables, by comparing the results of vivo’s “own” decoder and other companies’ “own” decoder, similar performance is observed. It is feasible for DUT to train it new encoder in Option 4. When paired with Huawei’s “own” decoder, Qualcomm’s “own” decoder and Samsung’s “own” decoder, even some performance gain is achieved.

Then we focus on the comparison between Option 3/4a/4b for track 2. It seems that Option 3, 4a and 4b have similar validation performance.

Table 2.2-1. Option 4 track 2 results, where Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset, Test dataset = mixed dataset.

| Option 4, Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset |  |  |  |
| --- | --- | --- | --- |
| Encoder | Decoder | Test dataset | SGCS results on mixed dataset |
| vivo’s “own” encoder | vivo’s “own” decoder | mixed dataset | 0.692 |
| vivo’s “own” encoder | Ericssion’s “own” decoder | mixed dataset | 0.689 |
| vivo’s “own” encoder | Nokia’s “own” decoder | mixed dataset | 0.684 |
| vivo’s “own” encoder | Huawei’s “own” decoder | mixed dataset | 0.702 |
| vivo’s “own” encoder | MTK’s “own” decoder | mixed dataset | 0.684 |
| vivo’s “own” encoder | Qualcomm’s “own” decoder | mixed dataset | 0.696 |
| vivo’s “own” encoder | Samsung’s “own” decoder | mixed dataset | 0.694 |

Table 2.2-2. Option 4 track 2 results, where Dataset 1 = Dataset 2 = Mixed training dataset, Dataset 3 = vivo dataset, Test dataset = mixed dataset.

| Option 4, Dataset 1 = Dataset 2 = Mixed training dataset, Dataset 3 = vivo dataset |  |  |  |
| --- | --- | --- | --- |
| Encoder | Decoder | Test dataset | SGCS results on mixed dataset |
| vivo’s “own” encoder | vivo’s “own” decoder | mixed dataset | 0.674 |
| vivo’s “own” encoder | Ericssion’s “own” decoder | mixed dataset | 0.674 |
| vivo’s “own” encoder | Nokia’s “own” decoder | mixed dataset | 0.667 |
| vivo’s “own” encoder | Huawei’s “own” decoder | mixed dataset | 0.686 |
| vivo’s “own” encoder | MTK’s “own” decoder | mixed dataset | 0.667 |
| vivo’s “own” encoder | Qualcomm’s “own” decoder | mixed dataset | 0.680 |
| vivo’s “own” encoder | Samsung’s “own” decoder | mixed dataset | 0.677 |

Table 2.2-3. Option 4 track 2 results, where Dataset 1 = Dataset 2 = Mixed training dataset, Dataset 3 = vivo dataset, Test dataset = vivo dataset.

| Option 4, Dataset 1 = Dataset 2 = Mixed training dataset, Dataset 3 = vivo dataset |  |  |  |
| --- | --- | --- | --- |
| Encoder | Decoder | Test dataset | SGCS results on mixed dataset |
| vivo’s “own” encoder | vivo’s “own” decoder | vivo dataset | 0.713 |
| vivo’s “own” encoder | Ericssion’s “own” decoder | vivo dataset | 0.713 |
| vivo’s “own” encoder | Nokia’s “own” decoder | vivo dataset | 0.707 |
| vivo’s “own” encoder | Huawei’s “own” decoder | vivo dataset | 0.723 |
| vivo’s “own” encoder | MTK’s “own” decoder | vivo dataset | 0.707 |
| vivo’s “own” encoder | Qualcomm’s “own” decoder | vivo dataset | 0.718 |
| vivo’s “own” encoder | Samsung’s “own” decoder | vivo dataset | 0.716 |

Observation 2: The additional results of Option 4 track 2 using of Samsung models with quantization, are shown in Table 2.2-1, Table 2.2-2 and Table 2.2-3.

Observation 3: By comparing the results of vivo’s “own” decoder and other companies’ “own” decoder, similar performance is observed. It is feasible for DUT to train it new encoder in Option 4.

Observation 4: For track 2, it seems that Option 3, 4a and 4b have similar validation performance.

  - Considerations on Option 3 and Option 4 of test decoder

In this subsection, details of Option 3 and Option 4 of test decoder will be discussed. For 2-sided AI/ML model tests, it was agreed that test decoder/encoder is to be used in UE conformance tests and gNB conformance tests, respectively.

Consideration on what would be standardized for option 3

In last meeting, it has been agreed that to verify the performance of CSI compression test decoder for calibration of test equipment, agree to define the reference encoder including encoder structure and parameters in RAN4. There are some FFS, which will be discussed in the section.

Agreement: - To verify the performance of CSI compression test decoder for calibration of test equipment, agree to define the reference encoder including encoder structure and parameters in RAN4 - FFS on whether to specify it in TS or TR, and capture the conclusion in the TR conclusion part - FFS on criterion for selection of reference encoder - FFS on decision whether RAN4 or RAN5 will specify it Agreements - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity)

Requirement should be considered before the alignment work in Option 3 and Option 4. The test decoder(s) in detailed test case(s), would be used to verify that DUT will meet the defined requirement when communicating with TE. So, we would need reference encoder and decoder, to define requirement.

For both Option 3 and Option 4, since TE vendors will need to implement the test decoder based on the encoder, decoder or dataset in the spec, a verification procedure would be needed to verify that the decoder is correctly implemented and has the right level of performance. A verification encoder will be needed for the verification vendor to verify TE test decoder. TE may also need a verification encoder to test the performance of its test decoder. The simplest and most efficient way is to specify the reference encoder, which is used as verification encoder. The vendor, who is responsible for the TE test decoder verification, could just use the specified reference encoder to test the TE test decoder.

- No extra training procedure to train the verification encoder will be needed. If there is no specified reference encoder, the verification vendor will need considerable training time of hours or days and considerable computation resources, to train the verification encoder.

- No performance degradation from imperfectly matched verification encoder will be caused using different training method and/or train dataset. Currently, there is no aligned training method. RAN4 has aligned training data generation method, but does not have aligned dataset. If there is no specified reference encoder, the verification vendor will need to train the verification encoder using its own training method and/or train dataset. Then, different verification vendor would have different verification encoder, which may imperfectly match with TE test decoder. When the inference performance of test decoder and verification encoder is not so good, it may not be the problem caused by TE vendor.

Also, as discussed in Subsection 2.4, using reference encoder could achieve better field performance, since only finetuning decoder is better than only finetuning encoder.

Currently, RAN1 and RAN4 both work on reference model. RAN4 reference model is designed to derive test decoder, and achieve not worse performance than eType II codebook at least on pre-defined test condition. RAN1 reference model is used to generate encoder and decoder in field, and achieve acceptable performance gain than eType II in field. Both reference encoder and reference decoder are considered in RAN1 now. Then, reference encoder is needed to facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4.

Then we have the following proposal for reference encoder.

Proposal 1: Reference encoder needs to be specified in TS, due to the following aspects:

- To be used as verification encoder in the TE test decoder verification.

  - No extra training procedure to train the verification encoder will be needed.

  - No performance degradation from imperfectly matched verification encoder will be caused.

- To achieve better field performance.

- To facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4.

During RAN4 discussion, companies will generate multiple encoder and decoder pairs, and then encoder and decoder pair(s) will be selected first, from which test decoder(s) is selected. And then, the paired reference encoder can be selected from the selected encoder and decoder pair. Since the encoder and decoder pairs are generated and selected in RAN4, it is more suitable for RAN4 to specify the reference encoder.

Proposal 2: Criterion for selection of reference encoder: reference encoder is selected from the encoder and decoder pair, from which the test decoder is selected.

Proposal 3: FFS on whether RAN4 or RAN5 to specify: it is more suitable for RAN4 to specify the reference encoder, since the encoder and decoder pairs are generated and selected in RAN4.

Consideration on sub-options of Option 4

In RAN4#112 meeting, the following agreement of sub-options of Option 4 has been achieved.

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

In last RAN4 meeting (RAN4#114bis), the following agreement is achieved for Option 4.

What needs to be captured or specified for Option 4 Agreement: - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase.

In the following, some further details of Option 4a/4b will be discussed.

For Option 4a, one method is agreed to generate the dataset: the specified dataset is generated by the aligned reference encoder and reference decoder. TE would train the test decoder based on the specified dataset.

- Model structure needs to be specified. As seen in Appendix A, model structure (back-bone, parameters, e.g., number of layers, etc) also has significant performance impact even if complexity of model (in terms of FLOPS) are similar. So, if the model structure is not specified, different TE vendor would obtain the test decoder with different performance, which has large impact on the test. Then, it is necessary to specify model structure.

- Reference encoder is suggested to be specified. Similar to Option 3, reference encoder can be used as verification encoder in the TE test decoder verification. When reference encoder is specified in Option 4a, no extra training procedure to train the verification encoder will be needed, and no performance degradation from imperfectly matched verification encoder will be caused. Also, reference encoder is needed to facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4.

Proposal 4: In Option 4a, the model structure needs to be fully specified, to guarantee the performance of test decoder.

Proposal 5: In Option 4a, reference encoder is suggested to be specified.

- To be used as verification encoder in the TE test decoder verification.

  - No extra training procedure to train the verification encoder will be needed.

  - No performance degradation from imperfectly matched verification encoder will be caused.

- To facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4.

In Option 4b, channel generation method would also need to be in the spec, so that TE can derive the Training samples including only channel information could be obtained from specified channel generation method. Using the reference encoder and generated channel data, both TE vendor and DUT vendor could derive the decoder, which is paired with the reference encoder. DUT could directly use the reference encoder, or derive a new encoder based on the derived test decoder and generated channel data. Since TE vendors may derive test decoders with different performance, which would impact UE to pass the test. As seen in Appendix, it is suggested to capture the reference decoder model structure as well in the spec.

Proposal 6: In Option 4b, channel generation method and reference decoder structure need to be fully specified in the spec.

Further consideration on reference model (Option 3, Option 4b) and reference dataset (Option 4a)

It is seen from above discussion that both reference model and reference dataset would need the alignment of reference encoder and reference decoder. One of the main differences would be that what will be put in the spec. Reference decoder is at least captured in spec in Option 3, and the reference decoder is at least captured in spec in Option 4b. In Option 4a, the dataset, which contains both target channel (encoder input or decoder output) and latent information (encoder output or decoder input), generated from the reference encoder and reference decoder is at least captured in spec.

The main differences between reference model and reference dataset would be discussed in the following.

- Reference dataset would cause some performance loss compared to reference model. For reference dataset, separate training is used to obtain the test decoder, which would have some performance loss. The performance loss would depend on at least the following aspects:

  - The quality of dataset, e.g., the number of samples and the ergodicity of dataset. If the number of samples is not enough or the ergodicity of dataset is not good, the performance loss may be not acceptable.

  - The training method. Different training method would cause different performance. If proper training method is not in the spec or aligned, the performance loss may be also not acceptable.

  - The model structure. As seen from the results in Appendix A, model structure (back-bone, parameters, e.g., number of layers, etc) also has significant performance impact even if complexity of model (in terms of FLOPS) are similar. So, if the model structure is not specified, different TE vendor would obtain the test decoder with different performance, which has large impact on the test.

- The spec impact of writing model and dataset in the spec would be similar. Both model and dataset could be uploaded in cloud or FTP, and the link could be captured in the spec. RAN4 already has aligned method to describe the model structure, and open format file (e. g., ONNX) could provide the details of model. So, the description of model is not an issue.

- Reference dataset needs extra step compared to reference model, which is dataset generating from the aligned reference encoder and reference decoder. After the alignment of reference encoder and reference decoder, the reference encoder or reference decoder can be capture in the spec and work is finished. However, for reference dataset, the dataset would need to be generated by the aligned reference encoder and reference decoder, where dataset of raw channel or encoder input needs to be aligned.

- The storage size of reference dataset is larger than reference model. From current discussion of logistical issues, it is aligned that the dataset would be 1GB~10GB and model would be 7MB~100MB. Note that the model size may be reduced in future RAN4 study.

- Reference model is more flexible than referend dataset for future release. In the future release, new channel model or scenario may be defined, and reference model may be also workable for these channel model or scenario.

From the analysis, it is seen that there is no essential difference between reference model and reference dataset. Reference dataset may have some disadvantages compared to reference model, and the benefit of reference dataset over reference model is unclear.

Observation 5: There is no essential difference between reference model and reference dataset. Reference dataset may have some disadvantages compared to reference model, and the benefit of reference dataset over reference model is unclear.

- Reference dataset would cause some performance loss compared to reference model.

- The spec impact of writing model and dataset in the spec would be similar.

- Reference dataset needs extra step compared to reference model, which is dataset generating from the aligned reference encoder and reference decoder.

- The storage size of reference dataset is larger than reference model.

- Reference model is more flexible than referend dataset for future release.

For feasibility studies of AI/ML model for test options, we believe similar conclusion can be drawn for both Option 3 and Option 4 (Option 4a and Option 4b). The difference between Option 3 and Option 4 is what AI/ML model(s) for test decoder implementation is captured in the spec. The evaluation should be same in terms of performance.

Proposal 7: Similar feasibility conclusion through the simulation campaign of test options can be draw for both Option 3 and Option 4 (Option 4a and Option 4b).

  - Relation between RAN4 test decoder Option 3/4a/4b and RAN1 inter-vendor collaboration direction C

After rounds of discussion, RAN1 has achieved significant progress on model structure standardization. As seen in Appendix C, RAN1#120 agreed the full-pictured model structure for studying the standardized model structure, and some options on the scalable model structure specification. As seen in Appendix D, RAN4 also has good progress on reference model standardization.

Therefore, we would like to present our view for relation between RAN4 reference model and RAN1 reference model.

The comparison between RAN1 reference model and RAN4 reference model

RAN1 reference model is used for inter-vendor training collaboration in field, where the actual encoder at UE and actual decoder at NW will be designed based on the RAN1 reference model. RAN4 reference model is used to test the AI/ML CSI compression functionality. There are many similarities, between RAN1 reference model and RAN4 reference model for both direction C and direction 3a-1. In fact, RAN1 already had agreement that the model for RAN1 direction C is equivalent to RAN4 model for testing.

For direction 3a-1, all the evaluations for model structure/parameters scalability study in RAN1 are based on the same model structure as direction C. Thus, direction 3a-1 model structure would also be equivalent to RAN4 model for testing.

We further elaborate the following similarities for RAN1 reference model and RAN4 reference model.

- Similarity 1: RAN1 and RAN4 both use UMa to generate reference model, and RAN4 from testing perspective need to consider suitable model structure and scalability, the design would be similar from model structure perspective since all the evaluation assumptions are the same. Under the similar design and evaluation assumptions, RAN1 reference model and RAN4 reference model would have similar performance.

- Similarity 2: No matter which option RAN1 takes for inter-vendor collaboration, RAN4 specified model needs to take field performance into account. Otherwise, the testing would be meaningless. If RAN1 3a-1 or direction C is supported, the simplest way to design RAN4 model for testing is to use the same model for RAN1 reference model since the testing would guarantee the performance in the field. Even if for Option 4-1, RAN4 also needs to guarantee the testing model provide good guidance for field.

- Similarity 3: Both RAN1 reference model and RAN4 reference model have low proprietary issue, since UE, NW and TE can develop their real models which are different from reference model. Reference model is just used for reference, and then UE, NW and TE are not forced to directly implement the reference model. They can choose their preferred model backbone and hyper-parameters, and then train the new model using reference model. So, both RAN1 reference model and RAN4 reference model have low proprietary issue for UE, NW and TE.

- Similarity 4: For all RAN4 testing options, reference/testing encoder/decoder needs to be defined based on agreement and conclusions of RAN1 and RAN4. Then the same model structure can be used for 3a-1 for reference. The reference model structure that works well in RAN1/RAN4 reference model, would also have good performance for RAN1 direction 3a-1. There is no need to design extra reference model structure for RAN1 direction 3a-1.

Thus, we have the following observation.

Observation 6: Following similarities for RAN1 reference model and RAN4 reference model are observed.

- Similarity 1: RAN1 and RAN4 both use UMa to generate reference model, and RAN4 from testing perspective need to consider suitable model structure and scalability, the design would be similar from model structure perspective since all the evaluation assumptions are the same.

- Similarity 2: No matter which option RAN1 take for inter-vendor collaboration, RAN4 specified model needs to take field performance into account. Otherwise, the testing would be meaningless. If RAN1 3a-1 or direction C is supported, the simplest way to design RAN4 model for testing is to use the same model for RAN1 reference model since the testing would guarantee the performance in the field.

- Similarity 3: Both RAN1 reference model and RAN4 reference model have low proprietary issue, since UE, NW and TE can develop their real models which are different from reference model.

- Similarity 4: For all RAN4 testing options, reference/testing encoder/decoder needs to be defined based on agreement and conclusions of RAN1 and RAN4. Then the same model structure can be used for 3a-1 for reference.

What is work assignment between RAN1 and RAN4 on reference model?

From the above discussion, it is seen that there are little differences between RAN1 reference model and RAN4 reference model. RAN4 will at least define or capture reference model, and RAN1 TS can quote the RAN4 reference model.

In sum, RAN1 and RAN4 can share the workload for defining the reference/testing model for potential recommended direction/sub-directions. RAN4 can take RAN1 study/work output on model structure into account.

Proposal 8: RAN1 and RAN4 can share the workload for defining the reference/testing model for potential recommended direction/sub-directions. RAN4 can take the lead for model structure/parameters specification and take RAN1 study/work output on model structure into account.

  - Initial field test result of reference model for CSI compression

Initial field test of reference model for CSI compression is conducted as following. The data is collected from actual 5G network and the whole collecting area including Cell 1 and Cell 2 is about 250m * 150m. The detailed parameters are provided in the below table.

Table 2.5-1. Parameters of field test of CSI compression.

| Parameters | Value |
| --- | --- |
| Scenario | Actual 5G network |
| Carrier frequency | 3.45GHz |
| Subcarrier spacing | 30KHz |
| Frequency resources | 52 RBs, 13 subbands for 4RBs per subband |
| Rank | Max rank 1 |
| gNB antenna | 8 antenna ports |
| UE antenna | 4 antenna ports |
| CSI payload | 55 bits payload |

The performance of different physical cells is analysed in the following. We have tested the coverage of different cells in the industrial park, according to the measured RSRP, RSRQ and SINR of mobile phones. The channel samples are collected by our prototype. The coverage areas of two typical cells in the industrial park are shown in the below figure. Note that in the coverage areas of each cell, the signal of target cell is acceptable in most places but would be not always the strongest among all cells. This means that the map of actual cell section would be different from the following figure.

Figure 2.5-1. The map of data collecting cells.

For Cell 1, we have collected about 144k outdoor samples. For Cell 2, we have collected about 65k outdoor samples. We choose 10% of them as validation samples.

We use the almost the same simulation assumptions of UMa and CDL in Subsections 2.1 and 2.2, except changing gNB 32Tx to 8Tx. Also, it is important to point out that there would be plenty of parameter mismatches between simulation data and field data.

The CNN encoder and the Transformer encoder trained by UMa and CDL are tested on Cell 1 and Cell 2 in field. Transformer decoder is used for both CNN encoder and Transformer encoder. The SGCS results are provided in the below table. It is seen that compared with eType II codebook, UMa transformer has 2.2% SGCS loss in Cell 1, but 2.7% SGCS gain in Cell 2. However, considerable SGCS loss is obtained by using AI/ML model trained by CDL. For UMa training data, Transformer would provide about 4.2%~5.1% SGCS gain compared with CNN. Note that the CNN/Transformer encoder structures used in this section are referred to the proposed model structures in our contributions [3] in the last meeting.

Table 2.5-2. The SGCS results of multiple AI/ML models trained by simulation data and tested on field data.

| 55 feedback bits | SGCS on Cell 1 | SGCS on Cell 2 |
| --- | --- | --- |
| eTypeII | 0.677 | 0.839 |
| Field Transformer encoder (Cell 1 + Cell 2) | 0.779 | 0.915 |
| Field CNN encoder (Cell 1 + Cell 2) | 0.765 | 0.910 |
| UMa CNN encoder | 0.630 | 0.827 |
| UMa Transformer encoder | 0.662 | 0.862 |
| CDL CNN encoder | 0.202 | 0.512 |
| CDL Transformer encoder | 0.309 | 0.557 |

Observation 7: From initial results for field test, the generalization performance of AI/ML model trained by UMa simulation data on field data seems acceptable, which has similar performance as eType II codebook. The generalization performance of AI/ML model trained by CDL simulation data on field data is worse than AI/ML model trained by UMa simulation data.

Based on the results of above table, we further study the case that NW-side and/or UE-side may separately train new decoder and/or encoder compatible to the reference AI/ML model.

The simulation results of different simulation cases are provided in the below table. Since transformer has better performance than CNN, transformer is used in the below table. Note that different from the above table, the results of AI/ML model trained by field data (Case 1) are obtained by using the field data of current cell, that is, the results of cell 1 is obtained by using Cell 1 data.

Case 2A-1 and Case 2A-2 are the cases that only one side finetunes their part based on reference encoder and decoder pair. Here the reference encoder and decoder pair is obtained by training with UMa data, which is generated by RAN4 aligned simulation assumptions. In Case 2A-1, the encoder is finetuned from reference encoder using field data of current cell, while the decoder is fixed to be reference decoder. In Case 2A-2, the decoder is finetuned from reference decoder using field data of current cell, while the encoder is fixed to be reference encoder. Case 2A-3 and Case 2A-4 is the case that both two sides finetune their part based on reference encoder and decoder pair, separately. Since UE and NW would not use the same training data in practical, mismatch on training data between UE and NW is considered in Case 2A-3 and Case 2A-4. In both Case 2A-3 and Case 2A-4, decoder is finetuning from reference decoder using field data of current cell. In Case 2A-3, small mismatch is considered, where encoder is finetuned from reference encoder using field data of Cell 1 and Cell 2. In Case 2A-4, large mismatch is considered, where encoder is finetuned from reference encoder using field data of the other cell.

Compared to directly use reference model (Case 2), it is seen that only finetuning encoder (Case 2A-1) has up to 8.6% SGCS gain and only finetuning decoder (Case 2A-2) has up to 11.0% SGCS gain. Compared to fully trained by field data (Case1), it is seen that Case 2A-1 has up to 9.7% SGCS loss and Case 2A-2 has up to 7.7% SGCS loss.

Compared to Case 2A-2, considering mismatch on training data between UE and NW, encoder and decoder separate training (Case 2A-3, Case 2A-4) has performance loss, and sometimes the performance is terrible (Case 2A-4 on Cell 1). Then, it seems that it is benefit for NW to finetune the decoder from reference decoder using field data, while UE just implements the reference encoder without finetuning.

Table 2.5-3. The SGCS results of AI/ML models trained by simulation data, and then finetuned on field data.

| Index | Simulation case | Performance on Cell 1 |  | Performance on Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SGCS | Gain to eType II | SGCS | Gain to eType II |
| Case 0 | eType II | 0.677 | 0.0% | 0.839 | 0.0% |
| Case 1 | upper bound | 0.796 | 17.6% | 0.929 | 10.7% |
| Case 2 | Reference model (Trained by UMa) | 0.662 | -2.2% | 0.862 | 2.7% |
| Case 2A-1 | Encoder trained by field data, while using reference decoder | 0.719 | 6.2% | 0.866 | 3.2% |
| Case 2A-2 | Decoder trained by field data, while using reference encoder | 0.735 | 8.6% | 0.886 | 5.6% |
| Case 2A-3 | Encoder and decoder separate training considering small mismatch on training data between UE and NW | 0.734 | 8.4% | 0.885 | 5.5% |
| Case 2A-4 | Encoder and decoder separate training considering large mismatch on training data between UE and NW | 0.439 | -35.2% | 0.850 | 1.3% |

Observation 8: From initial results of field test, it is observed that

- Directly use reference model in field (Case 2): similar performance on field data as eType II for one cell and performance loss compared to eType II is observed for another cell.

- Finetuned encoder or decoder using field data against reference decoder/encoder (Case 2A-1, Case 2A-2): performance improved compared to directly use reference model (Case 2), but still has performance loss compared to fully trained by field data (Case1). Finetuned decoder (Case 2A-2) is better than finetuned encoder (Case 2A-1).

- Pairing of finetuned encoder and finetuned decoder (Case 2A-3, Case 2A-4): performance loss in some cases considering mismatch on training data between UE and NW.

Then from the comparison between Case 2A-1, Case 2A-2, Case 2A-3 and Case 2A-4, it is seen that only finetuning decoder is better than only finetuning encoder. Encoder and decoder separate training may be the worst or better than only finetuning encoder, depending of the level of training data mismatch. Thus, reference encoder seems necessary in reference model based inter-vendor training collaboration, and then the reference model is at least the encoder.

Proposal 9: To achieve better field performance, the reference model should at least include the encoder part.

  - Suggested simulation assumptions for reference model for both encoder and decoder

In order to consider model performance with more concrete details, companies are encouraged to bring parameters/values proposals, considering RAN4 existing test configuration or RAN1 baseline scenario captured in TR 38.843 in Table 6.2.1-2 for example.

Below table presents the baseline link level simulation assumptions of CDL channel for reference model derivation, where some unsuitable information is removed.

Table 2.6-1. Baseline link level simulation assumptions of CDL channel for reference model derivation.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD OFDM |
| Carrier frequency | 2GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15kHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C |
| UE speed | 3kmhr |
| Delay spread | 30ns |
| Channel estimation | Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |

Based on our proposed LLS assumptions for CDL-C channel model and CNN model structures proposed in our contributions [3] in the previous meeting, below table shows the SGCS results for different model structures under the different tests and training datasets. Note that the UMa dataset is generated based on the SLS assumption from RAN1 and related parameters are same as our proposed LLS assumptions for CDL-C. The mixed dataset contains the partial CDL-C dataset and UMa dataset, which are mixed together in one dataset randomly for model training.

Table 2.6-2. SGCS under different channel model for the proposed CNN and Transformer.

| Model structure |  | CNN |  | Transformer |  |
| --- | --- | --- | --- | --- | --- |
| CSI feedback bits | Training dataset | Test dataset |  | Test dataset |  |
|  |  | CDL-C | UMa | CDL-C | UMa |
| 116 bits | CDL-C | 0.994 | 0.336 | 0.996 | 0.253 |
|  | UMa | 0.791 | 0.787 | 0.831 | 0.821 |
|  | Mixed | 0.982 | 0.773 | 0.988 | 0.801 |
| CNN encoder: FLOPS 126M, size 0.1M. CNN decoder: FLOPS 1974M, size 2.4M. Transformer encoder: FLOPS139M, size 5.6M. Transformer decoder: FLOPS 139M, size 5.6M Note: Mixed training dataset includes CDL-C and UMa channel model. |  |  |  |  |  |

It can be observed that, the models trained under CDL-C dataset could provide a very high SGCS results under the CDL-C test datasets, while the performance degrades severely under the UMa test datasets. However, the models trained under UMa could provide a medium good performance of SGCS for UMa test dataset, and provide a higher SGCS under the CDL-C test dataset compared with the model training under CDL-C but tested in UMa. For the model trained under mixed dataset, it could provide good SGCS results under CDL-C and UMa test datasets and only degrade a little compared with the results of <CDL-C trained, CDL-C tested> and <UMa trained, UMa tested>. Thus, the best option is to use the mixed dataset for training and one of types of the dataset for testing. Since the TDL channel is a simpler channel compare with CDL and UMa, it can also be used for mixing and RAN4 test case. Similar to the previous test case, RAN4 tests could only use TDL channel. Based on the following analysis, we could have the following proposal.

Proposal 10: Using the mixed dataset for model training, including the mixing of TDL, CDL and UMa, while using the TDL dataset for RAN4 tests. Other mixing rules are not precluded.

- Summary

Based on above analysis, following proposals and observations are present.

Observation 1: The additional results of Option 3 track 2 using of Samsung models with quantization, are shown in Table 2.1-1.

Table 2.1-1. SGCS results of Option 3 for track 2 using Samsung models, with quantization.

| Option 3 for track 2 using Samsung models, with quantization |  |  |  |  |
| --- | --- | --- | --- | --- |
| Cases | Encoder | Decoder | Test dataset | SGCS results |
| Baseline | Own | Own | mixed dataset | 0.696 |
| Option 3 results in RAN4#114bis | Trained based on Samsung’s decoder, using mixed dataset | Samsung’s decoder | mixed dataset | 0.689 |
| Additional results 1 for RAN4#115 | Trained based on Samsung’s decoder, using vivo dataset | Samsung’s decoder | mixed dataset | 0.691 |
| Additional results 2 for RAN4#115 | Trained based on Samsung’s decoder, using vivo dataset | Samsung’s decoder | vivo dataset | 0.731 |

Observation 2: The additional results of Option 4 track 2 using of Samsung models with quantization, are shown in Table 2.2-1, Table 2.2-2 and Table 2.2-3.

Table 2.2-1. Option 4 track 2 results, where Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset, Test dataset = mixed dataset.

| Option 4, Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset |  |  |  |
| --- | --- | --- | --- |
| Encoder | Decoder | Test dataset | SGCS results on mixed dataset |
| vivo’s “own” encoder | vivo’s “own” decoder | mixed dataset | 0.692 |
| vivo’s “own” encoder | Ericssion’s “own” decoder | mixed dataset | 0.689 |
| vivo’s “own” encoder | Nokia’s “own” decoder | mixed dataset | 0.684 |
| vivo’s “own” encoder | Huawei’s “own” decoder | mixed dataset | 0.702 |
| vivo’s “own” encoder | MTK’s “own” decoder | mixed dataset | 0.684 |
| vivo’s “own” encoder | Qualcomm’s “own” decoder | mixed dataset | 0.696 |
| vivo’s “own” encoder | Samsung’s “own” decoder | mixed dataset | 0.694 |

Table 2.2-2. Option 4 track 2 results, where Dataset 1 = Dataset 2 = Mixed training dataset, Dataset 3 = vivo dataset, Test dataset = mixed dataset.

| Option 4, Dataset 1 = Dataset 2 = Mixed training dataset, Dataset 3 = vivo dataset |  |  |  |
| --- | --- | --- | --- |
| Encoder | Decoder | Test dataset | SGCS results on mixed dataset |
| vivo’s “own” encoder | vivo’s “own” decoder | mixed dataset | 0.674 |
| vivo’s “own” encoder | Ericssion’s “own” decoder | mixed dataset | 0.674 |
| vivo’s “own” encoder | Nokia’s “own” decoder | mixed dataset | 0.667 |
| vivo’s “own” encoder | Huawei’s “own” decoder | mixed dataset | 0.686 |
| vivo’s “own” encoder | MTK’s “own” decoder | mixed dataset | 0.667 |
| vivo’s “own” encoder | Qualcomm’s “own” decoder | mixed dataset | 0.680 |
| vivo’s “own” encoder | Samsung’s “own” decoder | mixed dataset | 0.677 |

Table 2.2-3. Option 4 track 2 results, where Dataset 1 = Dataset 2 = Mixed training dataset, Dataset 3 = vivo dataset, Test dataset = vivo dataset.

| Option 4, Dataset 1 = Dataset 2 = Mixed training dataset, Dataset 3 = vivo dataset |  |  |  |
| --- | --- | --- | --- |
| Encoder | Decoder | Test dataset | SGCS results on mixed dataset |
| vivo’s “own” encoder | vivo’s “own” decoder | vivo dataset | 0.713 |
| vivo’s “own” encoder | Ericssion’s “own” decoder | vivo dataset | 0.713 |
| vivo’s “own” encoder | Nokia’s “own” decoder | vivo dataset | 0.707 |
| vivo’s “own” encoder | Huawei’s “own” decoder | vivo dataset | 0.723 |
| vivo’s “own” encoder | MTK’s “own” decoder | vivo dataset | 0.707 |
| vivo’s “own” encoder | Qualcomm’s “own” decoder | vivo dataset | 0.718 |
| vivo’s “own” encoder | Samsung’s “own” decoder | vivo dataset | 0.716 |

Observation 3: By comparing the results of vivo’s “own” decoder and other companies’ “own” decoder, similar performance is observed. It is feasible for DUT to train it new encoder in Option 4.

Observation 4: For track 2, it seems that Option 3, 4a and 4b have similar validation performance.

Proposal 1: Reference encoder needs to be specified in TS, due to the following aspects:

- To be used as verification encoder in the TE test decoder verification.

  - No extra training procedure to train the verification encoder will be needed.

  - No performance degradation from imperfectly matched verification encoder will be caused.

- To achieve better field performance.

- To facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4.

Proposal 2: Criterion for selection of reference encoder: reference encoder is selected from the encoder and decoder pair, from which the test decoder is selected.

Proposal 3: FFS on whether RAN4 or RAN5 to specify: it is more suitable for RAN4 to specify the reference encoder, since the encoder and decoder pairs are generated and selected in RAN4.

Proposal 4: In Option 4a, the model structure needs to be fully specified, to guarantee the performance of test decoder.

Proposal 5: In Option 4a, reference encoder is suggested to be specified.

- To be used as verification encoder in the TE test decoder verification.

  - No extra training procedure to train the verification encoder will be needed.

  - No performance degradation from imperfectly matched verification encoder will be caused.

- To facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4.

Proposal 6: In Option 4b, channel generation method and reference decoder structure need to be fully specified in the spec.

Observation 5: There is no essential difference between reference model and reference dataset. Reference dataset may have some disadvantages compared to reference model, and the benefit of reference dataset over reference model is unclear.

- Reference dataset would cause some performance loss compared to reference model.

- The spec impact of writing model and dataset in the spec would be similar.

- Reference dataset needs extra step compared to reference model, which is dataset generating from the aligned reference encoder and reference decoder.

- The storage size of reference dataset is larger than reference model.

- Reference model is more flexible than referend dataset for future release.

Proposal 7: Similar feasibility conclusion through the simulation campaign of test options can be draw for both Option 3 and Option 4 (Option 4a and Option 4b).

Observation 6: Following similarities for RAN1 reference model and RAN4 reference model are observed.

- Similarity 1: RAN1 and RAN4 both use UMa to generate reference model, and RAN4 from testing perspective need to consider suitable model structure and scalability, the design would be similar from model structure perspective since all the evaluation assumptions are the same.

- Similarity 2: No matter which option RAN1 take for inter-vendor collaboration, RAN4 specified model needs to take field performance into account. Otherwise, the testing would be meaningless. If RAN1 3a-1 or direction C is supported, the simplest way to design RAN4 model for testing is to use the same model for RAN1 reference model since the testing would guarantee the performance in the field.

- Similarity 3: Both RAN1 reference model and RAN4 reference model have low proprietary issue, since UE, NW and TE can develop their real models which are different from reference model.

- Similarity 4: For all RAN4 testing options, reference/testing encoder/decoder needs to be defined based on agreement and conclusions of RAN1 and RAN4. Then the same model structure can be used for 3a-1 for reference.

Proposal 8: RAN1 and RAN4 can share the workload for defining the reference/testing model for potential recommended direction/sub-directions. RAN4 can take the lead for model structure/parameters specification and take RAN1 study/work output on model structure into account.

Observation 7: From initial results for field test, the generalization performance of AI/ML model trained by UMa simulation data on field data seems acceptable, which has similar performance as eType II codebook. The generalization performance of AI/ML model trained by CDL simulation data on field data is worse than AI/ML model trained by UMa simulation data.

Observation 8: From initial results of field test, it is observed that

- Directly use reference model in field (Case 2): similar performance on field data as eType II for one cell and performance loss compared to eType II is observed for another cell.

- Finetuned encoder or decoder using field data against reference decoder/encoder (Case 2A-1, Case 2A-2): performance improved compared to directly use reference model (Case 2), but still has performance loss compared to fully trained by field data (Case1). Finetuned decoder (Case 2A-2) is better than finetuned encoder (Case 2A-1).

- Pairing of finetuned encoder and finetuned decoder (Case 2A-3, Case 2A-4): performance loss in some cases considering mismatch on training data between UE and NW.

Proposal 9: To achieve better field performance, the reference model should at least include the encoder part.

Proposal 10: Using the mixed dataset for model training, including the mixing of TDL, CDL and UMa, while using the TDL dataset for RAN4 tests. Other mixing rules are not precluded.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.

- R4-2408294, vivo, “Discussion on testability and interoperability issues for CSI compression and CSI prediction”, RAN4#111, May 20 – 24, 2024.

- Appendix A: Comparison of AI model performance for CSI compression

In order to better understand the performance difference for different AI models, initial evaluation was conducted under the CDL-A channel model with 30ns delay. Fig. A-1 shows the simulation procedure of the CSI compression.

Fig. A-1. Illustration of the simulation procedure of CSI compression

The simulation results are provided in Table A-1 with transformer encoder and Table A-2 with CNN encoder, respectively.

Table A-1. Performance comparison for different decoder structure and complexity (Transformer encoder)

| Encoder | Decoder |  |  |  | SGCS (R16 CB: 0.8375) |
| --- | --- | --- | --- | --- | --- |
|  | Back-bone | Model parameter | Number of model parameter | FLOPS |  |
| Back-bone: Transformer Number of model parameter: 4.1M FLOPS: 5×107 Depth: 10 transformer blocks Width: embedding=140 | Transformer | Depth: 18 transformer blocks Width: embedding=148 | 8.0M | 2×108 | 0.80 |
|  |  | Depth: 10 transformer blocks Width: embedding=198 | 8.0M | 2×108 | 0.92 |
|  |  | Depth: 20 transformer blocks Width: embedding=140 | 7.9M | 2×108 | 0.62 |
|  | CNN | Depth: conv layers Width: 165 feature maps | 0.2M | 2×108 | 0.88 |
|  |  | Depth: 17 conv layers Width: 41 feature maps | 0.2M | 2×108 | 0.91 |
|  |  | Depth: 150 conv layers Width: 20 feature maps | 0.2M | 2×108 | 0.92 |
|  | MLP | Depth: 1 FC layers Width: 9500 neurons | 98.7M | 2×108 | 0.91 |
|  |  | Depth: 10 FC layers Width: 3200 neurons | 109.6M | 2×108 | 0.89 |
|  |  | Depth: 15 FC layers Width: 2600 neurons | 106.6M | 2×108 | 0.83 |

In Table A-1, transformer encoder is used in the evaluation for verifying performance of different decoders. It can be seen that there could be large performance variance if model structure, including back-bone, parameters of decoder are different even if the complexity (FLOPs) are similar. For same type of decoder, if parameters are different, especially for transformer and MLP type of decoders, performance variance can be observed. The performance of CNN decoder seems not very sensitive to some parameters as listed in the table.

Table A-2. Performance comparison for different decoder structure and complexity (CNN encoder)

| Encoder | Decoder |  |  |  | SGCS (R16 CB: 0.8375) |
| --- | --- | --- | --- | --- | --- |
|  | Back-bone | Model parameter | Number of model parameter | FLOPS |  |
| Back-bone: CNN Number of model parameter: 4.1M FLOPS: 5×107 Depth: 17 conv layers Width: 32 feature maps | CNN | Depth: 17 conv layers Width: 16 feature maps | 0.08M | 4×107 | 0.88 |
|  | Transformer | Depth: 4 transformer blocks Width: embedding =110 | 1.4M | 4×107 | 0.84 |
|  | MLP | Depth: 3 FC layers Width: 2200 neurons | 16.5M | 4×107 | 0.89 |

In table A-2, CNN encoder is used in the evaluation for verifying performance of different decoders. Similar observations can be made.

- Appendix B: Agreement of assumptions for aligned reference encoder and reference decoder

In previous RAN4 meeting, the simulation assumptions have been agreed for feasibility study of aligning model among companies.

System-level simulation parameters (from TR): Parameter; Value. Duplex, Waveform; FDD, OFDM. Multiple access; OFDMA. Scenario; Dense Urban (Macro only). Frequency Range; FR1 only, [2GHz, 4GHz]. Inter-BS distance; 200m. Channel model; According to TR 38.901. Antenna setup and port layouts at gNB; Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ. Antenna setup and port layouts at UE; 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4). BS Tx power; 44dBm for 20MHz. BS antenna height; 25m. UE antenna height & gain; Follow TR36.873. UE receiver noise figure; 9dB. Numerology; Slot/non-slot; 14 OFDM symbol slot. SCS; Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz. Simulation bandwidth; Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz. Frame structure; Slot Format 0 (all downlink) for all slots. MIMO scheme; SU-MIMO. MIMO layers; Baseline: 1 Optional: 2. CSI feedback; Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms. Traffic load (Resource utilization); Baseline: 50% Optional: 20/70%. UE distribution; CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h). UE receiver; MMSE-IRC as the baseline receiver. Feedback assumption; Realistic. Channel estimation; Realistic or ideal channel estimation.

In the e-mail discussion after RAN4#111 meeting, a CNN based model structure pair for both encoder and decoder has been aligned for feasibility study.

Based on the two rounds of discussions, the following parameters are proposed: Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. Table 1. CNN model parameters for encoder. Model architecture parameters for encoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 47M (vs. 1.6M for eTypeII). Model size; 448Kbyte. Num of Resnet blocks; 6. Layer after resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,32). Size of message (feature map) passed across layers; (13,32). Table 2. CNN model parameters for decoder. Model architecture parameters for decoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 740M. Model size; 7.2Mbyte. Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,128). Size of message (feature map) passed across layers; (13,32). Other model related parameters: Activation function; ReLU. Latent message size; 32 float values. Payload size, bits; 64 per layer.. Quantization; 2 bits. System parameters: Frequency range; 2GHz (4GHz optional). MIMO Layers; 1 layer (2 layers optional, companies to. Sub-bands; 13. Channel estimation; ideal. Metrics; SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI.. Companies are also invited to provide the training parameters used.

In RAN4#112 meeting, more details of data generation and training method have also been aligned to further reduce the mismatch between companies

Parameter/Configurations; Options/Values; Company comments. Channel/data-set related. UE antenna element assumptions; - Option 1: Omni-directional antenna elements (Ericsson) - Option 2: Directional antenna elements - Option 3: Other; [Apple] We provided 2 simulation results: Directional (both horizontal/vertical) Omni (both horizontal/vertical) [QC] Option 1 (both horizontal and vertical) [Intel] Option 1 [OPPO]:Option1 [CMCC] option 1. Configuration for alignment: - Ideal omni-directional antenna elements following RAN1 assumptions. Sub-band channel averaging; - [Nokia, QC, Apple, vivo, Ericsson,OPPO] Average of covariance channel matrix per RBs in the sub-band before SVD - [ZTE]: Per sub-band - [CMCC]: Average of raw channel matrix per RBs in the sub-band before SVD; CATT: Prefer not to do the averaging before SVD.. Configuration for alignment: - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected. Any other pre-processing; - To be listed if used - [QC, Ericsson] Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - [OPPO] Normalize the phase of all entries of the eigenvector based on the phase of the last entry of the eigenvector - [Apple]: Normalize the phase of all entries of the eigenvector based on the phase of that entry that has the maximum amplitude averaged over the sub-bands; [Intel] prefer no other pre-processing. Configuration for alignment: - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1.. Data collection assumptions; - [Nokia, CATT, ZTE, Ericsson, CMCC] Each channel realization is collected from independent UE drop - Multiple channel realization can be collected from the same UE - Other - [QC] Multiple channels realizations are collected from multiple UEs and multiple drops of same UE - [Apple, Intel, OPPO]: Multiple drops. Each drop collect data from multiple UEs. Additionally collect samples from channel realization from each UE spaced at de-correlated interval.. Configuration for alignment: - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs.. Training dataset size; - [Nokia]: 630K - [QC]: 450K - [Samsung]: 25K - [R&S]: 614,440 - [MTK]: 700K - [Ericsson]: 800K - [Intel]: 200K - [CATT]: 102,600 - [ZTE]: 600k - [Apple]: 600K - [vivo]: 243000 - [OPPO]：600k - [CMCC]: 100K. Configuration for alignment: - Aim to use the dataset around 600K. - At least 100K sample to be used.. Dataset quantization; - Float32. Other data-set characteristics/statistics; - Companies to propose. - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII.. Model-related. Quantization of encoder output; - [Nokia]: Non-trainable quantization (bypassed during training) - [QC, CMCC] Quantization aware training - [Apple]: Quantization in forward and approximation in backward to enable learning. (avoid zero gradient of round function) - [vivo] VQ with quantization aware training - [Intel] Quantization aware training. Scalar quantization with non-trainable quantization grid. - [Ericsson] 2 bit scalar quantization, quantizer aware training; CATT: Consider to align the activation function and quantization method.. - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization. Normalization before quantization in the encoder; - [Nokia, Ericsson]: Normalization with tanh function after FC layer - [CATT, ZTE, vivo, CMCC]: Sigmoid function used after FC layer - [QC] Normalization not done due to the use of quantization aware training - [Apple]: Normalization with tanh function after FC layer. We don’t apply inverse of that in de-quantization - [Intel] Sigmoid after FC layer. Inverse sigmoid after de-quantization stage in decoder.. Configuration for alignment: - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional.. Use and configuration of drop-out; - [Nokia]: Use of on single drop-out layer after FC layer in the encoder and decoder, Drop-out rate = 0.4. - [CATT, QC, ZTE, Apple, vivo, Intel, Ericsson, CMCC]: No drop-out used.. Configuration for alignment: - Not to use drop-out layers. Training loss function, use of regularization; - [Nokia]: - SGCS with L2 regularization with weight decay, λ = 10-2 - [QC]: SGCS + additional terms for quantization aware training - [Apple, CATT, CMCC]: 1- SGCS - [R&S, MTK, Intel]: SGCS - [Ericsson]: NMSE - [ZTE]: MSE - [vivo]: 1-SGCS plus VQ loss for encoder output (MSE between original and quantized encoder output) - [Intel]: SGCS, no L1/L2 regularization. Configuration for alignment: - Use (1-SGCS) as loss function as reference. Optimizer; - [Samsung, QCM, Apple, MTK, Intel, CATT, Ericsson, CMCC]: Adam - [Nokia]: RMSProp (Root Mean Square Propagation) - [ZTE, vivo]: AdamW. Configuration for alignment: - Use Adam. Learning rate; - [Nokia]: - [QC, R&S, CATT, Ericsson]: 10^-4 - [R&S]: Learning rate adaptation: Reduce on plateau by factor 0.5 with patience of 50 epochs. - [Apple]: 0.0025 - [Samsung, Intel, MTK, CMCC]: 0.001 - [ZTE]:1.5e-3 - [vivo]: Warm-up to initial lr 1e-3 and then linearly decay to final lr 8e-4, update after each batch. Configuration for alignment: - Fixed learning rate of 10^-4. Batch size; - [Nokia]:1024 - [QC, MTK, CMCC]: 128 - [Apple]: 256 - [Samsung]: 200 - [R&S, CATT, vivo, Ericsson]: 512 - [Ericsson]: 32 - [Intel]: 10 - [ZTE]:400. Configuration for alignment: - 256. (Max) Number of epochs; - [QC]: 150 - [Apple, vivo, Intel]: 100 - [R&S, Nokia, CATT]: 200 - [MTK, ZTE]: 300 - [CMCC]: 500. Configuration for alignment: - 150 epochs. Stopping criteria; - [Nokia, Apple, CATT, QC, ZTE, Intel, CMCC]: No change in metric over a given number of epochs - Validation patience: [Intel, Nokia] 10, [QC] 50 - + Ericsson with constraint on max Epochs - [vivo] Stop when reaching the configured number of epochs. Configuration for alignment: - No improvement over 25 epochs.. Weight initialization; - [Apple, CATT, QC, vivo, Intel, CMCC]: Default - Ericsson: Variance scaling. Configuration for alignment: - Default. Kernel Parameters: - Dilation - Padding Type The value of groups (VG); - [CMCC, Samsung, Vivo, CATT]: Dilation:1, Padding type=0, VG= 1 - [QC, ZTE, Apple, Intel]: Default. Configuration for alignment: - Default.

Appendix C: Agreements of standardization of model structure in RAN1

RAN1#120 agreed the full-pictured model structure for studying the standardized model structure as following

Agreement For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose, Encoder description: Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer); Number of tokens Feature dimension of each token Output dimension dmodel per token. Positional encoding; Number of tokens positions Dimension of positional encoding for each token position: dmodel. Transformer blocks; Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice. Output linear layer; Reshape the matrix to a vector Output dimension zdim. Quantization; Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment.

And RAN1 #120 agreed some options on the scalable model structure specification.

A Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

Appendix D: Agreements of RAN4 reference model standardization

The agreements of RAN4 reference model standardization are listed in the following.

Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Issue 1-3: Option 4a (Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - Step 5: Label selected dataset(s) (mapping of encoder input to latent message) [using encoder] corresponding to the selected test decoder(s) from option 3 - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders(using different structures) Selected test decoder and matching encoder pair from Option 3 can be used to generate the common dataset (that includes latent messages) based on aggregated/mixed dataset Issue 1-4: Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - Step 6: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures) Option 3 track 1 Endorsed preliminary observations - The investigation on option 3 is based on using the agreed structure for the decoder and encoder - It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional - The performance of the encoder is very dependent on the alignment of the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testing dataset - The variation in the performance depending on the training datasets and test datasets can be as much as [20%] in SGCS. Further details are available in the results spreadsheet. - Preliminary observation; further results may be added/refined and the observation updated (for example, using the mixed dataset as in track 2) - For option 3, track 1 to be useful, careful attention would be needed to the alignment of datasets - Up to now, the same encoder structure is considered as used when creating the reference decoder; the impact of different encoder structure is not yet considered Option 3 track 2 Endorsed preliminary observations - There is reasonable SGCS convergence when companies train an encoder-decoder pair using the mixed dataset and test using the mixed testset - Around 1% variation in SGCS; see results spreadsheet for more details - Preliminary observation; further results may be added - It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional - When training “own” encoder with the frozen decoder and testing, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results from results presented so far - Around [+- 2%] variation in SGCS from the average; see results spreadsheet for more details - Preliminary observation; further results may be added - Up to now, the same encoder structure is considered as used when creating the reference decoder by most companies; the impact of different encoder structure is not yet fully considered Option 4 Endorsed preliminary observations - For option 4b: It is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder when the mixed dataset is used - For option 4a: For the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b) Sub-topic 4: What would be standardized for option 3 Agreement: - To verify the performance of CSI compression test decoder for calibration of test equipment, agree to define the reference encoder including encoder structure and parameters in RAN4 - FFS on whether to specify it in TS or TR, and capture the conclusion in the TR conclusion part - FFS on criterion for selection of reference encoder - FFS on decision whether RAN4 or RAN5 will specify it What needs to be captured or specified for Option 3 Agreements - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) What needs to be captured or specified for Option 4 Agreement: - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. Relation between RAN1 option 3 and RAN1 inter-vendor collaboration direction C Agreement: - FFS on the relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions.